3GPP TSG-RAN3 Meeting #105	R3-194162

Ljubljana, Slovenia, 26th August – 30th August 2019

Agenda Item:	14.3

Source:	Ericsson

Title: 	Common UP and CP aspects of CIoT when connected to 5GC

Document for:	Discussion and Approval

# 1	Introduction

In the previous RAN3 meeting, companies discussed how to structure the work for the “Rel-16 CIoT (LTE-M and NB-IoT) connected to 5GC” topic. A company proposed a working assumption in [1] in order to structure the work and assign the BL CRs for the agenda item 14.3.

As described in [1], companies need to sketch what are the common parts for each of LTE-M and NB-IoT WI when it comes to connecting them to 5G core, as well as the dedicated aspects to each WI: LTE-M only and NB-IoT only, with their respective WI code. Based on this first separation, the work can be separated into “common CP/UP aspects”, “UP only” and “CP only”. Further details of this separation are explained in the working assumption [1].

In the previous meeting, the following was agreed:

Connection with 5GC:

# eDRX_for_NBIOT

- Check details only for NBIOT

- Remove NB-IoT Default Paging DRX IE and NB-IoT UE Identity Index value

(HW)

Rev in R3-193159 Endorsed as BL; E/// to take over for further work

# Enhanced_Coverage

- Check details for both NBIOT and MTC

- Two WI codes

(E///)

Rev in R3-193160 (E///,ZTE) Endorsed as BL

# CRs_Management

- Check the principle how to maintain the CRs approved for each meeting

(HW)

Summary of offline in R3-193161 noted

“Tasks for next meeting” (slide 6) endorsed

Hence, as agreed, companies have to provide a dedicated BL CR (and/or TP for the already endorsed BL CRs) listing the new functionalities to be added for each WI, with the potential changes to the NR specifications (38.413, 38.423). Also, it was agreed that for this meeting companies should start with the common parts first, with the list of the potential common LTE IEs that can be transposed to NR.

In this paper, we provide a TP to the BL CR [2] for introducing support of common UP/CP aspects of CIoT, with the introduction of the following functionalities in NG-RAN:

-	Pending data indication

-	UE subscription-based differentiation

For the common UP/CP aspects of NB-IoT only, such as paging eDRX [3], we suggest discussing them in the next meeting, once the common parts have been agreed.

# 2	Discussion

New Rel-16 WID revisions for eMTC [4] and NB-IoT [5] have been proposed in RAN#84 with the following objectives: ​

Connection to 5GC: (LTE-M) - Specify support for the following features [RAN2, RAN3] - Support of extended DRX in CM-IDLE - Support of extended DRX in CM-CONNECTED with RRC_INACTIVE (support of sleep cycles up to the NAS and SMS retransmission timers) - Support of User Plane CIoT optimisation - Support of EDT for Data over NAS and User Plane CIoT optimization (see Note) - Support of restriction of use of Enhanced Coverage - Delivery of Expected UE Behaviour information to the RAN - Additional information in SIB to indicate supported CIoT features; indication of CIoT features supported by the UE in RRC Note: Based on the outcome of RAN2/SA2 liaison exchanges, EDT support for connection to 5G-CN may be later updated to also include RRC_INACTIVE. Connection to 5GC: (NB-IoT) - Specify support for the following features [RAN2, RAN3] - Support of extended DRX in CM-IDLE - Support of User Plane CIoT optimization - Support of EDT for Data over NAS and User Plane CIoT optimization - Support of Inter-UE QoS for data over NAS (resource prioritisation between different NB-IoT UEs) - Support of restriction of use of Enhanced Coverage - Delivery of Expected UE Behaviour information to the RAN - Additional information in SIB to indicate supported CIoT features; indication of CIoT features supported by the UE in RRC

## 2.1 Pending Data Indication

In LTE, the Pending Data Indication helps avoid an early return to idle (triggered by the eNB), and is provided to the eNB by the MME to indicate that some signaling or data is pending in the network for the UE. This IE is present in the following S1AP messages:

- INITIAL CONTEXT SETUP REQUEST

- HANDOVER REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

- DOWNLINK NAS TRANSPORT

It is proposed to include the Pending data indication IE in the equivalent NGAP messages.

Introduce the Pending Data Indication IE in the following NGAP messages: INITIAL CONTEXT SETUP REQUEST, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE and DOWNLINK NAS TRANSPORT

## 2.2 UE Subscription-based Differentiation

The UE subscription-based differentiation feature can also be transposed to NGAP. Similarly, to Coverage Enhancement restriction, this is introduced in the same S1AP procedures as above.

In addition, we note that in EPS, the IE is also passed at Xn handover; it should be FFS whether this indication should be included over Xn. We think it should not be necessary.

Introduce the UE Subscription-based Differentiation IE in the following NGAP messages: INITIAL CONTEXT SETUP REQUEST, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE and DOWNLINK NAS TRANSPORT

- We provide in [6] the TP for the BL CR to capture these changes in NGAP specification

## 2.3 Removal of CE mode B restriction support

In LTE, two sorts of extended coverage restrictions are supported: restriction against any use of CE and restriction against use of CE mode B. CE mode B is specific to LTE-M, while the general CE restriction is common to both LTE-M and NB-IoT.

As the dedicated aspects of LTE-M were attributed to be addressed in another BL CR by another company (according to slide 6 of [1]), we propose to remove from the BL CR the support of CE mode B restriction, so as to leave only the general feature of restriction against any use of CE, which is common to both LTE-M and NB-IoT.

Remove support of restriction of use of CE Mode B from the BL CR covering common UP/CP CIoT aspects.

- We provide in [7] a TP to capture this removal from the endorsed BL CR.

# 3 Conclusion

In the previous section we discussed what are the common UP/CP aspects of LTE-M and NB-IoT to capture in NGAP specifications. We propose the following:

- Introduce the Pending Data Indication IE in the following NGAP messages: INITIAL CONTEXT SETUP REQUEST, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE and DOWNLINK NAS TRANSPORT

- Introduce the UE Subscription-based Differentiation IE in the following NGAP messages: INITIAL CONTEXT SETUP REQUEST, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE and DOWNLINK NAS TRANSPORT

- Remove support of restriction of use of CE Mode B from the BL CR covering common UP/CP CIoT aspects.

- We provide in [6] and [7] the TPs for BL CR to capture these changes in NGAP specification.

# 4 References

- R3-193161, “CIoT Work Structure, Huawei”, Qualcomm. Inc, RAN3#104

- R3-193355, “Support of Enhanced Coverage restriction in NGAP for LTE-M Ues”, Ericsson, RAN3#105

- R3-193354, “Introduction of NB-IoT Paging and eDRX aspects”, Ericsson, RAN3#105

- RP-191356, “Revised WID: Additional MTC enhancements for LTE”, Ericsson, RAN#84

- RP-191576, “WID revision: Additional enhancements for NB-IoT”, Huawei, RAN#84

- R3-195163, “(TP for Support of Enhanced Coverage restriction in NGAP for LTE-M UEs BL CR for TS 38.413): Introduction of common UP/CP CIoT features to NGAP”, Ericsson, RAN3#105

- R3-194303, “(TP for LTE_eMTC5, NB_IOTenh3 BL CR for TS 38.413): Removal of CE modeB restriction”, Ericsson, RAN3#105