3GPP TSG-RAN WG3 Meeting #105	R3-194172

Ljubljana, 26th – 30th August 2019

Agenda Item:	19.5

Source:	Ericsson

Title:	NR uplink positioning for split gNB

Document for:	Approval

# Introduction

According to [1], the following has been agreed:

-	NR should support timing (UL-TDOA) based UL-only positioning techniques in FR1 and FR2.

-	UL-TDOA with two or more neighboring gNBs/TRPs for NR UL positioning should be supported.

The appropriate NR UL reference signals for positioning have been also agreed in [1].

- The agreed gNB measurements for NR positioning are:

- gNB Measurements for NR Positioning

The following measurements for NR UL PRS at serving and neighbor gNBs should be supported:

-	UL RTOA measurements

-	UL Angle Of Arrival (AOA) measurements (including Azimuth and Zenith Angles)

-	UL RSRP (reference signal received power) measurements

-	gNB RX-TX time difference measurements

In this contribution, our focus is UL-TDOA, therefore among the above gNB measurements only signaling support for the first one (UL RTOA measurement) will be discussed. Also, we try to define signaling support (NRPPa and F1AP) for UL-TDOA as an uplink only NR RAT-dependent positioning, according to the WI objectives [2].

# Discussion

In LTE, the UL RTOA measurements were done by Location Management Unit (LMU), which was defined as a separate unit connected to the location server (i.e. E-SMLC) via SLm protocol. Historically considering LMUs as a separate unit from eNBs has been a challenge, and sometimes a drawback, for the UTDOA method.

In NG-RAN, we need to consider the CU-DU split case. As the positioning measurements including (UL-RTOA) is dependent to the physical location of the gNB, therefore it is reasonable to consider that these measurements are done in the gNB-DU. In RAN3, a gNB-DU with only positioning functionality is not precluded. So even if a separate LMU exists, it would be connected to the gNB-CU via the F1 interface.

NR uplink measurements, including UL RTOA are done in gNB-DUs.

- Observation 1 	RAN3 has agreed that LMU functionality can be part of the gNB-DU functionality, and that the F1 interface will be used between the CU and the LMU functionality.

According to UL-TDOA method, UL RTOA measurements need to be collected at the location server (i.e. LMF). Given that the NRPPa protocol is between the LMF and the gNB-CU, we can consider that F1AP and NRPPa together shall transfer the UL RTOA measurements from gNB-DUs to the LMF.

The (F1AP + NRPPa) transfers the UL RTOA from gNB-DU to the gNB-CU to the LMF.

- In order for the gNB-DUs to know what uplink signals to measure, the configuration of the uplink signal for a certain UE would be similarly transferred in the opposite direction from the LMF to the gNB (and gNB-DU) via NRPPa + F1AP.

The (NRPPa + F1AP) transfer the uplink signal configurations, e.g. SRS from the LMF to one or more gNBs to one or more gNB-DUs which can receive the UE’s UL transmissions.

Consistently with current NG-RAN architecture principles, the LMF does not have visibility of the gNB-DU(s) involved in positioning, but only of the gNBs.

The (NRPPa + F1AP) transfer the uplink signal configurations, e.g. SRS from the LMF to one or more gNBs to one or more gNB-DUs which can receive the UE’s UL transmissions.

In order to support UL-TDOA with two or more neighboring gNBs/TRPs, the UL reference signal configuration shall be transmitted to the specified UE. There are two nodes which can send this configuration information to the UE:

- LMF  UE via LPP

- gNB-CU  UE via RRC

LPP and RRC behavior and interactions are out of RAN3 scope, but ultimately it is the gNB-CU which decides on the UL-RS configuration (e.g. including number of transmissions and bandwidth) for the UE. Therefore, it is the gNB-CU which informs the LMF of the uplink signal configurations, so that LMF can send this information to the other gNBs which are potentially capable of measuring UE’s uplink signal.

The gNB-CU provides the uplink signal configuration for a particular UE to the LMF via NRPPa.

The LMF does not decide the uplink signal configuration for a particular UE.

- Similar to what has been previously agreed in LTE, and also as the gNB-CU is the ultimate decision-maker in terms of uplink signal configuration for the UE, we believe that directly transferring the uplink signal configuration from gNB-CU to the UE via RRC is the most efficient way. For example, gNB-CU may need to provide bandwidth information, especially if the UE is configured with multiple UL bandwidth parts. While the LMF can still inform the gNB-CU of UL signal characteristic information, it is still up to the gNB-CU to decide upon this configuration. The gNB-CU may inform the LMF about the configured uplink signalling configuration.

The gNB-CU may inform the LMF about the configured uplink signalling configuration for the purpose of UL-TDOA of the UE.

In a split gNB scenario, as it is the gNB-DU which “owns” the radio interface configuration, the gNB-DU may reject the configuration proposed  by the gNB-CU e.g. if the appropriate radio resources cannot be made available for UL positioning.

The gNB-DU may reject the configuration proposed by the gNB-CU e.g. if the appropriate radio resources cannot be made available for UL positioning.

# Conclusion

Based on the discussion in section 2 we propose the following:

- The NR uplink measurements including UL RTOA is done in gNB-DUs.

The (F1AP + NRPPa) transfers the UL RTOA from gNB-DU to the gNB-CU to the LMF.

The (NRPPa + F1AP) transfer the uplink signal configurations e.x. SRS from the LMF to one or more gNBs to one or more gNB-DUs which can receive the UE’s UL transmissions.

The (NRPPa + F1AP) transfer the uplink signal configurations, e.g. SRS from the LMF to one or more gNBs to one or more gNB-DUs which can receive the UE’s UL transmissions.

The gNB-CU provides the uplink signal configuration for a particular UE to the LMF via NRPPa.

The LMF does not decide the uplink signal configuration for a particular UE.

The gNB-CU may inform the LMF about the configured uplink signalling configuration for the purpose of UL-TDOA of the UE.

The gNB-DU may reject the configuration proposed by the gNB-CU e.g. if the appropriate radio resources cannot be made available for UL positioning.

# References

[1] TR 38.855, Study on NR positioning support, Rel. 16.0.0.

[2] RP190752, New WID: NR positioning support; Rel.16, Intel, Ericsson.

[3] TS 38.473, NG-RAN F1 Application Protocol (F1AP).