3GPP TSG-RAN3 Meeting #105	R3-194180

Ljubljana, Slovenia, 26th August – 30th August 2019

Agenda Item:	31.2

Source: 	Ericsson

Title:  	Discussion and Solution on number of served cells in Xn/X2 Setup

Document for:	Discussion

# Introduction

During setup of an EN-DC X2 interface and of an Xn interface, the current standard imposes an eNB (for EN-DC X2 Setup) and an NG-RAN node (for Xn Setup) to receive the full list of cells served by an en-gNB or a gNB. While this decision was taken to allow the receiving node to have a full view of the cells served by the en-gNB/gNB, the decision was also taken under the assumption that the actual number of cells served by an en-gNB/gNB would be contained.

Mass deployment of 5G networks is now a very close reality and with that it is emerging that some implementations support very high amounts of cells at en-gNBs and gNBs. This creates a scalability problem, with building, transporting and decoding very big messages, where most of the contents is not relevant to the receiving node. The node receiving the full list of served cells may not even be able to process the message due to its very large size.

This paper discusses this issue and provides solutions for it.

# Discussion

An en-gNB and a gNB can support a maximum of 16384 cells. When an EN-DC X2 Setup Request/Response is issued by an en-gNB or when an Xn Setup Request/Response is issued by a gNB, the message may contain a List of Served NR Cells IE of up to 16384 cells. For each of the listed NR cells there could be a list of neighbouring cells, the NR Neighbour Information IE, which could reach a maximum of 1024 neighbour cells. The result is a potential maximum of 16384x1024 == 16.777.216 cell information in one ASN.1 encoded message.

Observation 1: the current standard allows for inclusion of information for up to 16.777.216 cells in one of the EN-DC X2 Setup Request/Response messages or Xn Setup Request/Response messages

Obviously, it is up to implementation how to dimension the memory of an eNB/NG-RAN node. Such memory would need to be dimensioned to allow storage of the full ASN.1 encoded message in order to allow the node to decode it correctly. However, given the very high number of cells that could be listed in the messages in question, it should be acknowledged that there might be implementations where the overall available memory in a RAN node may not be sufficient to host a message with a high number of cells information.

Conclusion 1: There might be implementations that are not able to correctly decode messages with very high numbers of cells information due to memory and ASN.1 decoding capacity limitations.

In light of the above conclusion, the standard should address the case where an eNB or an NG-RAN node receives an “oversized” EN-DC X2 Setup Request/Response or Xn Setup Request/Response message. Namely, there should be solutions where the procedure is not simply rejected, because that alternative would lead to making it impossible to setup an interface at all between the nodes involved.

Proposal 1: 3GPP should address the case of oversized EN-DC X2 Setup Request/Response and Xn Setup Request/Response messages with solutions that avoid failures or rejections of the procedure

In order to achieve the target in Proposal 1 the standard should be changed to avoid that the EN-DC X2 Setup Request/Response and Xn Setup Request/Response contain a full list of cells. In EN-DC it could still be feasible that an eNB responds with a full list of cells because the maximum number of cells an eNB can support is 256, i.e. a limited number. However, in all other cases (where the maximum number of supported cells is 16384), the standard should not mandate that the interface setup procedure includes a full list of cells. In order to make it clear whether the list of cells included in the setup procedure is full or partial, a new flag should be added by the sending node.

Below are excerpts of the standard with highlights on the parts that should be changed to eliminate the mandate of sending a full list of cells:

From TS 36.423:

#### 8.7.1	EN-DC X2 Setup

[...]

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: eNB Initiated EN-DC X2 Setup, successful operation

Figure 8.7.1.2-2: en-gNB Initiated EN-DC X2 Setup, successful operation

eNB initiated EN-DC X2 Setup:

An eNB initiates the procedure by sending the EN-DC X2 SETUP REQUEST message to a candidate en-gNB. The candidate en-gNB replies with the EN-DC X2 SETUP RESPONSE message. The initiating eNB shall transfer the complete list of its served cells to the candidate en-gNB. The candidate en-gNB shall reply with the complete list of its served cells.

[…]

en-gNB initiated EN-DC X2 Setup:

An en-gNB initiates the procedure by sending the EN-DC X2 SETUP REQUEST message to a candidate eNB. The candidate eNB replies with the EN-DC X2 SETUP RESPONSE message. The initiating en-gNB shall transfer the complete list of its served cells to the candidate eNB. The candidate eNB shall reply with the complete list of its served cells.

[…]

#### 9.1.2.31	EN-DC X2 SETUP REQUEST

This message is sent by an initiating node to a neighbouring node, both nodes able to interact for EN-DC, to transfer the initialization information for a TNL association.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Initiating NodeType | M |  |  |  |  |  |
| >eNB |  |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| >>List of Served E-UTRA Cells |  | 1 .. <maxCellineNB> |  | Complete list of cells served by the eNB | YES | reject |
| >>>Served E-UTRA Cell Information | M |  | Served Cell Information 9.2.8 |  | – | – |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | YES | ignore |
| >en-gNB |  |  |  |  |  |  |
| >>Global en-gNB ID | M |  | 9.2.112 |  | YES | reject |
| >>List of Served NR Cells |  | 1 .. <maxCellinengNB> |  | Complete list of cells served by the engNB. | YES | reject |
| >>>Served NR Cell Information | M |  | 9.2.110 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

#### 9.1.2.32	EN-DC X2 SETUP RESPONSE

This message is sent by a neighbouring node to an initiating node, both nodes able to interact for EN-DC, to transfer the initialization information for a TNL association.

Direction: eNB  en-gNB, en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  |  |  |
| >eNB |  |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.22 |  | YES | reject |
| >>List of Served E-UTRA Cells |  | 1 .. <maxCellineNB> |  | Complete list of cells served by the eNB | YES | reject |
| >>>Served E-UTRA Cell Information | M |  | Served Cell Information 9.2.8 |  | – | – |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | YES | ignore |
| >en-gNB |  |  |  |  |  |  |
| >>Global en-gNB ID | M |  | 9.2.112 |  | YES | reject |
| >>List of Served NR Cells |  | 1 .. <maxCellinengNB> |  | Complete list of cells served by the en-gNB | YES | reject |
| >>>Served NR Cell Information | M |  | 9.2.110 |  | – |  |
| >>>NR Neighbour Information | O |  | 9.2.98 | NR neighbours | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

From TS 38.423:

#### 9.1.3.1	XN SETUP REQUEST

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| AMF Set Information | M |  | 9.2.3.83 | List of all the AMF Sets to which the NG-RAN node belongs. | YES | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Complete list of cells served by the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Complete list of cells served by the ng-eNB. | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.2	XN SETUP RESPONSE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Complete list of cells served by the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Complete list of cells served by the ng-eNB | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

Proposal 2: It is proposed to amend TS 36.423 and TS 38.423 to remove the mandate for a node to send a full list of cells during EN-DC X2 Setup and Xn Setup procedures. eNBs in EN-DC X2 Setup can still provide a full list of cells as it is of limited size. A flag indicating whether the list of served cells is full or partial should be added, to allow for clear interpretation of the received information.

On top of removing the mandate of signalling a full list of cells, the EN-DC Setup Request and Xn Setup Request should also be enhanced with information on the maximum message size the sending node can support. Namely, a node sending an EN-DC X2 Setup Request, for example, should be able to indicate to the peer node that it would like to receive back a limited list of cells. This would avoid decoding issues when receiving the response message.

Proposal 3: The EN-DC X2 Setup Request and Xn Setup Request should be enhanced with an indication of the maximum cell list size the sending node is able to receive

Finally, the EN-DC X2 Setup Failure and the Xn Setup Failure messages should be enhanced with indications that the procedure has failed due to too large message size. This would allow the node receiving the failure message to determine that any future interface setup towards the peer node need to be performed with a reduced message size, i.e. by including a reduced number of served cells. Additionally, the messages could also include information regarding the maximum number of cells the node generating the failure can receive as part of the served cell information. This allows that, in future attempts of EN-DC X2 and Xn establishments, the node triggering the procedure signals information for served cells in a number that is within the limits indicated by the node where the failure occurred.

Proposal 4: The EN-DC X2 Setup Failure and Xn Setup Failure messages should include an indication of failure due to too large message size. Optionally an indication of the maximum cell list size supported by the node where the failure occurred could be included

# Conclusion

In this paper it has been explained how according to the current standard a node might not be able to process messages for the EN-DC X2 Setup or Xn Setup procedures, due to the very large number of cell information that might be included in such messages.

The Paper presents proposals on how the issue could be addressed. Such proposals are listed below and are aimed at allowing different types of implementations to correctly inter-operate, avoiding repeated failure scenarios.

Observation 1: the current standard allows for inclusion of information for up to 16.777.216 cells in one of the EN-DC X2 Setup Request/Response messages or Xn Setup Request/Response messages

Conclusion 1: There might be implementations that are not able to correctly decode messages with very high numbers of cells information due to memory and ASN.1 decoding capacity limitations.

Proposal 1: 3GPP should address the case of oversized EN-DC X2 Setup Request/Response and Xn Setup Request/Response messages with solutions that avoid failures or rejections of the procedure

Proposal 2: It is proposed to amend TS 36.423 and TS 38.423 to remove the mandate for a node to send a full list of cells during EN-DC X2 Setup and Xn Setup procedures. eNBs in EN-DC X2 Setup can still provide a full list of cells as it is of limited size. A flag indicating whether the list of served cells is full or partial should be added, to allow for clear interpretation of the received information.

Proposal 3: The EN-DC X2 Setup Request and Xn Setup Request should be enhanced with an indication of the maximum cell list size the sending node is able to receive

Proposal 4: The EN-DC X2 Setup Failure and Xn Setup Failure messages should include an indication of failure due to too large message size. Optionally an indication of the maximum cell list size supported by the node where the failure occurred could be included

CRs supporting the proposals above are available in R3-194181 and R3-194384