# 3GPP TSG-RAN WG3 #105	Tdoc R3-194191

Ljubljana, Slovenia, 26th – 30th August

Agenda Item:	31.3.06

Source:	Ericsson

Title:	About the case necessary UE capabilities not being supported

Document for:	Discussion, Decision

# Introduction

In the previous meeting we looked into the case that in EN-DC the SgNB cannot add the requested SCG because of limitations in the UE capabilities and we proposed ways that the gNB-DU can flag this case to the gNB-CU. These cases were described in R3-192889 and R3-192847. In the following we examine the issue again and argue about the importance and the added benefits of our proposal.

# Discussion

In R3-192847 we examined in detail the case where intra-band EN-DC with concurrent UL transmission on both cell groups is configured. A brief recap follows in order to set the context for our discussion.

The problem is that the UE may violate requirements on unwanted emissions outside the carriers when the two UL channels are active. RAN4 has determined that UEs should have a new capability to fulfil such emission requirements, namely the capability of meeting the required emissions limits. If the UE supports such capability, the UE is allowed power reduction (MPR).

The applicable emission limits are indicated by “NS values” sent in system information in each CG. Upon reading this information the UE is aware of the limits and of the allowed power reduction.

For EN-DC the applicable limits and allowed UE power back-off depends on the NS values sent in each CG. The MeNB can obtain the UE MR-DC capability for the EN-DC configuration. However, some of the UE capability information with respect to MPR, namely the modifiedMPRbehavior that is not supported by early UEs, can only be decoded by the SgNB.

Moreover, the MeNB is not aware of the NS value sent in the system information of the SCG. Hence the SgNB should be able to reject the MeNB request if the UE does not support the MPR capability and the NS value indicated in the SCG is NS_01. There may also be other cases in which the L1/L2 UE NR capabilities are not sufficient for configuration of the SCG.

Our proposal stems from the intention to provide a cause value for such rejection that allows to avoid the same failure in the future. Namely, the cause value would explicitly indicate that the capabilities of the UE were not sufficient to add the SN cell. Reception of the cause value together with the SN addition failure message, would prompt the MeNB to not add the same SN cell for the same UE in the future and will avoid setting up the same combination of cells/carriers for the UE. If it has no other configuration option (e.g. less SCells on MCG), the MeNB should actually consider de-configuring the NR measurements for such UE. 
it is worth noticing that if the SN addition failure is not identified clearly with a cause value that indicates insufficient UE capabilities, the MN is likely to try over and over again to add the same SCG for the same UE, as the UE measurements will likely indicate that cell as a good SCG candidate. This procedure will of course always generate an SN addition failure and it will consume time (gaps), battery power and signalling load leading to a rather inefficient scenario.

Also by including the cause value, we enable the MeNB to avoid subsequent wrong configuration for UEs with similar capabilities as the UE that has already incurred in an SN addition rejection. Considering that the number of different UE chipsets is limited, it would help to avoid most incorrect SN additions once the MeNB has learned the UE models.

Based on the above reasoning, we believe that it would indeed be very helpful to introduce a new cause value “insufficient UE capabilities”.

Proposal 1: 	Introduce a new cause value for the F1AP, namely “insufficient UE capabilities”.

# Conclusion

In this contribution, we discussed the case when SgNB cannot add the requested SCG because of limitations in the UE capabilities and made the following proposal.

Proposal 1: 	Introduce a new cause value for the F1AP, namely “insufficient UE capabilities”.

A CR matching proposal 1 is provided in R3-194122.