3GPP TSG-RAN WG3 Meeting #105 	R3-194235

# Ljubljana, Slovenia, August 26-30 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0133 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IPSec Pre-establishment E1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split |  |  |  |  |  |  | Date: |  | 2019-08-16 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification do not support mechanism to provide IPSec addresses for User Plane apriori. This hinders user experience as IPSec establishment for the user plane takes time and delays the overall setup. Further, gven IPSec support is mandatory also for th euser plane, an optimized mechanism is necessary. Additionally, the gNB-CU-CP is unable to indciate additional IPSec addresses made available (e.g. due to virtualized environment) for tunnels. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce means to convey IPsec addresses during interface setup and configuration update procedures. Impact assessment towards the previous version of the specification (same release): This CR has an impact under functional and protocol point of view The impact can be considered isolated because the change is limited to indication of TNL IP addresses. This CR is ASN.1 backwards compatible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IPSec tunnel cannot be set up beforehand, which incurs additional end user delays. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

Beginning of Text Proposal to TS 38.463

#### 9.2.1.4	GNB-CU-UP E1 SETUP REQUEST

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | Ignore |
| UP DL TNL Information |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| > NG UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP DL user plane transport layer information. |  |  |
| >NG IPSec UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG IPSec UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec DL user plane transport layer information. |  |  |
| > S1 UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>S1 UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP DL user plane transport layer information. |  |  |
| >NG IPSec UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG IPSec UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec DL user plane transport layer information. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofUPDLTLA | Maximum no. user-plane transport layer addresses that can be served by a gNB-CU-UP. Value is XX. |

#### 9.2.1.5	GNB-CU-UP E1 SETUP RESPONSE

This message is sent by the gNB-CU-CP to transfer information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP. | YES | Ignore |
| UP UL TNL Information |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| > NG UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP UL user plane transport layer information. |  |  |
| >NG IPSec UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG IPSec UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec UL user plane transport layer information. |  |  |
| > S1 UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>S1 UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP UL user plane transport layer information. |  |  |
| >NG IPSec UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG IPSec UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec UL user plane transport layer information. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUPULTLA | Maximum no. user-plane transport layer addresses that can be served by a gNB-CU-CP. Value is XX. |

#### 9.2.1.6	GNB-CU-UP E1 SETUP FAILURE

This message is sent by the gNB-CU-CP to indicate E1 Setup failure.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.7	GNB-CU-CP E1 SETUP REQUEST

This message is sent by the gNB-CU-CP to transfer information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP. | YES | Ignore |
| UP UL TNL Information |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| > NG UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP UL user plane transport layer information. |  |  |
| >NG IPSec UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG IPSec UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec UL user plane transport layer information. |  |  |
| > S1 UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>S1 UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP UL user plane transport layer information. |  |  |
| >NG IPSec UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG IPSec UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec UL user plane transport layer information. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-CP and the gNB-CU-UP. Value is 32. |
| maxnoofUPULTLA | Maximum no. user-plane transport layer addresses that can be served by a gNB-CU-CP. Value is XX. |

#### 9.2.1.8	GNB-CU-CP E1 SETUP RESPONSE

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | Ignore |
| UP DL TNL Information |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| > NG UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP DL user plane transport layer information. |  |  |
| >NG IPSec UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG IPSec UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec DL user plane transport layer information. |  |  |
| > S1 UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>S1 UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP DL user plane transport layer information. |  |  |
| >NG IPSec UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG IPSec UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec DL user plane transport layer information. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofUPDLTLA | Maximum no. user-plane transport layer addresses that can be served by a gNB-CU-UP. Value is XX. |

#### 9.2.1.9	GNB-CU-CP E1 SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate E1 Setup failure.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.10	GNB-CU-UP CONFIGURATION UPDATE

This message is sent by the gNB-CU-UP to transfer updated information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | O |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| Supported PLMNs |  | 0..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| gNB-CU-UP TNLA To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-CU-UP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-CP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| UP DL TNL Information |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| > NG UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP DL user plane transport layer information. |  |  |
| >NG IPSec UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG IPSec UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec DL user plane transport layer information. |  |  |
| > S1 UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>S1 UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP DL user plane transport layer information. |  |  |
| >NG IPSec UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG IPSec UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec DL user plane transport layer information. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-UP and the gNB-CU-CP. Value is 32. |
| maxnoofUPDLTLA | Maximum no. user-plane transport layer addresses that can be served by a gNB-CU-UP. Value is XX. |

#### 9.2.1.11	GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU-CP to a gNB-CU-UP to acknowledge update of information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Ignore |
| UP UL TNL Information |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| > NG UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP UL user plane transport layer information. |  |  |
| >NG IPSec UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG IPSec UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec UL user plane transport layer information. |  |  |
| > S1 UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>S1 UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP UL user plane transport layer information. |  |  |
| >NG IPSec UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG IPSec UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec UL user plane transport layer information. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUPULTLA | Maximum no. user-plane transport layer addresses that can be served by a gNB-CU-CP. Value is XX. |

#### 9.2.1.12	GNB-CU-UP CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-CU-CP to indicate gNB-CU-UP Configuration Update failure.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.13	GNB-CU-CP CONFIGURATION UPDATE

This message is sent by the gNB-CU-CP to transfer updated information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP | YES | ignore |
| gNB-CU-CP TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Usage | M |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNLA is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.462 [18]. | - | - |
| gNB-CU-CP TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-UP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | YES | reject |
| gNB-CU-CP TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Usage | O |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNLA is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.462 [18]. | - | - |
| UP UL TNL Information |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| > NG UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP UL user plane transport layer information. |  |  |
| >NG IPSec UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG IPSec UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec UL user plane transport layer information. |  |  |
| > S1 UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>S1 UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP UL user plane transport layer information. |  |  |
| >NG IPSec UP UL TNL Information List |  | 0..<maxnoofUPULTLAs> |  |  |  |  |
| >>NG IPSec UP UL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec UL user plane transport layer information. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-CP and the gNB-CU-UP. Value is 32. |
| maxnoofUPULTLA | Maximum no. user-plane transport layer addresses that can be served by a gNB-CU-CP. Value is XX. |

#### 9.2.1.14	GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU-UP to a gNB-CU-CP to acknowledge update of information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP TNLA Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP | - | - |
| gNB-CU-CP TNLA Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA Failed To Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP | - | - |
| >>Cause | M |  | 9.3.1.2 |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Ignore |
| UP DL TNL Information |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| > NG UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP DL user plane transport layer information. |  |  |
| >NG IPSec UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG IPSec UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec DL user plane transport layer information. |  |  |
| > S1 UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>S1 UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP DL user plane transport layer information. |  |  |
| >NG IPSec UP DL TNL Information List |  | 0..<maxnoofUPDLTLAs> |  |  |  |  |
| >>NG IPSec UP DL TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | Indicates CU-CP IPsec DL user plane transport layer information. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-CP and the gNB-CU-UP. Value is 32. |
| maxnoofUPDLTLA | Maximum no. user-plane transport layer addresses that can be served by a gNB-CU-UP. Value is XX. |

#### 9.2.1.15	GNB-CU-CP CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-CU-UP to indicate gNB-CU-CP Configuration Update failure.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

End of Text Proposal to TS 38.463