3GPP TSG-RAN WG3 Meeting #105 	R3-194240

Ljubljana, Slovenia, 26 - 30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0134 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Stand-alone NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN |  |  |  |  |  |  | Date: |  | 2019-08-16 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 2 for Stand-alone NPN has been agreed by SA2, which incurs changes to E1AP to convey the corresponding NID (Network IDentifier) per PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New IE to convey NID per PLMN. Impact assessment towards the previous version of the specification (same release): This CR has an impact under functional point of view. The impact can be considered isolated because the change affects only cell related information. This CR is ASN.1 backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stand-alone NPN functionality is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.2.3, 8.2.4, 8.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

Beginning of Text Proposal to TS 38.463

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[4]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error".

[6]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[7]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[8]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[9]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specificaiton”.

[11]	3GPP TS 23.401: “General Packet Radio Service (GPRS) Enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[12]	3GPP TS 23.203: “Policy and Charging Control Architecture”.

[13]	3GPP TS 33.501: “Security Architecture and Procedures for 5G System”.

[14]	IETF RFC 5905: “Network Time Protocol Version 4: Protocol and Algorithms Specification”.

[15]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[16]	3GPP TS 38.414: “NG-RAN; NG Data Transport”.

[17]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[18]	3GPP TS 38.462: "NG-RAN; E1 Signalling Transport".

[19]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[20]	3GPP TS 23.501: "System Architecture for the 5G System".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[22]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

Next Text Proposal to TS 38.463

### 8.2.3	gNB-CU-UP E1 Setup

#### 8.2.3.1	General

The purpose of the gNB-CU-UP E1 Setup procedure is to exchange application level data needed for the gNB-CU-UP and the gNB-CU-CP to correctly interoperate on the E1 interface. If the gNB-CU-UP initiates the first TNL association, it shall also initiate the gNB-CU-UP E1 Setup procedure. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: gNB-CU-UP E1 Setup procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-CP. The gNB-CU-CP responds with a GNB-CU-UP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-UP E1 SETUP REQUEST message contains the gNB-CU-UP Name IE the gNB-CU-CP may use this IE as a human readable name of the gNB-CU-UP.

If the Slice Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the NR CGI Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the QoS Parameters Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall take this IE into account.

For NG-RAN, if in GNB-CU-UP E1 SETUP REQUEST message, the NID IE is present in the Supported PLMNs IE, the gNB-CU-CP shall take this information into account.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: gNB-CU-UP E1 Setup procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the setup, it shall respond with a GNB-CU-UP E1 SETUP FAILURE and appropriate cause value.

If the GNB-CU-UP E1 SETUP FAILURE message includes the Time To Wait IE, the gNB-CU-UP shall wait at least for the indicated time before reinitiating the E1 setup towards the same gNB-CU-CP.

#### 8.2.3.4	Abnormal Conditions

If the first message received for a specific TNL association is not a GNB-CU-CP E1 SETUP REQUEST, GNB-CU-UP E1 SETUP RESPONSE, or GNB-CU-UP E1 SETUP FAILURE message then this shall be treated as a logical error.

If the gNB-CU-UP does not receive either GNB-CU-UP E1 SETUP RESPONSE message or GNB-CU-UP E1 SETUP FAILURE message, the gNB-CU-UP may reinitiate the gNB-CU-UP E1 Setup procedure towards the same gNB-CU-CP, provided that the content of the new GNB-CU-UP E1 SETUP REQUEST message is identical to the content of the previously unacknowledged GNB-CU-UP E1 SETUP REQUEST message.

If the gNB-CU-UP receives a GNB-CU-CP E1 SETUP REQUEST message from the peer entity on the same E1 interface:

-	In case the gNB-CU-UP answers with a GNB-CU-CP E1 SETUP RESPONSE message and receives a subsequent GNB-CU-UP E1 SETUP FAILURE message, the gNB-CU-UP shall consider the E1 interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.2.3.3.

-	In case the gNB-CU-UP answers with a GNB-CU-CP E1 SETUP FAILURE message and receives a subsequent GNB-CU-UP E1 SETUP RESPONSE message, the gNB-CU-UP shall ignore the GNB-CU-UP E1 SETUP RESPONSE message and consider the E1 interface as non operational.

Next Text Proposal to TS 38.463

### 8.2.4	gNB-CU-CP E1 Setup

#### 8.2.4.1	General

The purpose of the gNB-CU-CP E1 Setup procedure is to exchange application level data needed for the gNB-CU-CP and the gNB-CU-UP to correctly interoperate on the E1 interface. If the gNB-CU-CP initiates the first TNL association, it shall also initiate the gNB-CU-CP E1 Setup procedure.The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-CU-CP E1 Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending a GNB-CU-CP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-UP. The gNB-CU-UP responds with a GNB-CU-CP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-CP E1 SETUP REQUEST message contains the gNB-CU-CP Name IE the gNB-CU-UP may use this IE as a human readable name of the gNB-CU-CP.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall take this IE into account.

For NG-RAN, if in GNB-CU-CP E1 SETUP REQUEST RESPONSE message, the NID IE is present in the Supported PLMNs IE, the gNB-CU-CP shall take this information into account.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-CU-CP E1 Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot accept the setup, it shall respond with a GNB-CU-CP E1 SETUP FAILURE and appropriate cause value.

If the GNB-CU-CP E1 SETUP FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the E1 setup towards the same gNB-CU-UP.

#### 8.2.4.4	Abnormal Conditions

If the first message received for a specific TNL association is not a GNB-CU-UP E1 SETUP REQUEST, GNB-CU-CP E1 SETUP RESPONSE, or GNB-CU-CP E1 SETUP FAILURE message then this shall be treated as a logical error.

If the gNB-CU-CP does not receive either GNB-CU-CP E1 SETUP RESPONSE message or GNB-CU-CP E1 SETUP FAILURE message, the gNB-CU-CP may reinitiate the gNB-CU-CP E1 Setup procedure towards the same gNB-CU-UP, provided that the content of the new GNB-CU-CP E1 SETUP REQUEST message is identical to the content of the previously unacknowledged GNB-CU-CP E1 SETUP REQUEST message.

If the gNB-CU-CP receives a GNB-CU-UP E1 SETUP REQUEST message from the peer entity on the same E1 interface:

-	In case the gNB-CU-CP answers with a GNB-CU-UP E1 SETUP RESPONSE message and receives a subsequent GNB-CU-CP E1 SETUP FAILURE message, the gNB-CU-CP shall consider the E1 interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.2.4.3.

-	In case the gNB-CU-CP answers with a GNB-CU-UP E1 SETUP FAILURE message and receives a subsequent GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall ignore the GNB-CU-CP E1 SETUP RESPONSE message and consider the E1 interface as non operational.

Next Text Proposal to TS 38.463

### 8.2.5	gNB-CU-UP Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU-UP Configuration Update procedure is to update application level configuration data needed for the gNB-CU-UP and the gNB-CU-CP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU-UP Configuration Update procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP CONFIGURATION UPDATE message to the gNB-CU-CP including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU-CP responds with GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall interpret that the corresponding configuration data is not changed and shall continue to operate with the existing related configuration data.

If the Supported PLMNs IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall overwrite the whole list of information and store the corresponding information.

-	If the Slice Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the NR CGI Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the QoS Parameters Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall take this IE into account.

If the gNB-CU-UP ID IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall associate the TNLA to the E1 interface instance using the gNB-CU-UP ID.

If the GNB-CU-UP CONFIGURATION UPDATE message includes gNB-CU-UP TNLA To Remove List IE, and the Endpoint-IP-address-and-port IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU-UP TNLA To Remove List IE, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by both received TNL endpoints will be removed by the gNB-CU-UP. If the Endpoint-IP-address IE for one or both of the TNL endpoints is included in the gNB-CU-UP TNLA To Remove List IE in GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by the received endpoint IP address(es) will be removed by the gNB-CU-UP.

For NG-RAN, if in GNB-CU-UP CONFIGURATION UPDATE message, the NID IE is present in the Supported PLMNs IE, the gNB-CU-CP-CP shall take this information into account.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU-UP Configuration Update procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the update, it shall respond with a GNB-CU-UP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU-UP CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the GNB-CU-UP CONFIGURATION UPDATE message towards the same gNB-CU-CP.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

Next Text Proposal to TS 38.463

#### 9.2.1.4	GNB-CU-UP E1 SETUP REQUEST

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >NID | O |  | 9.3.3.X |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |

Next Text Proposal to TS 38.463

#### 9.2.1.8	GNB-CU-CP E1 SETUP RESPONSE

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >NID | O |  | 9.3.3.X |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |

Next Text Proposal to TS 38.463

#### 9.2.1.10	GNB-CU-UP CONFIGURATION UPDATE

This message is sent by the gNB-CU-UP to transfer updated information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | O |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| Supported PLMNs |  | 0..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >NID | O |  | 9.3.3.X |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| gNB-CU-UP TNLA To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-CU-UP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | - | - |
| >> TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-CP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-UP and the gNB-CU-CP. Value is 32. |

Next Text Proposal to TS 38.463

#### 9.3.1.x	NID

This information element indicates the Network identifier of the Stand-Alone NPN, as defined in TS 23.003 [23].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NID | M |  | BIT STRING (SIZE (xx)) |  |

End of Text Proposal to TS 38.463