3GPP TSG-RAN WG3 Meeting #105	R3-194312

# Ljubljana, Slovenien, 26th – 30th August 2019

Agenda Item:	17.2.4.2

Source:	Ericsson

Title: 	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity

Document for:	Discussion and Approval

# 1	Information

In [1], the Work Item Support of NR Industrial Internet of Things (IoT) is approved and one of the objectives is:

- The detailed objectives for NR PDCP duplication enhancements are:

- Specify PDCP duplication with up to 4 RLC entities configured by RRC in architectural combinations including CA only and NR-DC in combination with CA [RAN2, RAN3].

- Specify mechanisms relating to dynamic control of how a set or subset of configured RLC entities or legs are used for PDCP duplication [RAN2, RAN3].

- Specify enhancements for more resource efficient PDCP duplication by enhancing PDCP duplication activation/deactivation mechanisms (e.g. MAC CE based or based on UE configurable criteria), provided that complexity increase is reasonable. Per-packet selective duplication can also be considered. [RAN2].

- Specify enhancements for more efficient DL PDCP duplication without impacting the UE, provided that gains can be confirmed with a reasonable complexity. [RAN3].

- Specify enhancements to address potential impacts of higher-layer multi-connectivity based on SA2 progress and request [RAN2, RAN3].

In [2], for Key Issue 1, SA2 has concluded:

## Key Issue #1: Supporting high reliability by redundant transmission in user plane It is recommended that normative work proceed as follows: - Focusing on backhaul reliability improvements only i.e. without changes to the radio interface and associated protocols; and - Requiring single UE only i.e. no UE redundancy shall be specified; and - Introducing enablers in the network for: a) Redundancy of network nodes (UPF and gNB) and associated interface (N3), and concurrent PDU Sessions (see Solution #1); and b) GTP-U / TRANSPORT LAYER redundancy over N3 with single network nodes i.e. UPF and gNB (see Solutions #4, #7). No UE impact; c) Enablers to support appropriate gNB/UPF selection as applicable for a) and b). UE impact with a) shall be minimized. NOTE: An informative annex will be created during the normative work to document UE redundancy option with no 3GPP specification impact as enabled with Solution #2.

# 2	Discussion

At the last RAN3 meeting, we discussed more in detail how the solution 1 of Key Issue #1 works, refer to [2] and an LS is sent to SA2. A couple to questions were asked in the LS.

In the LS Reply [3], SA2 answers Q1 with the following:

Q1: We understand that the value of the RSN parameter is an index that indicates redundant user plane requirements for the PDU Sessions. Does it mean this index points to a particular UPF and a particular NG-RAN User Plane (UP) pair? Or it is only at the UPF side, i.e. NG-RAN is free to choose the user plane? For example, does it mean that UPF1/NG-RAN UP 1 are always selected when NG-RAN node receives RSN equals 1 and UPF2/NG-RAN UP 2 are always selected when NG-RAN node receives RSN equals 2 (i.e. UPF1/NG-RAN UP1 used by PDU session with RSN=1 for all pairs subject of redundancy) SA2 Response: The value of the RSN is not dependent on the selected UPF, even though the RSN may be also considered for UPF selection. Hence, the same RSN may be used with multiple UPFs. The value of the RSN parameter indicates to NG-RAN redundant user plane requirements for the PDU Sessions. NG-RAN selects NG-RAN UP by taking into account the RSN parameter and the associated RAN configuration to establish redundant User planes for the PDU Sessions. The NG-RAN UP selection for a PDU session with specific RSN value is decided by NG-RAN.

With this answer, we think it is clear that 5GC needs to indicate the RSN value to NG-RAN node.

NG-RAN selects NG-RAN UP by taking into account the RSN parameter and the associated RAN configuration to establish redundant User planes for the PDU Sessions. The NG-RAN UP selection for a PDU session with specific RSN value is decided by NG-RAN.

Observation 1: 5GC needs to indicate to NG-RAN node with the RSN value that the PDU sessions is set up for the Redundancy purpose, so that the NG-RAN node would use DC configuration and choose the user plane.

Proposal 1: RAN3 to agree to include the Requested Redundancy Sequence Numberin PDU Session resource setup procedures.

In the LS Reply [3], SA2 answers Q2 with the following:

Q2: TS 23.501 says that “NG RAN notifies CN about failure to establish dual connectivity and SMF based on local policy decides whether to continue with the PDU session or initiate release of the PDU session.” If NG-RAN receives PDU session setup for the second PDU session and the indicated RSN is not possible to be setup, RAN3 discussed two possibilities in NG-RAN: Alternative 1: NG-RAN node configures the second PDU session to the same RSN as the first PDU session (e.g. MN) Alternative 2: NG-RAN shall fail the second PDU session setup. Is alternative 1 allowed? If it is allowed, should SMF be notified and further decide whether to continue with the second PDU session or initiate release of the second PDU session? SA2 Response: SA2 has discussed the failure handling of the solution and has agreed to the CR in S2-1908296. The CR clarifies that NG-RAN local configuration indicates if NG-RAN shall reject the PDU session or continue establishing the PDU session in case redundant UP setup is not possible.

When relates to the failure handling during setup, NG-RAN has two options based on the local configurations. SA2 specifies that:

Using NG-RAN local configuration, NG-RAN determines whether the PDU Session shall be kept or not kept in case NG-RAN cannot satisfy the user plane requirements indicated by the RSN parameter by means of dual connectivity. If the PDU Session shall be kept, the PDU Session remains established even if the user plane requirements indicated by RSN cannot be satisfied. If the PDU Session shall not be kept when RSN requirement can not be fulfilled, the PDU Session shall be released/not established by NG-RAN.

Observation 2: NG-RAN node has two options when the indicated RSN cannot be satisfied by the local configuration.

Proposal 2: RAN3 to agree to include the Used Redundancy Sequence Number in the positive PDU Session resource response;

Proposal 3: RAN3 to agree to introduce a new cause value to indicate to 5GC that the failure is due to “RSN not available”.

In the LS Reply [3], SA2 answers Q3 with the following:

Q3: If the two SMFs are involved, how do the two SMFs coordinate the following? (PDU Session 1 and 2 are redundant) - RSN settings when the PDU Session1 is setup by SMF 1, and when PDU session 2 is setup by SMF 2; - When NG-RAN notifies SMF2 the failure of PDU Session 2, how does SMF1 know which one is PDU session 1 and makes the decision to release or continue the PDU session 1 SA2 Response: The two SMFs do not coordinate with each other. The RSN is determined by each SMF independently, based on the different combinations of DNN and S-NSSAI as provided by the UEs, where the SMFs may also consider subscription information and local policies. The failure handling is performed for each PDU Session separately and SMF1 is not notified of the failure status of SMF2 and vice versa.

RAN3 has been discussing if the redundant PDU Sessions ( PDU Session pair ) information is need.

As indicated by SA2, the two SMFs involved in the two redundant PDU Sessions are not coordinated, the PDU session pair information is thus not possible to be provided from 5GC to NG-RAN node. On the other hand, there is no need to get the PDU session pair information either, if NG-RAN takes the RSN information into consideration.

If the Redundancy Sequence Numbers (RSN) is attached to the PDU session, it is clear for NG-RAN node that for one value, one user plane is to be used; for another value, the other user plane is used.

Observation 3: the SMFs involved in the two redundant PDU Sessions are not coordinated.

Proposal 4: RAN3 to agree that the PDU Session pair information is not needed.

# 3	Proposal

Observation 1: 5GC needs to indicate to NG-RAN node with the RSN value that the PDU sessions is set up for the Redundancy purpose, so that the NG-RAN node would use DC configuration and choose the user plane.

Observation 2: NG-RAN node has two options when the indicated RSN cannot be satisfied by the local configuration.

Observation 3: the SMFs involved in the two redundant PDU Sessions are not coordinated.

Proposal 1: RAN3 to agree to include the Requested Redundancy Sequence Number in PDU Session resource setup procedures.

Proposal 2: RAN3 to agree to include the Used Redundancy Sequence Number in the positive PDU Session resource response;

Proposal 3: RAN3 to agree to introduce a new cause value to indicate to 5GC that the failure is due to “RSN not available”.

Proposal 4: RAN3 to agree that the PDU Session pair information is not needed.

CRs in [3] and [7] are submitted for the implementation of the solution.

# 4	Reference

[1]	RP-190276	New WID: Support of NR Industrial Internet of Things (IoT)

[2]	R3-193229	Summary of offline discussion on Solution 1 of Key Issue#1, Ericsson

[3]	R3-194251	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity, Ericsson

[4]	R3-194313	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity, Ericsson

[5]	R3-194314	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity, Ericsson

[6]	R3-194315	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity, Ericsson

[7]	R3-194252	Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity, Ericsson