3GPP TSG-RAN3 Meeting #105	R3-194418

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR |  | rev |  | Current version: | V15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.300 on support of NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN |  |  |  |  |  | Date: |  |  | 2019-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | R16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed to introduce NPN for RAN. There were agreements in SA2, which should be captured in RAN specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the general descriptions for NPN: - Overview to support SNPN and PNI-NPN - Mobility for both SNPN and PNI-NPN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The general description to support NPN is missing and the specification is imcomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.12(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1xCSFB	Circuit Switched Fallback to 1xRTT

5GC	5G Core Network

ABS	Almost Blank Subframe

AC	Access Category

ACK	Acknowledgement

ACLR	Adjacent Channel Leakage Ratio

AM	Acknowledged Mode

AMBR	Aggregate Maximum Bit Rate

ANDSF	Access Network Discovery and Selection Function

ANR	Automatic Neighbour Relation

ARP	Allocation and Retention Priority

ARQ	Automatic Repeat Request

AS	Access Stratum

AUL	Autonomous Uplink

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BL	Bandwidth reduced Low complexity

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

BSR	Buffer Status Report

C/I	Carrier-to-Interference Power Ratio

CA	Carrier Aggregation

CAZAC	Constant Amplitude Zero Auto-Correlation

CBC	Cell Broadcast Center

CC	Component Carrier

CG	Cell Group

CIF	Carrier Indicator Field

CIoT	Cellular Internet of Things

CMAS	Commercial Mobile Alert Service

CMC	Connection Mobility Control

C-plane	Control Plane

C-RNTI	Cell RNTI

CAG	Closed Access Group

CoMP	Coordinated Multi Point

CP	Cyclic Prefix

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRE	Cell Range Extension

CRS	Cell-specific Reference Signal

CSA	Common Subframe Allocation

CSG	Closed Subscriber Group

CSI	Channel State Information

CSI-IM	CSI interference measurement

CSI-RS	CSI reference signal

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCN	Dedicated Core Network

DeNB	Donor eNB

DFTS	DFT Spread OFDM

DL	Downlink

DMTC	Discovery Signal Measurement Timing Configuration

DRB	Data Radio Bearer

DRS	Discovery Reference Signal

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

DTX	Discontinuous Transmission

DwPTS	Downlink Pilot Time Slot

E-CID	Enhanced Cell-ID (positioning method)

E-RAB	E-UTRAN Radio Access Bearer

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EAB	Extended Access Barring

ECGI	E-UTRAN Cell Global Identifier

ECM	EPS Connection Management

EDT	Early Data Transmission

eHRPD	enhanced High Rate Packet Data

eIMTA	Enhanced Interference Management and Traffic Adaptation

EMM	EPS Mobility Management

eNB	E-UTRAN NodeB

EPC	Evolved Packet Core

EPDCCH	Enhanced Physical Downlink Control Channel

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

G-RNTI	Group RNTI

GBR	Guaranteed Bit Rate

GERAN	GSM EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GP	Guard Period

GRE	Generic Routing Encapsulation

GSM	Global System for Mobile communication

GUMMEI	Globally Unique MME Identifier

GUTI	Globally Unique Temporary Identifier

GWCN	GateWay Core Network

H-SFN	Hyper System Frame Number

HARQ	Hybrid ARQ

(H)eNB	eNB or HeNB

HO	Handover

HPLMN	Home Public Land Mobile Network

HRPD	High Rate Packet Data

HSDPA	High Speed Downlink Packet Access

ICIC	Inter-Cell Interference Coordination

IDC	In-Device Coexistence

IP	Internet Protocol

ISM	Industrial, Scientific and Medical

KPAS	Korean Public Alert System

L-GW	Local Gateway

LAA	Licensed-Assisted Access

LB	Load Balancing

LBT	Listen Before Talk

LCG	Logical Channel Group

LCR	Low Chip Rate

LCS	LoCation Service

LHN	Local Home Network

LHN ID	Local Home Network ID

LIPA	Local IP Access

LMU	Location Measurement Unit

LPPa	LTE Positioning Protocol Annex

LTE	Long Term Evolution

LWA	LTE-WLAN Aggregation

LWAAP	LTE-WLAN Aggregation Adaptation Protocol

LWIP	LTE WLAN Radio Level Integration with IPsec Tunnel

LWIP-SeGW	LWIP Security Gateway

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBR	Maximum Bit Rate

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCCH	Multicast Control Channel

MCE	Multi-cell/multicast Coordination Entity

MCG	Master Cell Group

MCH	Multicast Channel

MCS	Modulation and Coding Scheme

MDT	Minimization of Drive Tests

MeNB	Master eNB

MGW	Media Gateway

MIB	Master Information Block

MIMO	Multiple Input Multiple Output

MME	Mobility Management Entity

MMTEL	Multimedia telephony

MPDCCH	MTC Physical Downlink Control Channel

MSA	MCH Subframe Allocation

MSI	MCH Scheduling Information

MSP	MCH Scheduling Period

MTC	Machine-Type Communications

MTCH	Multicast Traffic Channel

MTSI	Multimedia Telephony Service for IMS

N2	Reference point between the NG-RAN and the AMF

NACK	Negative Acknowledgement

NAS	Non-Access Stratum

NB-IoT	Narrow Band Internet of Things

NCC	Next Hop Chaining Counter

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NG-RAN	NG Radio Access Network

NH	Next Hop key

NNSF	NAS Node Selection Function

NPBCH	Narrowband Physical Broadcast channel

NPDCCH	Narrowband Physical Downlink Control channel

NPDSCH	Narrowband Physical Downlink Shared channel

NPRACH	Narrowband Physical Random Access channel

NPUSCH	Narrowband Physical Uplink Shared channel

NPRS	Narrowband Positioning Reference Signal

NPSS	Narrowband Primary Synchronization Signal

NPN	Non-Public Network

NR	NR Radio Access

NRT	Neighbour Relation Table

NSSS	Narrowband Secondary Synchronization Signal

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OPI	Offload Preference Indicator

OTDOA	Observed Time Difference Of Arrival (positioning method)

P-GW	PDN Gateway

P-RNTI	Paging RNTI

PA	Power Amplifier

PAPR	Peak-to-Average Power Ratio

PBCH	Physical Broadcast CHannel

PBR	Prioritised Bit Rate

PCC	Primary Component Carrier

PCCH	Paging Control Channel

PCell	Primary Cell

PCFICH	Physical Control Format Indicator CHannel

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control CHannel

PDCP	Packet Data Convergence Protocol

PDN	Packet Data Network

PDSCH	Physical Downlink Shared CHannel

PDU	Protocol Data Unit

PHICH	Physical Hybrid ARQ Indicator CHannel

PHY	Physical layer

PLMN	Public Land Mobile Network

PMCH	Physical Multicast CHannel

PMK	Pairwise Master Key

PNI-NPN	Public Network Integrated NPN

PPPP	ProSe Per-Packet Priority

PPPR	ProSe Per-Packet Reliability

PRACH	Physical Random Access CHannel

PRB	Physical Resource Block

ProSe	Proximity based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSC	Packet Scheduling

PSCCH	Physical Sidelink Control CHannel

PSCell	Primary SCell

PSDCH	Physical Sidelink Discovery CHannel

PSK	Pre-Shared Key

PSM	Power Saving Mode

PSSCH	Physical Sidelink Shared CHannel

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared CHannel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QCI	QoS Class Identifier

QoE	Quality of Experience

QoS	Quality of Service

R-PDCCH	Relay Physical Downlink Control CHannel

RA-RNTI	Random Access RNTI

RAC	Radio Admission Control

RACH	Random Access Channel

RANAC	RAN-based Notification Area code

RAT	Radio Access Technology

RB	Radio Bearer

RBC	Radio Bearer Control

RCLWI	RAN Controlled LTE-WLAN Interworking

RF	Radio Frequency

RIBS	Radio-interface based synchronization

RIM	RAN Information Management

RLC	Radio Link Control

RMTC	RSSI Measurement Timing Configuration

RN	Relay Node

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNC	Radio Network Controller

RNL	Radio Network Layer

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

ROM	Receive Only Mode

RRC	Radio Resource Control

RRM	Radio Resource Management

RU	Resource Unit

S-GW	Serving Gateway

S-RSRP	Sidelink Reference Signal Received Power

S1-MME	S1 for the control plane

SAE	System Architecture Evolution

SAP	Service Access Point

SBCCH	Sidelink Broadcast Control Channel

SC-FDMA	Single Carrier – Frequency Division Multiple Access

SC-MCCH	Single Cell Multicast Control Channel

SC-MTCH	Single Cell Multicast Transport Channel

SC-N-RNTI	Single Cell Notification RNTI

SC-PTM	Single Cell Point To Multiploint

SC-RNTI	Single Cell RNTI

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCH	Synchronization Channel

SCTP	Stream Control Transmission Protocol

SD-RSRP	Sidelink Discovery Reference Signal Received Power

SDAP	Service Data Adaptation Protocol

SDF	Service Data Flow

SDMA	Spatial Division Multiple Access

SDU	Service Data Unit

SeGW	Security Gateway

SeNB	Secondary eNB

SFN	System Frame Number

SI	System Information

SI-RNTI	System Information RNTI

S1-U	S1 for the user plane

SIB	System Information Block

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

SL-BCH	Sidelink Broadcast Channel

SL-DCH	Sidelink Discovery Channel

SL-RNTI	Sidelink RNTI

SL-SCH	Sidelink Shared Channel

SNPN	Stand-alone NPN

SPDCCH	Short PDCCH

SPID	Subscriber Profile ID for RAT/Frequency Priority

SPT	Short Processing Time

SPUCCH	Short PUCCH

SR	Scheduling Request

SRB	Signalling Radio Bearer

sTAG	Secondary Timing Advance Group

STCH	Sidelink Traffic Channel

SU	Scheduling Unit

TA	Tracking Area

TAG	Timing Advance Group

TB	Transport Block

TCP	Transmission Control Protocol

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TFT	Traffic Flow Template

TM	Transparent Mode

TMGI	Temporary Mobile Group Identity

TNL	Transport Network Layer

TTI	Transmission Time Interval

U-plane	User plane

UAC	Unified Access Control

UDC	Uplink Data Compression

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UMTS	Universal Mobile Telecommunication System

UpPTS	Uplink Pilot Time Slot

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

V2I	Vehicle-to-Infrastructure

V2N	Vehicle-to-Network

V2P	Vehicle-to-Pedestrian

V2V	Vehicle-to-Vehicle

V2X	Vehicle-to-Everything

VRB	Virtual Resource Block

WLAN	Wireless Local Area Network

WT	WLAN Termination

WUS	Wake Up Signal

X2-C	X2-Control plane

X2 GW	X2 GateWay

X2-U	X2-User plane

Xw-C	Xw-Control plane

Xw-U	Xw-User plane

Next Change

## 4.11	Support for UE assistance information for local cache

UE assisted local cache is a solution to address long backhaul latency issue. The UE is allowed to transmit assistance information bit to eNB to enable the eNB to identify whether an uplink data needs to be transferred to the local cache entity, which may be co-sited with eNB or has direct connection with eNB, by operator implementation.

UE can report to the network its capability of supporting assistance information bit for local cache. If supported, the UE assisted local cache function can be activated by the eNB. After that, the UE may indicate the assistance information bit in the uplink PDCP PDU. Whether the UE indicates this assistance information bit is based on for instance the service from the application layer the UE requests that support local cache handling.

## 4.12	Support of NPN

E-UTRA connected to 5GC additionally supports NPN, which has the same characteristics as NPN of NR connected to 5GC, as specified in TS 38.300 [79].

End Change