3GPP TSG-RAN WG3 Meeting #105	R3-193390

Ljubljana, Slovenia, 26 – 30 Aug, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1337 | rev | - | Current version: | 13.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Handover Request Acknowledge |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13 |  |  |  |  |  | Date: |  |  | 2019-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Misalignment of the Tabular and asn.1 for HANDOVER REQUEST ACKNOWLEDGE message, in the asn.1 part there is one more optional IE with assigned criticality ignore, i.e. the id-SeNB-UE-X2AP-ID-Extension, and it should not be included in the message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add comment to asn.1 that the IE shall not be sent and shall be ignored, if received. Also add comment that the id-SeNB-UE-X2AP-ID-Extension should be removed in later versions of this message. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on Handover Request Acknowledge procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment of the the Tabular and asn.1 for HANDOVER REQUEST ACKNOWLEDGE message. HANDOVER REQUEST ACKNOWLEDGE message includes the IE which should be absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------Start of the change------

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//skip the unchanged part

-- **************************************************************

--

#### -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverRequestAcknowledge-IEs}},

...

}

HandoverRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID		PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID		PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-List						CRITICALITY ignore	TYPE E-RABs-Admitted-List	PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List		PRESENCE optional}|

{ ID id-TargeteNBtoSource-eNBTransparentContainer	CRITICALITY ignore	TYPE TargeteNBtoSource-eNBTransparentContainer	PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional}|

{ ID id-UE-ContextKeptIndicator						CRITICALITY ignore	TYPE UE-ContextKeptIndicator	PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension	PRESENCE optional}--This IE shall not be sent and shall be ignored, if received.--|

{ ID id-Old-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension	PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension	PRESENCE optional},

...

}

E-RABs-Admitted-List 		::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ItemIEs} }

E-RABs-Admitted-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-Item 	PRESENCE mandatory	}

}

E-RABs-Admitted-Item ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

uL-GTP-TunnelEndpoint			GTPtunnelEndpoint	OPTIONAL,

dL-GTP-TunnelEndpoint			GTPtunnelEndpoint	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-Item-ExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

--------End of the change------