3GPP TSG-RAN WG3 Meeting #105	R3-193554

# Ljubljana, Slovenia, 26 – 30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Activity Notification Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 2 (37.340) does not indicate support for the control of SN’s Activity Notification by the MN. However this is supported in stage 3 for both EN-DC and MR-DC (TS 36.413 and TS 38.413). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The existing flows related to Activity Notification are enhanced to show a step 0 (configuration of notification in the SN), or this is given as a pre-requisite. Note that the control via signalling is given as an example, since in principle this could be also done by configuration as a general setting in the SN. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release), as it aligns stage 2 with functionality (Notification Control) already supported in stage 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage 2 (TS 37.340) and stage 3 (TS 38.423 and TS 36.423). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.12.1, 10.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.12	Activity Notification

### 10.12.1	EN-DC

The Activity Notification function is used to report user plane activity within SN resources. It can either report inactivity or resumption of activity after inactivity was reported. In EN-DC the Activity Reporting is provided from the SN only. The MN may take further actions.

Figure 10.12.1-1: Support of Activity Notification in EN-DC

Support of Activity Notification in EN-DC is used to keep the MN informed about user traffic activity in resources owned by the SN. The MN may take appropriate action upon receiving such notification.

0.	SN Activity Notification is configured in the SN for the UE (e.g. during SN Addition or Modification).

1.	The SN informs the MN about user data inactivity of resources owned by the SN.

2.	The MN decides to keep SN resources.

3.	After a while the SN reports resumption of user plane activity.

### 10.12.2	MR-DC with 5GC

The Activity Notification function is used to report user plane activity within SN resources or to report a RAN Paging Failure event to the SN. It can either report inactivity or resumption of activity after inactivity was reported. In MR-DC with 5GC the Activity Reporting is provided from the SN only. The MN may take further actions. RAN Paging Failure Reporting is provided from the MN only.

MR-DC with 5GC Activity Notification

Figure 10.12.2-1: Support of Activity Notification in MR-DC with 5GC

0.	SN Activity Notification is configured in the SN for the UE (e.g. during SN Addition or Modification).

1.	The SN notifies the MN about user data inactivity.

2.	The MN decides further actions that impact SN resources (e.g. send UE to RRC_INACTIVE, bearer reconfiguration). In the case shown, MN takes no action.

3.	The SN notifies the MN that the (UE or PDU Session or QoS flow) is no longer inactive.

MR-DC with 5GC with RRC_INACTIVE

The Activity Notification function may be used to enable MR-DC with 5GC with RRC_INACTIVE operation. The MN node may decide, after inactivity is reported from the SN and also MN resources show no activity, to send the UE to RRC_INACTIVE. Resumption to RRC_CONNECTED may take place after activity is reported from the SN for SN terminated bearers.

Figure 10.12.2-2: Support of Activity Notification in MR-DC with 5GC with RRC_Inactive

Figure 10.12.2-2 shows how Activity Notification function interacts with NG-RAN functions for RRC_INACTIVE and SN Modification procedures in order to keep the higher layer MR-DC configuration established for UEs in RRC_INACTIVE, including NG and Xn interface C-plane, U-plane and bearer contexts established while lower layer MCG and SCG resources are released. When the UE transits successfully back to RRC_CONNECTED, lower layer MCG and SCG resources are established afterwards by means of RRC Connection Reconfiguration.

A pre-requisite for the above flow is that SN Activity Notification has been configured in the SN for the UE (e.g. during SN Addition or Modification).

Editors Note: RRC related details are FFS.

Editors Note: Figure 10.12.2-2 depicts the case where the UE when coming back to RRC_CONNECTED consumes radio resources from the same SN and MN as before it was sent to RRC_INACTIVE. Whether further scenarios need to be depicted needs further discussions.

1.	The SN notifies the MN about user data inactivity for SN terminated bearers.

2.	The MN decides to send the UE to RRC_INACTIVE.

3/4.	The MN triggers the MN initiated SN Modification procedure, requesting the SN to release lower layers.

5.	The UE is sent to RRC_INACTIVE.

6-8.	After a period of inactivity, upon activity notification from the SN, the UE returns to RRC_CONNECTED.

9/10.	The MN triggers the MN initiated SN Modification procedure to re-establish lower layers. The SN provides configuration data within an SN RRC configuration message.

11-14.	The RRCConnectionReconfiguration procedure commences.