3GPP TSG-RAN WG3 Meeting #105 	R3-193793

# Ljubljana, Slovenia, August 26-30 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0397 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Overload reporting enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  |  | Date: |  | 2019-08-26 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | New WI for support for SON/MDT Support for NR was agreed, which includes covering enhancements for Mobility Load Balancing (MLB) also in disaggregated architecture to optimize load balancing and load sharing. The existing mechanism for overload reporting is limited to a single indication of overloaded/not-overloaded status applicable to the condition of the entire gNB-DU level. This is suboptimal as it does not discriminate neither cell level nor slice. Thus, it is not possible to account for the conditions of an overloaded cell or slice accordingly at the gNB-CU when selecting cells. Hence, the GNB-DU STATUS INDICATION procedure should be enhanced to also report on cell and slice level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce cell level and slice level event-triggered load reporting over F1. Impact assessment towards the previous version of the specification (same release): This CR has an impact under functional and protocol point of view The impact can be considered isolated because the change is limited to event load reporting. This CR is ASN.1 backwards compatible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB-CU will not have knowledge of gNB-DU overload status per slice and cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7.2, 9.2.1.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

Beginning of Text Proposal to TS 38.473

### 8.2.7	gNB-DU Status Indication

#### 8.2.7.1	General

The purpose of the gNB-DU Status Indication procedure is informing the gNB-CU that the gNB-DU is overloaded so that overload reduction actions can be applied. The procedure uses non-UE associated signalling.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: gNB-DU Status Indication procedure

If the gNB-DU Overload Information IE in the GNB-DU STATUS INDICATION message indicates that the gNB-DU is overloaded, the gNB-CU shall apply overload reduction actions until informed, with a new GNB-DU STATUS INDICATION message, that the overload situation has ceased.

If the Cell Overload Information List IE is included in the GNB-DU STATUS INDICATION message, it indicates whether a cell is overloaded or not overloaded.

The detailed overload reduction policy is up to gNB-CU implementation.

#### 8.2.7.3	Abnormal Conditions

Void.

Next Text Proposal to TS 38.473

#### 9.2.1.15	GNB-DU STATUS INDICATION

This message is sent by the gNB-DU to indicate to the gNB-CU its status of overload.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | YES | reject |
| Cell Overload Information List |  | 0..1 |  |  | YES | ignore |
| >Cell Overload Information Item |  | 1 .. <maxCellingNB-DU> |  |  | EACH | ignore |
| >>NR Cell ID | M |  | NCGI 9.3.1.12 |  | – | – |
| >>Slice To Report |  | 1 |  | Cell ID list to which the request applies. | YES | ignore |
| >>>Slice To Report Item |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>>>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| >>>>Slice Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | – | – |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by an gNB-DU. Value is 512. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

End of Text Proposal to TS 38.473