3GPP TSG-RAN3 Meeting #105	R3-193921

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.413 | CR | 0004 | rev |  | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of N3IWF key |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 33.501, for the non-3GPP access to the 5G core network, the AMF delivers a key KN3IWF to the N3IWF. The key KN3IWF is then used by UE and N3IWF to complete authentication within IKEv2. Currently, in TS 38.413 the Security Key IE is used to apply security in the NG-RAN for different sceanrios as defined in TS 33. 501. The Security key IE can be used as N3IWF key but without any procedural texts. Therefore, it is proposed to add the description of security key usage for the non-3GPP access to the 5G core network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions that the Security Key IE in the applicable NGAP messages includes the N3IWF key for untrusted non-3GPP access. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the security key function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security key used for non-3GPP access is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

For the NGAP messages transferred between the N3IWF and the AMF, the following exceptions to the specification in TS 38.413 [2] shall be applied:

<Unchanged Text Omitted>

The User Location Information IE in the applicable NGAP messages includes the IP address and port number as specified in TS 38.413 [2].

The Security Key IE in the applicable NGAP messages includes the N3IWF key as specified in TS 38.413 [2].