3GPP TSG-RAN WG3 Meeting #105	R3-193949

# Ljubljana, Slovenia, 26-30 August 2019

Agenda Item:	31.3.3

Source:	Ericsson, ZTE, China Unicom, CMCC, China Telecom, Altiostar Networks

Title:	Trace (de)activation for F1AP and E1AP

Document for:	Discussion and Decision

# Introduction

In the previous meetings the issue of trace activation in various interfaces was discussed. The cases of X2AP and S1AP were agreed in R3-190779 and R3-191152. F1AP and E1AP were discussed for few more meetings and the following agreements was reached during RAN3#104:

CRs are technically agreeable; to be resubmitted and agreed at the next RAN3 meeting as Rel-16 Cat F CRs

# Discussion

The CRs covered by the RAN3#104 agreement are:

- F1: R3-192261 and R3-192840

- E1: R3-192262 and R3-192841

For this meeting it is proposed to agree these CRs as it is, expect for the following change, which does not change the procedures, but moves some text out of stage-3 to stage-2 (with a new CR) where it is more appropriate:

The gNB-CU shall, upon reception of a TRACE START message, initiate the Trace Start procedure by sending a TRACE START message to the gNB-DU for that specific UE.

This was done for both F1AP and E1AP CR. The resulting stage-2 CR is a call flow linking NG/X2 messages to F1/E1 messages:

Figure 1: Trace activation/deactivation over F1 and E1

1.	The gNB-CU-CP receives a TRACE START message from the AMF or the MeNB (in case of EN-DC).

2.	The gNB-CU-CP initiates the Trace Start procedure by sending a TRACE START message to the gNB-DU, the gNB-CU-UP or both.

3.	Upon reception of the TRACE START message, the gNB-DU, the gNB-CU-UP or both initiate the requested trace session.

4.	The gNB-CU-CP receives a DEACTIVATE TRACE message from the AMF or the MeNB (in case of EN-DC).

5.	The gNB-CU-CP initiates the Deactivate Trace procedure by sending a DEACTIVATE TRACE message to the gNB-DU, the gNB-CU-UP or both.

6. Upon reception of the DEACTIVATE TRACE message, the gNB-DU, the gNB-CU-UP or both stop the trace session for the indicated trace reference.

And the above text is a copy-paste of the text previously present in stage-3.

The remaining parts of the CRs are unchanged. Therefore, it is proposed to agree the stage-2 and stage-3 CRs including the changes described above.

Proposal 1: Agree stage-2 and stage-3 CRs supporting UE Trace Activation/Deactivation for F1 and E1

# Conclusion

In this contribution, we discussed the issue of including trace activation for F1 and E1 and we make the following proposal:

Proposal 1: Agree stage-2 and stage-3 CRs supporting UE Trace Activation/Deactivation for F1 and E1