3GPP TSG-RAN WG3 Meeting #105	R3-194276

# Ljubljana, Slovenien, 26th – 30th August 2019

Agenda Item:	8.1

Source:	Ericsson

Title: 	LTE-M Devices

Document for:	Discussion and Approval

# 1	Information

In the current specification, LTE-M indication is sent from RAN to MME in UE CAPABILITY INFO INDICATION.

In TS 36.413, the procedure is specified in below:

## 8.9 UE Capability Info Indication ### 8.9.1 General The purpose of the UE Capability Info Indication procedure is to enable the eNB to provide to the MME UE capability-related information. ### 8.9.2 Successful Operation Figure 8.9.2-1: UE Capability Info Indication procedure. Successful operation. The eNB controlling a UE-associated logical S1-connection initiates the procedure by sending a UE CAPABILITY INFO INDICATION message to the MME including the UE capability information. The UE CAPABILITY INFO INDICATION message may also include paging specific UE capability information within the UE Radio Capability for Paging IE. The UE capability information received by the MME shall replace previously stored corresponding UE capability information in the MME for the UE, as described in TS 23.401 [11]. If UE CAPABILITY INFO INDICATION message contains the LTE-M indication IE, the MME shall, if supported, use it according to TS 23.401 [11]. If the UE indicates the support for UE Application Layer Measurement, the eNB shall if supported include the UE Application Layer Measurement Capability IE in the UE CAPABILITY INFO INDICATION message. The MME shall, if supported, store and use the information when initiating UE Application Layer Measurement.

In TS 36.300, the capability handling is specified:

# 2	Discussion

## 2.1	LTE-M Indication in 5GS

In the context of 5G_CIoT, SA2 asks RAN2 and RAN3 [1] to investigate solutions that enable LTE-M to be identified from the Initial UE message that is sent from NG-RAN node to AMF.

In the context of the work on 5G_CIoT, SA2 would like to specify a solution to identify traffic from Cat.M1/M2 UEs in 5GC. While in EPC due to certain design constraints (no UE impacts, minimal RAN impacts, support for legacy UEs) the agreed solution (documented in TS 23.401 from Rel-15) allows identification of user plane traffic, it does not allow for using the LTE-M indication for selection of CN nodes. Given that these design constraints do not apply for 5G_CIOT, SA2 would like to define a solution that allows identification of LTE-M already when the UE first establishes the RRC connection. SA2 would therefore like to ask RAN2 and RAN3 to investigate solutions that enable LTE-M to be identified from the Initial UE message that is sent to AMF.

The solution needs the RRC support which is up to RAN2 decision. From RAN3 point of view, when the RRC part is in place, the LTE-M device can be identified in the Initial UE Message.

## 2.2	LTE-M Indication for LTE

In our view, if the solution proposed in the LS above is beneficial for NR, it’s beneficial to have the same solution for LTE.

During the initial attach procedure, UE does random access and the first message from eNB to CN is INITIAL UE MESSAGE. At this stage, eNB does not have any UE capability information. MME requests eNB to enquire UE capability in the Initial Context Setup Request. MME stores the acquired UE capability information until UE is detached.

SA2 introduced a new RAT type for LTE-M in order to charge the traffic from LTE-M devices separately and introduce requirements for identification and transparency when roaming. To fulfil those requirements, MME needs to indicate whether a UE is Cat-M1/M2 to S-GW/P-GW as a new RAT type in Create Session Request. MME also needs to block inbound roaming for Cat-M1/M2 devices in the initial attach if it is required by the operator.

With the mechanism currently specified in 3GPP, which uses Cat-M1/M2 as an indication to assign the new RAT type LTE-M in the core network, MME can only get explicit LTE-M indication in UE CAPABILITY INFO INDICATION. This would may lead to the following problems:

- In the initial attach procedure, when MME receives the Initial UE message, it is required to send Create Session Request to SGW to indicate that this is for an LTE-M1/M2 UE. However, UE capability, if it is not available at the MME prior to initial attach, can only be received from eNB after Initial Context Setup Request/Response (ICSR) or after DL NAS TRANSPORT (Attach Accept), which would be late. Note that even though it is possible for the eNB to indicate to the MME that UE is a BL UE, i.e., Cat-M1/M2, or UE in CE, it would not be possible for the CN to determine which, i.e., whether it is a BL UE or UE in CE, until after UE Capability Info Indication is sent from the eNB to the MME.

- When blocking of an inbound roaming device, a BL UE or UE in CE, is considered; the device will always be accepted for attach procedure and when MME eventually receives UE capability, i.e. after ICSR, the device may need to be detached. This means that the roaming device can access the network during initial attach even though it should be blocked.

For Dedicated CN (DCN) selection, CAT-M1/M2 devices may be routed to a wrong DCN during attach (UUT setting now is per IMSI level, however, this is in case an MBB USIM is inserted into a Cat-M1/M2 device).  After the MME in the wrong DCN receives UE capabilities, a re-route may be needed to steer the UE to the intended DCN. This would lead to additional network signalling and the reroute may not happen immediately.

Proposal 1: RAN3 to discuss and agree to include LTE-M Indication in Initial UE Message, as in [2] and [3].

Proposal 2: Send a LS Reply to SA2/RAN2 as in [4].

# 3	Proposal

Proposal 1: RAN3 to discuss and agree to include LTE-M Indication in Initial UE Message, as in [2] and [3].

Proposal 2: Send a LS Reply to SA2/RAN2 as in [4].

# 4	Reference

[1]	S2 1906831	LS on LTE-M identification in 5GC, SA2

[2]	R3-194277	LTE-M Indication, Ericsson

[3]	R3-194250	LTE-M Indication, Ericsson

[4]	R3-194278	Draft LS reply on LTE-M identification in 5GC, Ericsson