3GPP TSG-RAN WG3 #105	R3-194283

Ljubljana, SI, 26th August – 30st August 2019

Agenda Item:	10.2.1

Source:	Ericsson

Title:	RLM Wrong Beam Configuration Detection and Resolution

Document for:	Discussion, Decision

# Introduction

In RAN3 WG, several Self Organizing Network (SON) functions were discussed as part of the RAN centric data collection and utilization SI, and some of the SON functions were also agreed to be discussed in normative phase. One of the agreed functions is mobility robustness optimization (MRO). In RAN3#104 an overall solution (including measurement and signalling), based on the peculiarities of NR while taking LTE as baseline, has been agreed as part of a text proposal for MRO [1].

In this paper, we discuss how the measurement information agreed as part of radio link failure (RLF) report can be used to detect and resolve issues particularly in Radio Link Monitoring (RLM). We discuss how misconfiguration of RLM mechanisms may cause wrong classification of RLF at handover time.

# Discussion

Several new features are introduced in NR that could potentially influence the radio link failure (RLF) declaration at the UE.  Some of these features are listed below.

- Radio link monitoring (RLM) configuration

- Beam failure detection (BFD)

- Beam failure recovery (BFR) configuration

- Cell quality derivation parameters

- Beam based random access allocation

Hereafter, we analyze the RLM configuration and discuss use cases wherein measurements included in RLF reports [1] could be leveraged to enhance the RLM functionality in NR via optimizing its configuration parameters. This helps reducing the number of RLF happening due to the wrong RLM configuration. We further discuss how such wrong configurations may cause wrong classification of RLF at the time of handover.

## RLM resource configuration

RLM is a well-known procedure from LTE related to beam management, where the UE is monitoring the quality of its serving cell to determine if the network is unable to reach the UE. In LTE, the UE performs measurements on the CRS, and uses these measurements to estimate what the BLER of PDCCH would be if it were transmitted. In practice, the UE estimates the channel quality, e.g., SINR. The UE then triggers an internal out-of-sync (OOS) event if the BLER of a PDCCH received at this SINR level would be higher than 10%. When the UE has detected a certain number of consecutive OOS indications, the UE starts the T310 timer, and when the T310 expires, the UE declares RLF.

Hence, triggering of RLF events that may also be connected to mobility, may be due to misconfiguration of RLM resources. When radio link failure happens UE initiates re-establishment procedure and select a suitable cell to connect. If the UE declare the failure at cell boundaries it may re-establish a connection to another cell and this may be interpreted by the network as Too Early HO, Too Late HO, or HO to Wrong cell, while it was simply due to misconfiguration in how the UE measurements on its serving cell were set.  For this reason, RLM process should be considered as part of the MRO optimisation.

In NR as specified in TS 38.331 [2] failure detection resources are configured as part of RadioLinkMonitoringConfig as shown in Figure 1.

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=       SEQUENCE {

failureDetectionResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS   OPTIONAL,   -- Need N

failureDetectionResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id    OPTIONAL,-- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                                    OPTIONAL,   -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}            OPTIONAL,   -- Need R

...

}

RadioLinkMonitoringRS ::=           SEQUENCE {

radioLinkMonitoringRS-Id                RadioLinkMonitoringRS-Id,

purpose                             ENUMERATED {beamFailure, rlf, both},

detectionResource                   CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

...

}

Figure 1: Radio Link Monitoring Config information element used to configure failure detection resources.

RLF caused by RLM is a severe failure case, that essentially indicates the UE has no coverage from its serving cell. One situation where this may happen is if the network has failed to perform a handover to a new cell. After declaring RLF, the UE can re-establish connection with the new cell. However, misconfiguration of RLM resources may cause RLF while there are beams (not monitored by the UE) that could provide good enough cell quality to the UE.

Looking at Figure 1, we can focus on this IE: failureDetectionResourcesToAddModList:

This field is a list of reference signals for detecting beam failure (RadioLinkMonitoringRS). This set of resources are a subset of total number of beams providing the serving cell coverage.

The maximum number of beams that can be monitored for failure detection is 10 (see TS38.331). As shown, RadioLinkMonitoringRS can be configured either only for beam failure detection (BFD) or RLM purpose or for both of them. Usually BFD resources are a sub-set of RLM resources but they can be different as well.

It is therefore clear that the resources monitored for failure detection are less than the total number of beams a cell can adopt and because of that it is possible that such resources do not include the beams providing best quality signals to the UE.

- The maximum number of SSB beams that can be configured for RLM purposes is less than the maximum number of SSB beams that can be transmitted by the serving cell.

Therefore, due to the wrong beam configuration for such lower layer features such as RLM, there might be some cases where a UE may fail at RLM and declare RLF while the cell provides good coverage, as the UE is not configured with the beast beams for RLM purpose.

- RLM beams may not include the beams providing best quality signals to the UE. Such wrong configurations may cause RLF.

Wrong configurations of RLM resources and consequent RLFs may mislead the network in detecting Too Late HO or Too Early HO if RLF caused by wrong beam configuration takes place at cell boundaries. This is exemplified in the following.

Figure 2: Too late HO due to sub-optimal configuration of RLM resources.

In Figure 2 it is shown that a UE configured with sub-optimal RLM resources is subject to an RLF in its serving cell that may be followed by re-establishment in a different cell. The latter is especially true if the UE is moving towards the target cell at high speed where radio signal fluctuation is more abrupt. However, the RLF could have been avoided if the RLM resources were optimally configured (e.g. if they included Beam 3). Here the RAN node owning the original cell may wrongly interpret the failure as Too Late HO while the issue is related to the wrong RLM resource configuration.

In Figure 3 it is shown how a UE may be successfully handed over to a target cell (Cell A, in the figure). However, due to sub-optimal configuration of RLM resources, the UE may be subject to RLF at the target cell. The UE might re-establish to its source cell (i.e. Cell B). The network may interpret this event as Too Early HO and may decide to change CIO from source to target, while the issue comes from wrong configuration of resources assigned to the RLM at target cell. Changing of CIO for such failures not only fails to mitigate this issue but also may cause additional failures (by changing the optimal HO point).  The failure might have been avoided if the RLM resources in Cell A were optimally configured (e.g. if they included Beam 3).

Figure 3: Too early HO due to sub-optimal configuration of RLM resources

- RLF caused by wrong beam configuration for RLM may be wrongly interpreted as Too Early and Too Late HOs.

In this regard, we propose that, when an RLF occurs, the UE performs measurement not only on RLM resources but also resources not used for RLM purpose. This information is proposed to be part of the RLF Report, as captured in TR37.816.

- RAN3 to base the MRO solution on reporting of measurement information of both failure detection resources (SSB/CSI-RS beams) assigned for RLM purpose and resources (SSB/CSI-RS beams) not assigned for RLM purpose, as part of RLF report.

Once the RLF report is ready at the UE, it can be retrieved by the network by means of the UE information request/response procedure. The RLF report can be also forwarded to the serving cell over Xn (if required) in case to avoid wrong detection of Too Early HO. This is shown in Figure 2 and Figure 3. In DU-CU split architecture, RLF report may need to be forwarded from CU to DU that may need to detect the configuration issue and perform required reconfigurations. For example, beam measurements revealing the need for a beam reconfigurations could be of great help to the gNB-DU.

- RAN 3 to agree that CU forwards the RLF report to DU for further analysis on detection of the configuration issues and performing the required reconfiguration actions.

# Conclusion

In section 2 we made the following observations:

Observation 1	The maximum number of SSB beams that can be configured for RLM purposes is less than the maximum number of SSB beams that can be transmitted by the serving cell.

Observation 2	RLM beams may not include the beams providing best quality signals to the UE. Such wrong configurations may cause RLF.

Observation 3	RLF caused by wrong beam configuration for RLM may be wrongly interpreted as Too Early and Too Late HOs.

Based on the discussion in section 2 we propose the following:

- RAN3 agree reporting measurement information of both failure detection resources (SSB/CSI-RS beams) assigned for RLM purpose and resources (SSB/CSI-RS beams) not assigned for RLM purpose, as part of RLF report.

- RAN 3 agree that CU forwards the RLF report to DU for further analysis on detection of the configuration issues and performing the required reconfiguration actions.

# References

TR 37.816, 3GPP WG RAN3, Reno, US,

38.331, 3GPP TSG RAN, NR RRC Protocol specification Release-15, v15.4.0, 2019-01.