3GPP TSG-RAN WG3 Meeting #105	R3-194427

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.473 | CR |  | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interface Management procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, TIM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo-Core |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The agreed Interface Management procedures in RAN3 have not been captured in the current TS37.473. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Interface Management related pocdures, including: - Reset procedure - Error indication procedure - W1 setup procedure - ng-eNB-DU Configuration Update proceudre - ng-eNB-CU Configuration Update procedure - ng-eNB-DU Resource Coordination procedure - ng-eNB-DU Status Indication procedure Corresponding tabulars are also introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interface Management in case of disaggregated Ng-eNB architecture does not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRAN); Radio Resource Control (RRC) Protocol Specification".

[x2]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[x3]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[x4]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[x5]	GPP TS 38.423: "NG-RAN; Xn application protocol (XnAP)".

[x6]	3GPP TS 23.501: "System Architecture for the 5G System".

[x7]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.X	Interface Management procedures

8.X.1	Reset

8.X.1.1	General

The purpose of the Reset procedure is to initialise or re-initialise t3he W1AP UE-related contexts, in the event of a failure in the ng-eNB-CU or ng-eNB-DU. This procedure does not affect the application level configuration data exchanged during, e.g., the W1 Setup procedure.

The procedure uses non-UE associated signalling.

8.X.1.2	Successful Operation

8.X.1.2.1	Reset Procedure Initiated from the ng-eNB-CU

Figure 8.X.1.2.1-1: Reset procedure initiated from the ng-eNB-CU. Successful operation

In the event of a failure at the ng-eNB-CU, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the ng-eNB-DU.

At reception of the RESET message the ng-eNB-DU shall release all allocated resources on W1 and radio resources related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the indicated UE contexts including W1AP ID.

After the ng-eNB-DU has released all assigned W1 resources and the UE W1AP IDs for all indicated UE associations which can be used for new UE-associated logical W1-connections over the W1 interface, the ng-eNB-DU shall respond with the RESET ACKNOWLEDGE message. The ng-eNB-DU does not need to wait for the release of radio resources to be completed before returning the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical W1-connection list IE, then:

-	The ng-eNB-DU shall use the ng-eNB-CU UE W1AP ID IE and/or the ng-eNB-DU UE W1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The ng-eNB-DU shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical W1-connection Item IE in the UE-associated logical W1-connection list IE. The UE-associated logical W1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical W1-connections. Empty UE-associated logical W1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the ng-eNB-CU UE W1AP ID IE is included in the UE-associated logical W1-connection Item IE for a UE association, the ng-eNB-DU shall include the ng-eNB-CU UE W1AP ID IE in the corresponding UE-associated logical W1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the ng-eNB-DU UE W1AP ID IE is included in the UE-associated logical W1-connection Item IE for a UE association, the ng-eNB-DU shall include the ng-eNB-DU UE W1AP ID IE in the corresponding UE-associated logical W1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same W1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

8.X.1.2.2	Reset Procedure Initiated from the ng-eNB-DU

Figure 8.X.1.2.2-1: Reset procedure initiated from the ng-eNB-DU. Successful operation

In the event of a failure at the ng-eNB-DU, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the ng-eNB-CU.

At reception of the RESET message the ng-eNB-CU shall release all allocated resources on W1 related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the W1AP ID for the indicated UE associations.

After the ng-eNB-CU has released all assigned W1 resources and the UE W1AP IDs for all indicated UE associations which can be used for new UE-associated logical W1-connections over the W1 interface, the ng-eNB-CU shall respond with the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical W1-connection list IE, then:

-	The ng-eNB-CU shall use the ng-eNB-CU UE W1AP ID IE and/or the ng-eNB-DU UE W1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The ng-eNB-CU shall in the RESET ACKNOWLEDGE message include, for each UE association to be reset, the UE-associated logical W1-connection Item IE in the UE-associated logical W1-connection list IE. The UE-associated logical W1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical W1-connections. Empty UE-associated logical W1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the ng-eNB-CU UE W1AP ID IE is included in the UE-associated logical W1-connection Item IE for a UE association, the ng-eNB-CU shall include the ng-eNB-CU UE W1AP ID IE in the corresponding UE-associated logical W1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the ng-eNB-DU UE W1AP ID IE is included in a UE-associated logical W1-connection Item IE for a UE association, the ng-eNB-CU shall include the ng-eNB-DU UE W1AP ID IE in the corresponding UE-associated logical W1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same W1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

8.X.1.3	Abnormal Conditions

Not applicable.

8.X.2	Error Indication

8.X.2.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE associated signalling, then the Error Indication procedure uses UE associated signalling. Otherwise the procedure uses non-UE associated signalling.

8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Error Indication procedure, ng-eNB-CU originated. Successful operation

Figure 8.X.2.2-2: Error Indication procedure, ng-eNB-DU originated. Successful operation

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE. In case the Error Indication procedure is triggered by utilising UE associated signalling the ng-eNB-CU UE W1AP ID IE and ng-eNB-DU UE W1AP ID IE shall be included in the ERROR INDICATION message. If one or both of the ng-eNB-CU UE W1AP ID IE and the ng-eNB-DU UE W1AP ID IE are not correct, the cause shall be set to appropriate value, e.g., "Unknown or already allocated ng-eNB-CU UE W1AP ID", "Unknown or already allocated ng-eNB-DU UE W1AP ID" or "Unknown or inconsistent pair of UE W1AP ID".

8.X.2.3	Abnormal Conditions

Not applicable.

8.X.3	W1 Setup

8.X.3.1	General

The purpose of the W1 Setup procedure is to exchange application level data needed for the ng-eNB-DU and the ng-eNB-CU to correctly interoperate on the W1 interface. The procedure uses non-UE associated signalling. This procedure shall be the first W1AP procedure triggered for the W1-C interface instance after a TNL association has become operational.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the W1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

8.X.3.2	Successful Operation

Figure 8.X.3.2-1: W1 Setup procedure: Successful Operation

The ng-eNB-DU initiates the procedure by sending a W1 SETUP REQUEST message including the appropriate data to the ng-eNB-CU. The ng-eNB-CU responds with a W1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the W1 interface is operational and other W1 messages may be exchanged.

If the W1 SETUP REQUEST message contains the ng-eNB-DU Served Cells List IE, the ng-eNB-CU shall take into account as specified in TS 38.401 [x3].

For NG-RAN, the ng-eNB-DU shall include the ng-eNB-DU System Information IE and the TAI Slice Support List IE.

The ng-eNB-CU may include the Cells to be Activated List IE in the W1 SETUP RESPONSE message. The Cells to be Activated List IE includes a list of cells that the ng-eNB-CU requests the ng-eNB-DU to activate. The ng-eNB-DU shall activate the cells included in the Cells to be Activated List IE and reconfigure the physical cell identity for cells for which the E-UTRAN PCI IE is included.

For NG-RAN, the ng-eNB-CU shall include the ng-eNB-CU System Information IE in the W1 SETUP RESPONSE message.

For NG-RAN, the ng-eNB-DU may include the RAN Area Code IE in the W1 SETUP REQUEST message. The ng-eNB-CU may use it according to TS 38.300 [x4].

For NG-RAN, the ng-eNB-CU may include Available PLMN List IE, and optionally also Extended Available PLMN List IE, if the available PLMN(s) are different from what ng-eNB-DU has provided in W1 SETUP REQUEST message, ng-eNB-DU shall take this into account and only broadcast the PLMN(s) included in the received Available PLMN list(s).

8.X.3.3	Unsuccessful Operation

Figure 8.X.3.3-1: W1 Setup procedure: Unsuccessful Operation

If the ng-eNB-CU cannot accept the setup, it should respond with a W1 SETUP FAILURE and appropriate cause value.

8.X.3.4	Abnormal Conditions

Not applicable.

8.X.4	ng-eNB-DU Configuration Update

8.X.4.1	General

The purpose of the ng-eNB-DU Configuration Update procedure is to update application level configuration data needed for the ng-eNB-DU and the ng-eNB-CU to interoperate correctly on the W1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.X.4.2	Successful Operation

Figure 8.X.4.2-1: ng-eNB-DU Configuration Update procedure: Successful Operation

The ng-eNB-DU initiates the procedure by sending an ng-eNB-DU CONFIGURATION UPDATE message to the ng-eNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The ng-eNB-CU responds with NG-ENB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the NG-ENB-DU CONFIGURATION UPDATE message, the ng-eNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the W1-C interface with the existing related configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If Served Cells To Add Item IE is contained in the NG-ENB-DU CONFIGURATION UPDATE message, the ng-eNB-CU shall add cell information according to the information in the Served Cell Information IE. For NG-RAN, the ng-eNB-DU shall include the ng-eNB-DU System Information IE.

If Served Cells To Modify Item IE is contained in the NG-ENB-DU CONFIGURATION UPDATE message, the ng-eNB-CU shall modify information of cell indicated by Old E-UTRAN CGI IE according to the information in the Served Cell Information IE and overwrite the served cell information for the affected served cell. Further, if the ng-eNB-DU System Information IE is present the ng-eNB-CU shall store and replace any previous information received.

If Served Cells To Delete Item IE is contained in the NG-ENB-DU CONFIGURATION UPDATE message, the ng-eNB-CU shall delete information of cell indicated by Old E-UTRAN CGI IE.

If Cells Status Item IE is contained in the NG-ENB-DU CONFIGURATION UPDATE message, the ng-eNB-CU shall update the information about the cells, as described in TS 38.401 [x3].

If Cells to be Activated List Item IE is contained in the NG-ENB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the ng-eNB-DU shall activate the cell indicated by E-UTRAN CGI IE and reconfigure the physical cell identity for cells for which the E-UTRAN PCI IE is included.

If Cells to be Activated List Item IE is contained in the NG-ENB-DU CONFIGURATION UPDATE ACKNOWLEDGE message and the indicated cells are already activated, the ng-eNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Deactivated List Item IE is contained in the NG-ENB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the ng-eNB-DU shall deactivate all the cells with E-UTRAN CGI listed in the IE.

For NG-RAN, the ng-eNB-CU shall include the ng-eNB-CU System Information IE in the NG-ENB-DU CONFIGURATION UPDATE ACKNOWLEDGE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

For NG-RAN, the ng-eNB-DU may include the RAN Area Code IE in the NG-ENB-DU CONFIGURATION UPDATE message. The ng-eNB-CU shall store and replace any previously provided RAN Area Code IE by the received RAN Area Code IE.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in NG-ENB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the ng-eNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

8.X.4.3	Unsuccessful Operation

Figure 8.X.4.3-1: ng-eNB-DU Configuration Update procedure: Unsuccessful Operation

If the ng-eNB-CU cannot accept the update, it shall respond with an ng-eNB-DU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

8.X.4.4	Abnormal Conditions

Not applicable.

8.X.5	ng-eNB-CU Configuration Update

8.X.5.1	General

The purpose of the ng-eNB-CU Configuration Update procedure is to update application level configuration data needed for the ng-eNB-DU and ng-eNB-CU to interoperate correctly on the W1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.X.5.2	Successful Operation

Figure 8.X.5.2-1: ng-eNB-CU Configuration Update procedure: Successful Operation

The ng-eNB-CU initiates the procedure by sending an ng-eNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the ng-eNB-DU. The ng-eNB-DU responds with an ng-eNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the NG-ENB-CU CONFIGURATION UPDATE message, the ng-eNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the W1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the NG-ENB-CU CONFIGURATION UPDATE message, the ng-eNB-DU shall activate the cell indicated by E-UTRAN CGI IE and reconfigure the physical cell identity for which the E-UTRAN PCI IE is included.

If Cells to be Deactivated List Item IE is contained in the NG-ENB-CU CONFIGURATION UPDATE message, the ng-eNB-DU shall deactivate the cell indicated by E-UTRAN CGI IE.

If Cells to be Activated List Item IE is contained in the NG-ENB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the ng-eNB-DU shall update the cell information received in Cells to be Activated List Item IE.

For NG-RAN, the ng-eNB-CU shall include the ng-eNB-CU System Information IE in the NG-ENB-CU CONFIGURATION UPDATE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in NG-ENB-CU CONFIGURATION UPDATE message, the ng-eNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Cells Failed to be Activated Item IE is contained in the NG-ENB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the ng-eNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [x3].

8.X.5.3	Unsuccessful Operation

Figure 8.X.5.3-1: ng-eNB-CU Configuration Update: Unsuccessful Operation

If the ng-eNB-DU cannot accept the update, it shall respond with an ng-eNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

8.X.5.4	Abnormal Conditions

Not applicable.

8.X.6	ng-eNB-DU Resource Coordination

8.X.6.1	General

The purpose of the ng-eNB-DU Resource Coordination procedure is to enable coordination of radio resource allocation between an ng-eNB-CU and an ng-eNB-DU for the purpose of spectrum sharing between E-UTRA and NR. This procedure is to be used only for the purpose of spectrum sharing between E-UTRA and NR.

The procedure uses non-UE-associated signalling.

8.X.6.2	Successful Operation

Figure 8.X.X.2-1: ng-eNB-DU Resource Coordination, successful operation

An ng-eNB-CU initiates the procedure by sending the NG-ENB-DU RESOURCE COORDINATION REQUEST message to an ng-eNB-DU over the W1 interface.

The ng-eNB-DU extracts the E-UTRA – NR Cell Resource Coordination Request Container IE and it replies by sending the NG-ENB-DU RESOURCE COORDINATION RESPONSE message.

In case of NR-initiated ng-eNB-DU Resource Coordination procedure, the Ignore Coordination Request Container IE shall be present and set to “yes” and the E-UTRA – NR Cell Resource Coordination Request Container IE in the NG-ENB-DU RESOURCE COORDINATION REQUEST message shall be ignored.

8.X.7	ng-eNB-DU Status Indication

8.X.7.1	General

The purpose of the ng-eNB-DU Status Indication procedure is informing the ng-eNB-CU that the ng-eNB-DU is overloaded so that overload reduction actions can be applied. The procedure uses non-UE associated signalling.

8.X.7.2	Successful Operation

Figure 8.X.X.2-1: ng-eNB-DU Status Indication procedure

If the ng-eNB-DU Overload Information IE in the NG-ENB-DU STATUS INDICATION message indicates that the ng-eNB-DU is overloaded, the ng-eNB-CU shall apply overload reduction actions until informed, with a new NG-ENB-DU STATUS INDICATION message, that the overload situation has ceased.

The detailed overload reduction policy is up to ng-eNB-CU implementation.

8.X.7.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

9.x1.y1	Interface Management messages

9.x1.y1.1	RESET

This message is sent by both the ng-eNB-CU and the ng-eNB-DU and is used to request that the W1 interface, or parts of the W1 interface, to be reset.

Direction: ng-eNB-CU  ng-eNB-DU and ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Cause | M |  | 9.x2.y2.2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >W1 interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all,...) |  | - |  |
| >Part of W1 interface |  |  |  |  |  |  |
| >>UE-associated logical W1-connection list |  | 1 |  |  | - |  |
| >>>UE-associated logical W1-connection Item |  | 1 .. <maxnoofIndividualW1ConnectionsToReset> |  |  | EACH | reject |
| >>>> ng-eNB-CU UE W1AP ID | O |  | 9.x2.y2.4 |  | - |  |
| >>>> ng-eNB-DU UE W1AP ID | O |  | 9.x2.y2.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualW1ConnectionsToReset | Maximum no. of UE-associated logical W1-connections allowed to reset in one message. Value is 65536. |

9.x1.y1.2	RESET ACKNOWLEDGE

This message is sent by both the ng-eNB-CU and the ng-eNB-DU as a response to a RESET message.

Direction: ng-eNB-DU  ng-eNB-CU and ng-eNB-CU  ng-eNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| UE-associated logical W1-connection list |  | 0..1 |  |  | YES | ignore |
| >UE-associated logical W1-connection Item |  | 1 .. <maxnoofIndividualW1ConnectionsToReset> |  |  | EACH | ignore |
| >>ng-eNB-CU UE W1AP ID | O |  | 9.x2.y2.4 |  | - |  |
| >>ng-eNB-DU UE W1AP ID | O |  | 9.x2.y2.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualW1ConnectionsToReset | Maximum no. of UE-associated logical W1-connections allowed to reset in one message. Value is 65536. |

9.x1.y1.3	ERROR INDICATION

This message is sent by both the ng-eNB-CU and the ng-eNB-DU and is used to indicate that some error has been detected in the node.

Direction: ng-eNB-CU  ng-eNB-DU and ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | ignore |
| Transaction ID | M |  | 9.x2.y2.23 | This IE shall be ignored if received in UE associated signalling message. | YES | reject |
| ng-eNB-CU UE W1AP ID | O |  | 9.x2.y2.4 |  | YES | ignore |
| ng-eNB-DU UE W1AP ID | O |  | 9.x2.y2.5 |  | YES | ignore |
| Cause | O |  | 9.x2.y2.2 |  | YES | ignore |

9.x1.y1.4	W1 SETUP REQUEST

This message is sent by the ng-eNB-DU to transfer information associated to a W1-C interface instance.

Direction: ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| ng-eNB-DU ID | M |  | 9.x2.y2.9 |  | YES | reject |
| ng-eNB-DU Served Cells List |  | 0.. 1 |  | List of cells configured in the ng-eNB-DU | YES | reject |
| >ng-eNB-DU Served Cells Item |  | 1.. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.x2.y2.10 | Information about the cells configured in the ng-eNB-DU | - |  |
| >>ng-eNB-DU System Information | O |  | 9.x2.y2.18 | RRC container with system information owned by ng-eNB-DU | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinng-eNBDU | Maximum no. cells that can be served by an ng-eNB-DU. Value is 512. |

9.x1.y1.5	W1 SETUP RESPONSE

This message is sent by the ng-eNB-CU to transfer information associated to a W1-C interface instance.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Cells to be Activated List |  | 0.. 1 |  |  | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellinng-eNBDU> |  | List of cells to be activated | EACH | reject |
| >> E-UTRAN CGI | M |  | 9.x2.y2.12 |  | - |  |
| >> E-UTRAN PCI | O |  | INTEGER (0..503) | Physical Cell ID | - |  |
| >>ng-eNB-CU System Information | O |  | 9.x2.y2.42 | RRC container with system information owned by ng-eNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.x2.y2.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.x2.y2.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellinng-eNBDU | Maximum no. cells that can be served by an ng-eNB-DU. Value is 512. |

9.x1.y1.6	W1 SETUP FAILURE

This message is sent by the ng-eNB-CU to indicate W1 Setup failure.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Cause | M |  | 9.x2.y2.2 |  | YES | ignore |

9.x1.y1.7	NG-ENB-DU CONFIGURATION UPDATE

This message is sent by the ng-eNB-DU to transfer updated information associated to a W1-C interface instance.

Direction: ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the ng-eNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.x2.y2.10 | Information about the cells configured in the ng-eNB-DU | - |  |
| >>ng-eNB-DU System Information | O |  | 9.x2.y2.18 | RRC container with system information owned by ng-eNB-DU | - |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the ng-eNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >>Old E-UTRAN CGI | M |  | E-UTRAN CGI 9.x2.y2.12 |  | - |  |
| >>Served Cell Information | M |  | 9.x2.y2.10 | Information about the cells configured in the ng-eNB-DU | - |  |
| >>ng-eNB-DU System Information | O |  | 9.x2.y2.18 | RRC container with system information owned by ng-eNB-DU | - |  |
| Served Cells To Delete List |  | 0..1 |  | Complete list of deleted cells served by the ng-eNB-DU | YES | reject |
| >Served Cells To Delete Item |  | 1.. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >>Old E-UTRAN CGI | M |  | E-UTRAN CGI 9.x2.y2.12 |  | - |  |
| Cells Status List |  | 0..1 |  | Complete list of active cells | YES | reject |
| > Cells Status Item |  | 0 .. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >> E-UTRAN CGI | M |  | 9.x2.y2.12 |  | - |  |
| >> Service Status | M |  | 9.x2.y2.68 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinng-eNBDU | Maximum no. cells that can be served by an ng-eNB-DU. Value is 512. |

9.x1.y1.8	NG-ENB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by an ng-eNB-CU to an ng-eNB-DU to acknowledge update of information associated to a W1-C interface instance.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Cells to be Activated List |  | 0.. 1 |  | List of cells to be activated | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >> E-UTRAN CGI | M |  | 9.x2.y2.12 |  | - |  |
| >> E-UTRAN PCI | O |  | INTEGER (0..503) | Physical Cell ID | - |  |
| >> ng-eNB-CU System Information | O |  | 9.x2.y2.42 | RRC container with system information owned by ng-eNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.x2.y2.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.x2.y2.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| Cells to be Deactivated List |  | 0.. 1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >> E-UTRAN CGI | M |  | 9.x2.y2.12 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellinng-eNBDU | Maximum no. cells that can be served by an ng-eNB-DU. Value is 512. |

9.x1.y1.9	NG-ENB-DU CONFIGURATION UPDATE FAILURE

This message is sent by the ng-eNB-CU to indicate ng-eNB-DU Configuration Update failure.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Cause | M |  | 9.x2.y2.2 |  | YES | ignore |

9.x1.y1.10	NG-ENB-CU CONFIGURATION UPDATE

This message is sent by the ng-eNB-CU to transfer updated information associated to a W1-C interface instance.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >> E-UTRAN CGI | M |  | 9.x2.y2.12 |  | - |  |
| >> E-UTRAN PCI | O |  | INTEGER (0..503) | Physical Cell ID | - |  |
| >> ng-eNB-CU System Information | O |  | 9.x2.y2.42 | RRC container with system information owned by ng-eNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.x2.y2.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.x2.y2.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >> E-UTRAN CGI | M |  | 9.x2.y2.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinng-eNBDU | Maximum nunmerbs of cells that can be served by an ng-eNB-DU. Value is 512. |

9.x1.y1.11	NG-ENB-CU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by an ng-eNB-DU to an ng-eNB-CU to acknowledge update of information associated to a W1-C interface instance.

Direction: ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Cells Failed to be Activated List |  | 0..1 |  | List of cells which are failed to be activated | YES | reject |
| >Cells Failed to be Activated Item |  | 1.. <maxCellinng-eNBDU> |  |  | EACH | reject |
| >> E-UTRAN CGI | M |  | 9.x2.y2.12 |  | - |  |
| >>Cause | M |  | 9.x2.y2.2 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinng-eNBDU | Maximum no. cells that can be served by an ng-eNB-DU. Value is 512. |

9.x1.y1.12	NG-ENB-CU CONFIGURATION UPDATE FAILURE

This message is sent by the ng-eNB-DU to indicate ng-eNB-CU Configuration Update failure.

Direction: ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Cause | M |  | 9.x2.y2.2 |  | YES | ignore |

9.x1.y1.13	NG-ENB-DU RESOURCE COORDINATION REQUEST

This message is sent by an ng-eNB-CU to an ng-eNB-DU, to express the desired resource allocation for data traffic, for the sake of resource coordination.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| Request type | M |  | ENUMERATED (offer, execution, ...) |  | YES | reject |
| E-UTRA – NR Cell Resource Coordination Request Container | M |  | OCTET STRING | Includes the XNAP E-UTRA – NR CELL RESOURCE COORDINATION REQUEST message as defined in subclause 9.1.2.23 in TS 38.423 [x5]. | YES | reject |
| Ignore Coordination Request Container | O |  | ENUMERATED (yes, ...) |  | YES | reject |

9x1.y1.14	NG-ENB-DU RESOURCE COORDINATION RESPONSE

This message is sent by an ng-eNB-DU to an ng-eNB-CU, to express the desired resource allocation for data traffic, as a response to the NG-ENB-DU RESOURCE COORDINATION REQUEST.

Direction: ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | reject |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| E-UTRA – NR Cell Resource Coordination Response Container | M |  | OCTET STRING | Includes the XNAP E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE message as defined in subclause 9.1.2.24 in TS 38.423 [x5]. | YES | reject |

9.x1.y1.15	NG-ENB-DU STATUS INDICATION

This message is sent by the ng-eNB-DU to indicate to the ng-eNB-CU its status of overload.

Direction: ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.x2.y2.1 |  | YES | ignore |
| Transaction ID | M |  | 9.x2.y2.23 |  | YES | reject |
| ng-eNB-DU Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

9.x2.y2	Radio Network Layer Related IEs

9.x2.y2.1	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Code | M |  | INTEGER (0..255) |  |
| >Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, ...) |  |

9.x2.y2.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the W1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, RL failure-RLC, Unknown or already allocated ng-eNB-CU UE W1AP ID, Unknown or already allocated ng-eNB-DU UE W1AP ID, Unknown or inconsistent pair of UE W1AP ID, Interaction with other procedure, Not supported QCI Value, Action Desirable for Radio Reasons, No Radio Resources Available, Procedure cancelled, Normal Release, ..., Cell not available, RL failure-others, UE rejection, Resources not available for the slice, AMF initiated abnormal release, Release due to Pre-Emption, PLMN not served by the ng-eNB-CU) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, ...) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| RL Failure-RLC | The action is due to an RL failure caused by exceeding the maximum number of ARQ retransmissions. |
| Unknown or already allocated ng-eNB-CU UE W1AP ID | The action failed because the ng-eNB-CU UE W1AP ID is either unknown, or (for a first message received at the ng-eNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated ng-eNB-DU UE W1AP ID | The action failed because the ng-eNB-DU UE W1AP ID is either unknown, or (for a first message received at the ng-eNB-DU) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE W1AP ID | The action failed because both UE W1AP IDs are unknown, or are known but do not define a single UE context. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| Not supported QCI Value | The action failed because the requested QCI is not supported. |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. |
| Normal Release | The action is due to a normal release of the UE (e.g. because of mobility) and does not indicate an error. |
| Cell Not Available | The action failed due to no cell available in the requested node. |
| RL Failure-others | The action is due to an RL failure caused by other radio link failures than exceeding the maximum number of ARQ retransmissions. |
| UE rejection | The action is due to ng-eNB-CU’s rejection of a UE access request. |
| Resources not available for the slice | The requested resources are not available for the slice. |
| AMF initiated abnormal release | The release is triggered by an error in the AMF or in the NAS layer. |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |
| PLMN not served by the ng-eNB-CU | The PLMN indicated by the UE is not served by the ng-eNB-CU. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |
| Transport Resource Unavailable | The required transport resources are not available. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | Control processing overload. |
| Not Enough User Plane Processing Resources Available | No enough resources are available related to user plane processing. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |
| Unspecified Failure | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

9.x2.y2.4	ng-eNB-CU UE W1AP ID

The ng-eNB-CU UE W1AP ID uniquely identifies the UE association over the W1 interface within the ng-eNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ng-eNB-CU UE W1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

9.x2.y2.5	ng-eNB-DU UE W1AP ID

The ng-eNB-DU UE W1AP ID uniquely identifies the UE association over the W1 interface within the ng-eNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ng-eNB-DU UE W1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

9.x2.y2.9	ng-eNB-DU ID

The ng-eNB-DU ID uniquely identifies the ng-eNB-DU at least within an ng-eNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ng-eNB-DU ID | M |  | INTEGER (0 .. 236-1) | The ng-eNB-DU ID is independently configured from cell identifiers, i.e. no connection between ng-eNB-DU ID and cell identifiers. |

9.x2.y2.10	Served Cell Information

This IE contains cell configuration information of a cell in the ng-eNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRAN CGI | M |  | 9.x2.y2.12 |  | - |  |
| E-UTRAN PCI | M |  | INTEGER (0..503) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.x2.y2.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.x2.y2.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | - |  |
| >PLMN Identity | M |  | 9.x2.y2.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.x2.y2.37 | Supported S-NSSAIs per TA. | YES | ignore |
| CHOICE NR-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.x2.y2.17 |  | - |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.x2.y2.17 |  | - |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.x2.y2.15 |  | - |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.x2.y2.15 |  | - |  |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>> NR FreqInfo | M |  | NR Frequency Info 9.x2.y2.17 |  | - |  |
| >>> Transmission Bandwidth | M |  | Transmission Bandwidth 9.x2.y2.15 |  | - |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message defined in TS 36.331 [x1]. | - |  |
| RANAC | O |  | RAN Area Code 9.x2.y2.57 |  | YES | ignore |
| Extended Served PLMNs List |  | 0..1 |  | This is included if more than 6 Served PLMNs is to be signalled. | YES | ignore |
| >Extended Served PLMNs Item |  | 1 ..<maxnoofExtendedBPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.x2.y2.14 |  | - |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.x2.y2.37 | Supported S-NSSAIs per TA. | - |  |
| Cell Type | O |  | 9.x2.y2.87 |  | YES | ignore |
| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR-1> |  | This IE corresponds to the PLMN-IdentityInfoList IE in SIB1 as specified in TS 36.331 [x1]. The PLMN Identities and associated information contained in this IE shall be provided in the same order as broadcast in SIB1. | YES | ignore |
| >PLMN Identity List | M |  | Available PLMN List 9.x2.y2.65 |  | - |  |
| >Extended PLMN Identity List | O |  | Extended Available PLMN List 9.x2.y2.76 |  | - |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | - |  |
| >E-UTRAN Cell Identity | M |  | BIT STRING (28) |  | - |  |
| >RANAC | O |  | RAN Area Code 9.x2.y2.57 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |

9.x2.y2.12	E-UTRAN CGI

This information element is used to globally identify a cell (see TS 36.401 [x2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.x2.y2.7 |  |
| Cell Identity | M |  | BIT STRING (SIZE(28)) | The leftmost bits of the Cell Identity correspond to the eNB ID. |

9.x2.y2.14	PLMN Identity

This information element indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE(3)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n -The PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |

9.x2.y2.15	Transmission Bandwidth

The Transmission Bandwidth IE is used to indicate the UL or DL transmission bandwidth.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NRB | M |  | ENUMERATED (mbw6, mbw15, mbw25, mbw50, mbw75, mbw100, ...) | This IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 36.104 [x2]). The values mbw6, mbw15, etc. correspond to the number of resource blocks “NRB” 6, 15, etc. |

9.x2.y2.16	Void

Reserved for future use.

9.x2.y2.17	E-UTRAN Frequency Info

The E-UTRAN Frequency Info defines the carrier frequency used in a cell for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-UTRAN ARFCN | M |  | INTEGER (0.. maxE-UTRANARFCN) | RF Reference Frequency as defined in TS 36.104 [x2]. |
| Frequency Band List |  | 1 |  |  |
| >Frequency Band Item |  | 1..<maxnoof E-UTRAN CellBands> |  |  |
| >>E-UTRAN Frequency Band | M |  | INTEGER (1.. maxBandsEUTRA) | Operating Band as defined in TS 36.104 [x2]. |

| Range bound | Explanation |
| --- | --- |
| maxE-UTRANARFCN | Maximum value of E-UTRAN ARFCNs. Value is 262143. |
| maxnoofE-UTRANCellBands | Maximum no. of frequency bands supported for an E-UTRAN cell. Value is 8. |
| maxBandsEUTRA | Maximum value of E-UTRAN Bands. Value is 256. |

9.x2.y2.18	ng-eNB-DU System Information

This IE contains the system information generated by the ng-eNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MIB message | M |  | OCTET STRING | MIB message, as defined in TS 36.331 [x1]. |
| SIB1 message | M |  | OCTET STRING | SIB1 message, as defined in TS 36.331 [x1]. |
| SIB2 message | M |  | OCTET STRING | SIB1 message, as defined in TS 36.331 [x1]. |
| SIB3 message | M |  | OCTET STRING | SIB1 message, as defined in TS 36.331 [x1]. |
| SIB8 message | M |  | OCTET STRING | SIB1 message, as defined in TS 36.331 [x1]. |
| SIB16 message | M |  | OCTET STRING | SIB1 message, as defined in TS 36.331 [x1]. |

9.x2.y2.23	Transaction ID

The Transaction ID IE uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer. Messages belonging to the same procedure shall use the same Transaction ID. The Transaction ID is determined by the initiating peer of a procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transaction ID | M |  | INTEGER (0..255, ...) |  |

9.x2.y2.29	5GS TAC

This information element is used to identify Tracking Area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5GS TAC | M |  | OCTET STRING (SIZE (3)) |  |

9.x2.y2.29a	Configured EPS TAC

This information element is used to identify a configured EPS Tracking Area Code in order to enable application of Roaming and Access Restrictions for DC as specified in TS 37.340 [7]. This IE is configured for the cell, but not broadcast.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Configured EPS TAC | M |  | OCTET STRING (SIZE (2)) |  |

9.x2.y2.37	Slice Support List

This IE indicates the list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Support Item IEs |  | 1..<maxnoofSliceItems> |  |  |
| >S-NSSAI | M |  | 9.3.1.38 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

9.x2.y2.38	S-NSSAI

This IE indicates the S-NSSAI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SST | M |  | OCTET STRING (SIZE(1)) |  |
| SD | O |  | OCTET STRING (SIZE(3)) |  |

9.x2.y2.42	ng-eNB-CU System Information

This IE contains the system information encoded by the ng-eNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB type to Be Updated List |  | 1 |  |  |
| >SIB type to Be Updated Item IEs |  | 1... <maxnoofSIBTypes> |  |  |
| >>SIB type | M |  | INTEGER (4,... 7, 9…15,,..32, ...) | Indicates a certain SIB block. |
| >>SIB message | M |  | OCTET STRING | SIB message containing SIB as defined in TS 36.331 [x1]. |
| >>Value Tag | M |  | INTEGER (0..31, ...) |  |

9.x2.y2.57	RAN Area Code

This information element is used to uniquely identify a RAN Area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RANAC | M |  | INTEGER (0..255) | RAN Area Code |

9.x2.y2.65	Available PLMN List

This IE indicates the list of available PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available PLMN Item IEs |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.x2.y2.14 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |

9.x2.y2.68	Service Status

This IE is used to indicate the service status of a cell by the ng-eNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Service State | M |  | ENUMERATED (In-Service, Out-Of-Service, ...) | Indicates the Service State of the cell. In-Service and Out-of-Service Service States are defined in TS 38.401 [x3]. |
| Switching Off Ongoing | O |  | ENUMERATED (True, ...) | This IE indicates that the ng-eNB-DU will delete the cell after some time using a new ng-eNB-DU Configuration Update procedure. When this IE is set to “True” the Service State IE shall be set to “In-Service”. |

9.x2.y2.76	Extended Available PLMN List

This IE indicates the list of available PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Available PLMN Item IEs |  | 1..< maxnoofExtendedBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.x2.y2.14 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |

9.x2.y2.87	Cell Type

This IE provides the cell coverage area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Size | M |  | ENUMERATED (verysmall, small, medium, large, …) |  |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

10	Handling of unknown, unforeseen and erroneous protocol data

Clause 10 of TS 38.413 [x7] is applicable for the purposes of the present document, with the following additions for non-UE-associated procedures:

-	In case of Abstract Syntax Error, when reporting the Criticality Diagnostics IE for not comprehended IE/IEgroups or missing IE/IE groups, the Transaction ID IE shall also be included;

-	In case of Logical Error, when reporting the Criticality Diagnostics IE, the Transaction ID IE shall also be included;

- 	In case of Logical Error in a response message of a Class 1 procedure, or failure to comprehend Transaction ID IE from a received message, the procedure shall be considered as unsuccessfully terminated or not terminated (e.g., transaction ID unknown in response message), and local error handling shall be initiated.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>