# 3GPP TSG-RAN WG3 #105	Tdoc R3-194598

Ljubljana, Slovenia, 26th – 30th August

Agenda Item:	9.3

Source:	Ericsson

Title:	Summary of discussions on use of SCTP ports for multiple TNLA

Document for:	Discussion, Decision

# Introduction

A discussion on the use of SCTP ports when establishing one or more TNLAs has been taken during RAN3-105. This document summarises the main inputs for the discussion and points out the aspects on which there appears to be consensus.

# Discussion

During discussions on the use of SCTP ports for establishment of multiple SCTP associations, two main inputs were received in [1] and [2].

The main difference between the two proposals seems to be the following:

- In [1] it is discussed that current specifications foresee the use of the IANA port number, to be used at the time of creating the very first SCTP association between SCTP server and SCTP client, to be used at least for the SCTP server endpoint

- In [2] it is proposed that, at establishment of the very first SCTP association between SCTP server and SCTP client, the SCTP client uses a preconfigured port number (i.e. not necessarily the IANA port) for the SCTP server endpoint

However, the two proposals seem to converge on the following aspect, where the NG interface is taken as example:

When the AMF request to dynamically add additional SCTP association between the NG-RAN node/AMF pair, the SCTP Destination Port number value may be 38412, or any dynamic port value (IETF RFC 6335 [x]).

It is therefore proposed to capture the above principle in TS38.422, TS38.462 and TS38.472 in order to clarify non controversial aspects on the topic.

Proposal: it is proposed to agree to CRs to TS38.412, TS38.422, TS38.462 and TS38.47 provided in R3-194680, R3-194681, R3-194682, R3-194683, which capture the following principle (NG case taken as example):

When the AMF request to dynamically add additional SCTP association between the NG-RAN node/AMF pair, the SCTP Destination Port number value may be 38412, or any dynamic port value (IETF RFC 6335 [x]).

# References

[1] R3-194146, Use of SCTP ports for multiple TNLA (Ericsson)

[2] R3-193844, Support for Dynamic Port (Nokia, Nokia Shanghai Bell)