3GPP TSG-RAN3 Meeting #105	R3-194789

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.413 | CR | 0003 | rev | 2 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for introducing WWC in RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Telecom Italia, BT, Broadcom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC-NG_interface-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the work time phase SA2 has specified in the Release 16 of TS 23. 316, TS 23. 501, TS 23. 502 and TS 23. 503, to support the Trusted Access Network in addition of the current R15 Untrusted Access network, and support connectivity to 5GC using wireline broadband access. There is a need to support trusted non-3GPP access and wireline access via the N2 reference point as well. Since the non-3GPP access only includes the untrusted non-3GPP access network in the current specification of 29.413, the description about trusted non-3GPP and wireline access should be included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FFS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NGAP would stay incomplete to support signalling of trusted non-3GPP access and wireline access via the N2 reference point. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1; 2; 3; 4.1; 5.1; 5.2; 5.3; 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.413 CR 0192 |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | RV1: Add cosigned companies on top of [R3-193850], and updated online. Rev2: provide version with tracking change |  |  |  |  |  |  |  |  |

Change Begins

# 1	Scope

The present document describes the applicability of NG Application Protocol (NGAP) messages and procedures, defined in 3GPP TS 38.413 [2], to non-3GPP access. A general description for non-3GPP access can be found in 3GPP TS 23.501 [3], 3GPP TS 23.502 [4], and 3GPP TS 23.316 [x].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 23.502: "Procedures for the 5G System".

[x]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

# 3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G-RG	5G Residential Gateway

FN-RG	Fixed Network Residential Gateway

N3IWF	Non-3GPP InterWorking Function

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWIF	Trusted WLAN Interworking Function

W-AGF	Wireline Access Gateway Function

# 4	Principles for the use of NGAP for non-3GPP access

## 4.1	General

TS 23.501 [3] specifies the NGAP used between the Non-3GPP access network node and the AMF. The Non-3GPP access network node is either an Non-3GPP InterWorking Function (N3IWF), or a Trusted Non-3GPP Gateway Function (TNGF), or a Trusted WLAN Interworking Function (TWIF), or a Wireline Access Gateway Function (W-AGF). NGAP is used as specified in TS 38.413 [2] with clarifications or additions as specified in Clause 5.

# 5	Non-3GPP access

## 5.1	Use of the NGAP for non-3GPP access

The following NGAP procedures are used between the Non-3GPP access network node and the AMF:

-	PDU Session Management Procedures

-	PDU Session Resource Setup

-	PDU Session Resource Release

-	PDU Session Resource Modify

-	PDU Session Resource Notify

-	UE Context Management Procedures

-	Initial Context Setup

-	UE Context Release Request

-	UE Context Release

-	UE Context Modification

-	Transport of NAS Messages Procedures

-	Initial UE Message

-	Downlink NAS Transport

-	Uplink NAS Transport

-	NAS Non Delivery Indication

-	Reroute NAS Request

-	Interface Management Procedures

-	NG Setup

-	RAN Configuration Update

-	AMF Configuration Update

-	NG Reset

-	Error Indication

-	AMF Status Indication

-	Overload Start

-	Overload Stop

-	UE TNLA Binding Procedures

-	UE TNLA Binding Release

For the NGAP procedures used between the Non-3GPP access network node and the AMF, the Non-3GPP access network node fulfils the behaviour of the NG-RAN node as specified in clause 8 of TS 38.413 [2], with clarifications as specified in Clause 5.3. The text in clause 8 of TS 38.413 [2] referring to Uu should be understood as referring to the Y2 reference point as specified in TS 23.501 [3].

## 5.2	NGAP messages used for non-3GPP access

The list given below shows the NGAP messages, as specified in TS 38.413 [2] subclause 9.2 (tabular format) and 9.4

(ASN.1 notation) that are used between the Non-3GPP access network node and the AMF.

-	PDU SESSION RESOURCE SETUP REQUEST

-	PDU SESSION RESOURCE SETUP RESPONSE

-	PDU SESSION RESOURCE RELEASE COMMAND

-	PDU SESSION RESOURCE RELEASE RESPONSE

-	PDU SESSION RESOURCE MODIFY REQUEST

-	PDU SESSION RESOURCE MODIFY RESPONSE

-	PDU SESSION RESOURCE NOTIFY

-	INITIAL CONTEXT SETUP REQUEST

-	INITIAL CONTEXT SETUP RESPONSE

-	INITIAL CONTEXT SETUP FAILURE

-	UE CONTEXT RELEASE REQUEST

-	UE CONTEXT RELEASE COMMAND

-	UE CONTEXT RELEASE COMPLETE

-	UE CONTEXT MODIFICATION REQUEST

-	UE CONTEXT MODIFICATION RESPONSE

-	UE CONTEXT MODIFICATION FAILURE

-	INITIAL UE MESSAGE

-	DOWNLINK NAS TRANSPORT

-	UPLINK NAS TRANSPORT

-	NAS NON DELIVERY INDICATION

-	REROUTE NAS REQUEST

-	NG SETUP REQUEST

-	NG SETUP RESPONSE

-	NG SETUP FAILURE

-	RAN CONFIGURATION UPDATE

-	RAN CONFIGURATION UPDATE ACKNOWLEDGE

-	RAN CONFIGURATION UPDATE FAILURE

-	AMF CONFIGURATION UPDATE

-	AMF CONFIGURATION UPDATE ACKNOWLEDGE

-	AMF CONFIGURATION UPDATE FAILURE

-	NG RESET

-	NG RESET ACKNOWLEDGE

-	ERROR INDICATION

-	AMF STATUS INDICATION

-	OVERLOAD START

-	OVERLOAD STOP

-	UE TNLA BINDING RELEASE REQUEST

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

For the NGAP messages transferred between the Non-3GPP access network node and the AMF, the following exceptions to the specification in TS 38.413 [2] shall be applied:

PDU SESSION RESOURCE SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE;

-	PDU Session Aggregate Maximum Bit Rate IE: the information contained within this IE corresponds to the PDU Session Total Maximum Bit Rate, as specified in TS 23.316 [x]. A W-AGF receving the request to setup resources for a PDU Session without this IE being indicated shall fail the setup of resources for this PDU Session.

PDU SESSION RESOURCE RELEASE COMMAND message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE.

PDU SESSION RESOURCE MODIFY REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE;

-	PDU Session Aggregate Maximum Bit Rate IE: the information contained within this IE corresponds to the PDU Session Total Maximum Bit Rate, as specified in TS 23.316 [x]. A W-AGF receving the request to setup resources for a PDU Session without this IE being indicated shall fail the setup of resources for this PDU Session.

INITIAL CONTEXT SETUP REQUEST	message:

-	the following IEs shall be ignored, when received:

-	Core Network Assistance Information IE

-	Trace Activation IE

-	MobilityRestriction List IE

-	UE Radio Capability IE

-	Index to RAT/Frequency Selection Priority IE

-	Emergency Fallback Indicator IE

-	RRC Inactive Transition Report Request IE

-	UE Radio Capability for Paging IE

-	RG Level Wireline Access Characteristics IE: the information given within this IE between the W-AGF and the AMF shall be stored in the UE context by the W-AGF as specified in TS 23.316 [x].

Editor’s Note:	Details are FFS.

-	Security Key IE: the information given within this IE between the TNGF and the AMF shall include the TNGF key as specified in TS 23.316 [x].

UE CONTEXT RELEASE COMPLETE message:

-	the following IEs shall be ignored, when received:

-	Information on Recommended Cells and RAN Nodes for Paging IE

UE CONTEXT MODIFICATION REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	Index to RAT/Frequency Selection Priority IE

-	Core Network Assistance Information IE

-	Emergency Fallback Indicator IE

-	RRC Inactive Transition Report Request IE

-	if this is the first message received from a new AMF, the N3IWF shall identify the old AMF and the UE using the received RAN UE NGAP ID, release the UE-associated logical NG-connection to the old AMF and create a new UE-associated logical NG-connection to the new AMF

UE CONTEXT MODIFICATION RESPONSE message:

-	the following IEs shall be ignored, when received:

-	RRC State IE

INITIAL UE MESSAGE message:

-	RRC Establishment Cause IE: the information given within this IE is to indicate the establishment cause as specified in TS 23.502 [4].

#### Editor’s Note: 	Which reference point is actually impacted (N1, N2) by FN-RG authentication, and in which way (existing or new IE, message) necessary is FFS).

DOWNLINK NAS TRANSPORT message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	MobilityRestriction List IE

-	Index to RAT/Frequency Selection Priority IE

UPLINK NAS TRANSPORT message:

#### Editor’s Note: 	A List of W-AGF may be included by the W-AGF for the SMF to select the UPF. This may not be visible explicitly on NGAP and explicit visibility of respective info in this message  FFS.

NG SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

RAN CONFIGURATION UPDATE message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

The Global RAN Node ID IE in the applicable NGAP messages between the N3IWF and the AMF includes the Global N3IWF ID as specified in TS 38.413 [2].

Editor’s Note:	It is FFS whether a TNGF ID, a W-AGF, a TWIF ID needs to be explicitly defined or can be mapped e.g. on other existing (R)AN node IDs.

The User Location Information IE in the applicable NGAP messages between the N3IWF and the AMF includes the IP address and port number as specified in TS 38.413 [2].

Editor’s Note:	It is FFS whether a User Location Information relevant for a TNGF, aW-AGF, a TWIF ID needs to be explicitly defined or can be mapped e.g. on other existing NGAP IEs, whether it needs to be defined in NGAP and whether a single IE wouldn’t be sufficient.

The RAN UE NGAP ID IE in the applicable NGAP messages identifies the UE association over the NG interface within the N3IWF node, or the TNGF node, or the TWIF node, or the W-AGF node, as specified in TS 38.413 [2].

The PDU Session Resource Setup Request Transfer IE in the applicable NGAP messages between the W-AGF and the AMF includes the PDU Session Aggregated Maximum Bit Rate as specified in TS 38.413 [2] which is interpreted as the PDU Session Total Maximum Bit Rate as specified in TS 23.316 [x].

The PDU Session Resource Modify Request Transfer IE in the applicable NGAP messages between the W-AGF and the AMF includes the PDU Session Aggregated Maximum Bit Rate as specified in TS 38.413 [2]which is interpreted as the PDU Session Total Maximum Bit Rate as specified in TS 23.316 [x].

## 5.4	Handling of NGAP messages not specified to be applicable between the Non-3GPP access network node and AMF

If the Non-3GPP access network node or the AMF receive an NGAP message not listed in section 5.2 as being applicable between the Non-3GPP access network node and AMF, the receiving node shall act according to the criticality defined for the elementary procedure and ignore the message or discard the message and send an ERROR INDICATION message indicating that the procedure is not supported, as specified in in TS 38.413 [2].

End Change