3GPP TSG-RAN WG3 Meeting #106 	R3-196471

# Reno, USA, November 18-22 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0150 | rev | 4 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IPsec address exchange E1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  |  | Date: |  | 2019-11-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification does not support an explicit mechanism to exchange IPSec addresses for U-Plane during setup and configuration update procedures. However, for IPsec, given that both gNB-CU-CP and gNB-CU-UP may be virtualized, IPsec addresses terminated directly at the node (e.g., when SEG is not used) may need to be updated as they may change over time due to up/down-scaling of resources. Further, for U-Plane case, exchanging IPsec addresses at interface setup and configuration update procedures allows “apriori” IPsec tunnel establishment. This is useful in order to avoid the incurred delayed when establishing the IPsec tunnel. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce means to convey IPsec addresses during interface setup and configuration update procedures. Impact assessment towards the previous version of the specification (same release): This CR has an impact under functional and protocol point of view The impact can be considered isolated because the change is limited to indication of TNL IP addresses. This CR is ASN.1 backwards compatible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IPSec tunnel cannot be set up beforehand, which incurs additional end user delays. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3, 8.2.4, 8.2.5, 8.2.6, 9.2.1, 9.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev.1: structure modified to provide only IPsec addreses for user plane. Rev.2: procedural text corrected Rev.3: procedural text updated, added ASN.1 Rev.4: resubmitted to RAN3#106 |
| --- | --- |

Beginning of Text Proposal to TS 38.463

### 8.2.3	gNB-CU-UP E1 Setup

#### 8.2.3.1	General

The purpose of the gNB-CU-UP E1 Setup procedure is to exchange application level data needed for the gNB-CU-UP and the gNB-CU-CP to correctly interoperate on the E1 interface. If the gNB-CU-UP initiates the first TNL association, it shall also initiate the gNB-CU-UP E1 Setup procedure. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: gNB-CU-UP E1 Setup procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-CP. The gNB-CU-CP responds with a GNB-CU-UP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-UP E1 SETUP REQUEST message contains the gNB-CU-UP Name IE the gNB-CU-CP may use this IE as a human readable name of the gNB-CU-UP.

If the Slice Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the NR CGI Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the QoS Parameters Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall take this IE into account.

If the GNB-CU-UP E1 SETUP REQUEST message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-UP E1 SETUP RESPONSE message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: gNB-CU-UP E1 Setup procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the setup, it shall respond with a GNB-CU-UP E1 SETUP FAILURE and appropriate cause value.

If the GNB-CU-UP E1 SETUP FAILURE message includes the Time To Wait IE, the gNB-CU-UP shall wait at least for the indicated time before reinitiating the E1 setup towards the same gNB-CU-CP.

#### 8.2.3.4	Abnormal Conditions

If the first message received for a specific TNL association is not a GNB-CU-CP E1 SETUP REQUEST, GNB-CU-UP E1 SETUP RESPONSE, or GNB-CU-UP E1 SETUP FAILURE message then this shall be treated as a logical error.

If the gNB-CU-UP does not receive either GNB-CU-UP E1 SETUP RESPONSE message or GNB-CU-UP E1 SETUP FAILURE message, the gNB-CU-UP may reinitiate the gNB-CU-UP E1 Setup procedure towards the same gNB-CU-CP, provided that the content of the new GNB-CU-UP E1 SETUP REQUEST message is identical to the content of the previously unacknowledged GNB-CU-UP E1 SETUP REQUEST message.

If the gNB-CU-UP receives a GNB-CU-CP E1 SETUP REQUEST message from the peer entity on the same E1 interface:

-	In case the gNB-CU-UP answers with a GNB-CU-CP E1 SETUP RESPONSE message and receives a subsequent GNB-CU-UP E1 SETUP FAILURE message, the gNB-CU-UP shall consider the E1 interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.2.3.3.

-	In case the gNB-CU-UP answers with a GNB-CU-CP E1 SETUP FAILURE message and receives a subsequent GNB-CU-UP E1 SETUP RESPONSE message, the gNB-CU-UP shall ignore the GNB-CU-UP E1 SETUP RESPONSE message and consider the E1 interface as non operational.

### 8.2.4	gNB-CU-CP E1 Setup

#### 8.2.4.1	General

The purpose of the gNB-CU-CP E1 Setup procedure is to exchange application level data needed for the gNB-CU-CP and the gNB-CU-UP to correctly interoperate on the E1 interface. If the gNB-CU-CP initiates the first TNL association, it shall also initiate the gNB-CU-CP E1 Setup procedure.The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-CU-CP E1 Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending a GNB-CU-CP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-UP. The gNB-CU-UP responds with a GNB-CU-CP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-CP E1 SETUP REQUEST message contains the gNB-CU-CP Name IE the gNB-CU-UP may use this IE as a human readable name of the gNB-CU-CP.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall take this IE into account.

If the GNB-CU-CP E1 SETUP REQUEST message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-CP E1 SETUP RESPONSE message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-CU-CP E1 Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot accept the setup, it shall respond with a GNB-CU-CP E1 SETUP FAILURE and appropriate cause value.

If the GNB-CU-CP E1 SETUP FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the E1 setup towards the same gNB-CU-UP.

#### 8.2.4.4	Abnormal Conditions

If the first message received for a specific TNL association is not a GNB-CU-UP E1 SETUP REQUEST, GNB-CU-CP E1 SETUP RESPONSE, or GNB-CU-CP E1 SETUP FAILURE message then this shall be treated as a logical error.

If the gNB-CU-CP does not receive either GNB-CU-CP E1 SETUP RESPONSE message or GNB-CU-CP E1 SETUP FAILURE message, the gNB-CU-CP may reinitiate the gNB-CU-CP E1 Setup procedure towards the same gNB-CU-UP, provided that the content of the new GNB-CU-CP E1 SETUP REQUEST message is identical to the content of the previously unacknowledged GNB-CU-CP E1 SETUP REQUEST message.

If the gNB-CU-CP receives a GNB-CU-UP E1 SETUP REQUEST message from the peer entity on the same E1 interface:

-	In case the gNB-CU-CP answers with a GNB-CU-UP E1 SETUP RESPONSE message and receives a subsequent GNB-CU-CP E1 SETUP FAILURE message, the gNB-CU-CP shall consider the E1 interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.2.4.3.

-	In case the gNB-CU-CP answers with a GNB-CU-UP E1 SETUP FAILURE message and receives a subsequent GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall ignore the GNB-CU-CP E1 SETUP RESPONSE message and consider the E1 interface as non operational.

### 8.2.5	gNB-CU-UP Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU-UP Configuration Update procedure is to update application level configuration data needed for the gNB-CU-UP and the gNB-CU-CP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU-UP Configuration Update procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP CONFIGURATION UPDATE message to the gNB-CU-CP including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU-CP responds with GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall interpret that the corresponding configuration data is not changed and shall continue to operate with the existing related configuration data.

If the Supported PLMNs IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall overwrite the whole list of information and store the corresponding information.

-	If the Slice Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the NR CGI Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the QoS Parameters Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall take this IE into account.

If the gNB-CU-UP ID IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall associate the TNLA to the E1 interface instance using the gNB-CU-UP ID.

If the GNB-CU-UP CONFIGURATION UPDATE message includes gNB-CU-UP TNLA To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU-UP TNLA To Remove List IE, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by both received TNL endpoints will be removed by the gNB-CU-UP. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU-UP TNLA To Remove List IE in GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by the received endpoint IP address(es) will be removed by the gNB-CU-UP.

If the GNB-CU-UP CONFIGURATION UPDATE message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU-UP Configuration Update procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the update, it shall respond with a GNB-CU-UP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU-UP CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the GNB-CU-UP CONFIGURATION UPDATE message towards the same gNB-CU-CP.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

### 8.2.6	gNB-CU-CP Configuration Update

#### 8.2.6.1	General

The purpose of the gNB-CU-CP Configuration Update procedure is to update application level configuration data needed for the gNB-CU-CP and the gNB-CU-UP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: gNB-CU-CP Configuration Update procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending a GNB-CU-CP CONFIGURATION UPDATE message to the gNB-CU-UP including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU-UP responds with GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU-CP CONFIGURATION UPDATE message, the gNB-CU-UP shall interpret that the corresponding configuration data is not changed and shall continue to operate with the existing related configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If the gNB-CU-CP TNLA To Add List IE is contained in the gNB-CU-CP CONFIGURATION UPDATE message, the gNB-CU-UP shall, if supported, use it to establish the TNL association(s) with the gNB-CU-CP. The gNB-CU-UP shall report to the gNB-CU-CP, in the gNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU-CP as follows:

-	A list of TNL address(es) with which the gNB-CU-UP successfully established the TNL association shall be included in the gNB-CU-CP TNLA Setup List IE;

-	A list of TNL address(es) with which the gNB-CU-UP failed to establish the TNL association shall be included in the gNB-CU-CP TNLA Failed To Setup List IE.

If the GNB-CU-CP CONFIGURATION UPDATE message includes gNB-CU-CP TNLA To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU-CP TNLA To Remove List IE, the gNB-CU-UP shall, if supported, initiate removal of the TNL association(s) indicated by both received TNL endpoints towards the gNB-CU-CP. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU-CP TNLA To Remove List IE, the gNB-CU-UP shall, if supported, initiate removal of the TNL association(s) indicated by the received endpoint IP address(es).

If the gNB-CU-CP TNLA To Update List IE is contained in the gNB-CU-CP CONFIGURATION UPDATE message the gNB-CU-UP shall, if supported, overwrite the previously stored information for the related TNL association.

If the TNLA Usage IE is included in the gNB-CU-CP TNLA To Add List IE or the gNB-CU-CP TNLA To Update List IE in the gNB-CU-CP CONFIGURATION UPDATE message, the gNB-CU-UP shall, if supported, use it as described in TS 38.462 [18].

If the GNB-CU-CP CONFIGURATION UPDATE message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

#### 8.2.6.3	Unsuccessful Operation

Figure 8.2.6.3-1: gNB-CU-CP Configuration Update procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot accept the update, it shall respond with a GNB-CU-CP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU-CP CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the GNB-CU-CP CONFIGURATION UPDATE message towards the same gNB-CU-UP.

#### 8.2.6.4	Abnormal Conditions

Not applicable.

<< unchanged portions omitted >>

#### 9.2.1.4	GNB-CU-UP E1 SETUP REQUEST

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | Ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |

#### 9.2.1.5	GNB-CU-UP E1 SETUP RESPONSE

This message is sent by the gNB-CU-CP to transfer information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP. | YES | Ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.x |  | YES | ignore |

<< unchanged portions omitted >>

#### 9.2.1.7	GNB-CU-CP E1 SETUP REQUEST

This message is sent by the gNB-CU-CP to transfer information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP. | YES | Ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.x |  | YES | ignore |

#### 9.2.1.8	GNB-CU-CP E1 SETUP RESPONSE

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | Ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |

<< unchanged portions omitted >>

#### 9.2.1.10	GNB-CU-UP CONFIGURATION UPDATE

This message is sent by the gNB-CU-UP to transfer updated information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | O |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| Supported PLMNs |  | 0..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| gNB-CU-UP TNLA To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-CU-UP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-CP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| Transport Network Layer Address Info | O |  | 9.3.2.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-UP and the gNB-CU-CP. Value is 32. |

<< unchanged portions omitted >>

#### 9.2.1.13	GNB-CU-CP CONFIGURATION UPDATE

This message is sent by the gNB-CU-CP to transfer updated information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP | YES | ignore |
| gNB-CU-CP TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Usage | M |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNLA is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.462 [18]. | - | - |
| gNB-CU-CP TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-UP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | YES | reject |
| gNB-CU-CP TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Usage | O |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNLA is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.462 [18]. | - | - |
| Transport Network Layer Address Info | O |  | 9.3.2.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-CP and the gNB-CU-UP. Value is 32. |

<< unchanged portions omitted >>

#### 9.3.2.x	Transport Network Layer Address Info

This IE is used for signalling TNL address information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Addresses Info to Add List | O |  |  |  |
| >Transport Layer Addresses Info to Add Item |  | 1..<maxnoofTLAs> |  |  |
| >>IPsec Transport Layer Address | M |  | Transport Layer Address 9.3.2.4 | Transport Network Layer address for IPsec endpoint. |
| Transport Layer Addresses Info to Remove List | O |  |  |  |
| >Transport Layer Addresses Info to Remove List |  | 1..<maxnoofTLAs> |  |  |
| >>IPsec Transport Layer Address | M |  | Transport Layer Address 9.3.2.4 | Transport Network Layer address for IPsec endpoint. |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAs | Maximum no. of Transport Layer Addresses in the message. Value is 32. |

<< unchanged portions omitted >>

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

GNB-CU-UP-TNLA-To-Remove-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RANUEID,

GNB-DU-ID,

Transport-Layer-Addresses-Info

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-GNB-CU-UP-TNLA-To-Remove-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RANUEID,

id-GNB-DU-ID,

id-Transport-Layer-Addresses-Info,

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset,

maxnoofTLAs

<<<<unchanged texts omitted>>

-- **************************************************************

--

#### -- GNB-CU-UP E1 SETUP

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Request

--

-- **************************************************************

GNB-CU-UP-E1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupRequestIEs} },

...

}

GNB-CU-UP-E1SetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-CNSupport							CRITICALITY reject	TYPE CNSupport						PRESENCE mandatory	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-Transport-Layer-Addresses-Info	CRITICALITY ignore	TYPE Transport-Layer-Addresses-Info	PRESENCE optional	},

...

}

SupportedPLMNs-List	::=	SEQUENCE (SIZE (1..maxnoofSPLMNs)) OF SupportedPLMNs-Item

SupportedPLMNs-Item ::= SEQUENCE {

pLMN-Identity					PLMN-Identity,

slice-Support-List				Slice-Support-List												OPTIONAL,

nR-CGI-Support-List				NR-CGI-Support-List												OPTIONAL,

qoS-Parameters-Support-List		QoS-Parameters-Support-List										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SupportedPLMNs-ExtIEs } }		OPTIONAL,

...

}

SupportedPLMNs-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Response

--

-- **************************************************************

GNB-CU-UP-E1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupResponseIEs} },

...

}

GNB-CU-UP-E1SetupResponseIEs

E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name					PRESENCE optional	}|

{ ID id-Transport-Layer-Addresses-Info	CRITICALITY ignore	TYPE Transport-Layer-Addresses-Info	PRESENCE optional	},

...

}

<<<<unchanged texts omitted>>

-- **************************************************************

--

#### -- GNB-CU-CP E1 SETUP

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Request

--

-- **************************************************************

GNB-CU-CP-E1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupRequestIEs} },

...

}

GNB-CU-CP-E1SetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name					PRESENCE optional	}|

{ ID id-Transport-Layer-Addresses-Info	CRITICALITY ignore	TYPE Transport-Layer-Addresses-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Response

--

-- **************************************************************

GNB-CU-CP-E1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupResponseIEs} },

...

}

GNB-CU-CP-E1SetupResponseIEs

E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-CNSupport							CRITICALITY reject	TYPE CNSupport						PRESENCE mandatory	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-Transport-Layer-Addresses-Info	CRITICALITY ignore	TYPE Transport-Layer-Addresses-Info	PRESENCE optional	},

...

}

<<<<unchanged texts omitted>>

-- **************************************************************

--

#### -- GNB-CU-UP CONFIGURATION UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-UP Configuration Update

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory }|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE optional	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-GNB-CU-UP-TNLA-To-Remove-List		CRITICALITY reject	TYPE GNB-CU-UP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Addresses-Info	CRITICALITY ignore	TYPE Transport-Layer-Addresses-Info	PRESENCE optional	},

...

}

GNB-CU-UP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-UP-TNLA-To-Remove-Item

-- **************************************************************

--

-- GNB-CU-UP Configuration Update Acknowledge

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateAcknowledgeIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateAcknowledgeIEs

E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	}|

{ ID id-Transport-Layer-Addresses-Info	CRITICALITY ignore	TYPE Transport-Layer-Addresses-Info	PRESENCE optional	},

...

}

<<<<unchanged texts omitted>>

-- **************************************************************

--

#### -- GNB-CU-CP CONFIGURATION UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-CP Configuration Update

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name						PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Add-List			CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Add-List		PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Remove-List		CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Update-List		CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Update-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Addresses-Info	CRITICALITY ignore	TYPE Transport-Layer-Addresses-Info	PRESENCE optional	},

...

}

GNB-CU-CP-TNLA-To-Add-List       ::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Add-Item

GNB-CU-CP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Remove-Item

GNB-CU-CP-TNLA-To-Update-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Update-Item

-- **************************************************************

--

-- GNB-CU-CP Configuration Update Acknowledge

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateAcknowledgeIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateAcknowledgeIEs

E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-Setup-List			CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-Setup-List					PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-Failed-To-Setup-List	CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-Failed-To-Setup-List		PRESENCE optional	}|

{ ID id-Transport-Layer-Addresses-Info	CRITICALITY ignore	TYPE Transport-Layer-Addresses-Info	PRESENCE optional	},

...

}

GNB-CU-CP-TNLA-Setup-List				::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-Setup-Item

GNB-CU-CP-TNLA-Failed-To-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-Failed-To-Setup-Item

<<<<unchanged texts omitted>>

#### -- G

GNB-CU-CP-Name 				::= PrintableString(SIZE(1..150,...))

GNB-CU-CP-UE-E1AP-ID		::= INTEGER (0..4294967295)

GNB-CU-UP-Capacity				::= INTEGER (0..255)

GNB-CU-UP-CellGroupRelatedConfiguration	 ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF GNB-CU-UP-CellGroupRelatedConfiguration-Item

GNB-CU-UP-CellGroupRelatedConfiguration-Item ::= SEQUENCE {

cell-Group-ID				Cell-Group-ID,

uP-TNL-Information			UP-TNL-Information,

uL-Configuration			UL-Configuration		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GNB-CU-UP-CellGroupRelatedConfiguration-Item-ExtIEs } }	OPTIONAL

}

GNB-CU-UP-CellGroupRelatedConfiguration-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-ID				::= INTEGER (0..68719476735)

GNB-CU-UP-Name 				::= PrintableString(SIZE(1..150,...))

GNB-CU-UP-UE-E1AP-ID		::= INTEGER (0..4294967295)

GNB-CU-CP-TNLA-Setup-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-Setup-Item-ExtIEs} } OPTIONAL,

...

}

GNB-CU-CP-TNLA-Setup-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-Failed-To-Setup-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Add-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Add-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Remove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-TNLAssociationTransportLayerAddressgNBCUUP	CRITICALITY reject	EXTENSION CP-TNL-Information	PRESENCE optional},

...

}

GNB-CU-CP-TNLA-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

tNLAssociationUsage						TNLAssociationUsage 	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Update-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Update-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress			CP-TNL-Information,

tNLAssociationTransportLayerAddressgNBCUCP	CP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-UP-TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-UP-TNLA-To-Remove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink			BitRate,

maxFlowBitRateUplink			BitRate,

guaranteedFlowBitRateDownlink	BitRate,

guaranteedFlowBitRateUplink		BitRate,

maxPacketLossRateDownlink		MaxPacketLossRate		OPTIONAL,

maxPacketLossRateUplink			MaxPacketLossRate		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTunnel				::= SEQUENCE {

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

iE-Extensions						ProtocolExtensionContainer { { GTPTunnel-ExtIEs} } OPTIONAL,

...

}

GTPTunnel-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-OverloadInformation ::= ENUMERATED {overloaded, not-overloaded}

GNB-DU-ID	::= INTEGER (0..68719476735)

<<<<unchanged texts omitted>>

#### -- T

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TransportLayerAddress		::= 	BIT STRING (SIZE(1..160, ...))

TransactionID				::= INTEGER (0..255, ...)

T-Reordering	::=	ENUMERATED	{ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750, ms3000, ...}

T-ReorderingTimer ::= SEQUENCE {

t-Reordering				T-Reordering,

iE-Extensions				ProtocolExtensionContainer { { T-ReorderingTimer-ExtIEs } }	OPTIONAL,

...

}

T-ReorderingTimer-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

Transport-Layer-Address-Info ::= SEQUENCE {

Transport-Layer-Addresses-Info-To-Add-List	Transport-Layer-Addresses-Info-To-Add-List	OPTIONAL,

Transport-Layer-Addresses-Info-To-Remove-List	Transport-Layer-Addresses-Info-To-Remove-List	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Transport-Layer-Address-Info-ExtIEs} }	OPTIONAL,

...

}

Transport-Layer-Address-Info-ExtIEs	E1AP-PROTOCOL-EXTENSION	::=	{

...

}

Transport-Layer-Addresses-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-Layer-Addresses-Info-To-Add-Item

Transport-Layer-Addresses-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { { Transport-Layer-Addresses-Info-To-Add-ItemExtIEs } }    OPTIONAL,

...

}

Transport-Layer-Addresses-Info-To-Add-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-Layer-Addresses-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-Layer-Addresses-Info-To-Remove-Item

Transport-Layer-Addresses-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { { Transport-Layer-Addresses-Info-To-Remove-ItemExtIEs } }    OPTIONAL,

...

}

Transport-Layer-Addresses-Info-To-Remove-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<unchanged texts omitted>>

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired										ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS															ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID													ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance											ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication									ProtocolIE-ID ::= 80

id-Transport-Layer-Addresses-Info								ProtocolIE-ID ::= XXX

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>