3GPP TSG-RAN WG3 Meeting #106 	R3-196589

# Reno, NV, USA, 18. - 22.11.2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for MCG RLF information delivery over SRB3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 30.10.2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has been agreed in RAN2 to enable transfer of MCG failure information over SRB3 (as well as resulting RRC reconfiguration from the MN). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of the RRC Transfer procedure is enhanced to mention also that it can carry a new RRC container with the MCG failure information towards the MN, and with the RRC reconfiguration information (or RRC release) from the MN to the SN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 0269 TS 36.423 CR 1403 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change, skipped text not changed ***

## 10.10	RRC Transfer

### 10.10.1	EN-DC

The RRC Transfer procedure is used to deliver an RRC message, encapsulated in a PDCP PDU between the MN and the SN (and vice versa) so that it may be forwarded to/from the UE using split SRB. The RRC transfer procedure is also used for:

-	providing an NR measurement report or NR failure information from the UE to the SN via the MN;

-	providing MCG failure information from the UE to the MN via the SN and an RRC reconfiguration or release from the MN to the UE via the SN.

Additional details of the RRC transfer procedure are defined in TS 36.423 [9].

Split SRB:

Figure 10.10.1-1: RRC Transfer procedure for the split SRB (DL operation)

Figure 10.10.1-1 shows an example signaling flow for the DL RRC Transfer in case of the split SRB:

1.	The MN, when it decides to use the split SRBs, starts the procedure by initiating the RRC Transfer procedure. The MN encapsulates the RRC message in a PDCP-C PDU and ciphers with own keys.

NOTE:	The usage of the split SRBs shall be indicated in the Secondary Node Addition procedure or Modification procedure.

2.	The SN forwards the RRC message to the UE.

3.	The SN may send PDCP delivery acknowledgement of the RRC message forwarded in step 2.

Figure 10.10.1-2: RRC Transfer procedure for the split SRB (UL operation)

Figure 10.10.1-2 shows an example signaling flow for the UL RRC Transfer in case of the split SRB:

1.	When the UE provides response to the RRC message, it sends it to the SN.

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received PDCP-C PDU with encapsulated RRC message.

NR measurement report or NR failure information:

Figure 10.10.1-3: RRC Transfer procedure for NR measurement report or NR failure information

Figure 10.10.1-3 shows an example signaling flow for RRC Transfer in case of the forwarding of the NR measurement report or NR failure information from the UE:

1.	When the UE sends a measurement report or NR failure information, it sends it to the MN in a container called ULInformationTransferMRDC as specified in TS 36.331 [10].

2.	The MN initiates the RRC Transfer procedure, in which it transfers the received NR measurement report or NR failure information as an octet string.

MCG failure information and RRC Reconfiguration / RRC Release:

Figure 10.10.1-A: RRC Transfer procedure for MCG failure information

Figure 10.10.1-A shows an example signaling flow for RRC Transfer in case of the forwarding of the MCG failure information from the UE:

1.	When the UE sends an MCG failure information, it sends it to the SN in a container called ULInformationTransferMRDC as specified in TS 38.331 [4].

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received MCG failure information as an octet string.

3.	The MN initiates the RRC Transfer procedure, in which it transfers the RRC reconfiguration or RRC release information as an octet string.

4.	The SN sends the received information to the UE in a container called DLInformationTransferMRDC, as specified in TS 38.331 [4].

### 10.10.2	MR-DC with 5GC

The RRC Transfer procedure is used to deliver RRC message, encapsulated in a PDCP PDU between the MN and the SN (and vice versa) so that it may be forwarded to/from the UE using split SRB. The RRC transfer procedure is also used for:

-	providing a measurement report or failure information report from the UE to the SN;

-	providing MCG failure information from the UE to the MN via the SN and an RRC reconfiguration or release from the MN to the UE via the SN.

Additional details of the RRC transfer procedure are defined in TS 38.423 [5].

Split SRB:

Figure 10.10.2-1: RRC Transfer procedure for split SRB (DL operation)

Figure 10.10.2-1 shows an example signaling flow for DL RRC Transfer in case of the split SRB:

1.	The MN, when it decides to use the split SRBs, starts the procedure by initiating the RRC Transfer procedure. The MN encapsulates the RRC message in a PDCP PDU and ciphers with own keys.

NOTE:	The usage of the split SRBs shall be indicated in the Secondary Node Addition procedure or Modification procedure.

2.	The SN forwards the RRC message to the UE.

3.	The SN may send PDCP delivery acknowledgement of the RRC message forwarded in step 2.

Figure 10.10.2-2: RRC Transfer procedure for split SRB (UL operation)

Figure 10.10.2-2 shows an example signaling flow for UL RRC Transfer in case of the split SRB:

1.	When the UE provides response to the RRC message, it sends it to the SN.

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received PDCP PDU with encapsulated RRC message.

SN measurement report or failure information report:

Figure 10.10.2-3: RRC Transfer procedure for SN measurement report or failure information report

Figure 10.10.2-3 shows an example signaling flow for RRC Transfer in case of the forwarding of the SN measurement report or failure information report from the UE:

1.	When the UE sends an SN measurement report or failure information report, it sends it to the MN in a container called ULInformationTransferMRDC as specified in TS 38.331 [4].

2.	The MN initiates the RRC Transfer procedure, in which it transfers the received SN measurement report or failure information as an octet string.

MCG failure information and RRC Reconfiguration / RRC Release:

Figure 10.10.1-B: RRC Transfer procedure for MCG failure information

Figure 10.10.1-B shows an example signaling flow for RRC Transfer in case of the forwarding of the MCG failure information from the UE:

1.	When the UE sends an MCG failure information, it sends it to the SN in a container called ULInformationTransferMRDC as specified in TS 38.331 [4].

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received MCG failure information as an octet string.

3.	The MN initiates the RRC Transfer procedure, in which it transfers the RRC reconfiguration or RRC release information as an octet string.

4.	The SN sends the received information to the UE in a container called DLInformationTransferMRDC, as specified in TS 38.331 [4].

*** Remaining text not changed ***