3GPP RAN WG3 Meeting #106	R3-196597

Reno, USA, 18-22 November 2019				                 (Resubmission: R3-195612)

Agenda item:	15.4.1

Source:	Apple Inc.

Title:	On Early Data Forwarding for CHO

Document for:	Decision

# Introduction

In RAN3#104, it was agreed that “We agree to specify both “early forwarding” and “late forwarding” according to definitions in agreed St2 TPs; whether to use early data forwarding for CHO is FFS”.

In this contribution, we evaluate the feasibility and benefits of early data forwarding for CHO in FR2 networks and propose RAN3 to specify it.

# Discussion

## Background

NR is expected to work in the frequency range higher than 24 GHz known as FR2. UE beams are directional in FR2 and therefore when a UE moves or rotates it may experience rapid signal degradation. Such rapid degradation will result in high frequency of HO attempts, and associated failure rates, and more ping-pong between cells. Therefore, HO is considerably more challenging in NR at FR2 than in LTE.

In previous RAN3 meetings, some contributions addressed data forwarding specifics [1-2]. It was finally agreed to “specify both “early forwarding” and “late forwarding” according to definitions in agreed St2 TPs; whether to use early data forwarding for CHO is FFS”.

Figure 1 shows different options for data forwarding for CHO as summarized in [1].  In this paper, we refer to option 1 as depicted in Figure 1 as “early data forwarding”.

In early data forwarding for CHO, the serving cell will start forwarding DL data packet of the UE to potential target cells after sending CHO configuration to the UE. In this scenario, a target cell may receive data packets of a UE before UE performing CHO to that target cell.

We note that in RAN2#107 it was agreed that

Agreement (RAN2#107)

Do not introduce “bye” message from UE to the source cell for CHO.

It is therefore understood that option 2 as shown in Figure 1 is not a viable option as RAN2 is not defining any indication message from UE to source cell for CHO.

Figure 1: Data Forwarding for CHO [1]

In this paper, we present some simulation results evaluating performance of CHO in FR2 networks and discuss the benefits of deploying early data forwarding for CHO.

## Simulation Model

In order to study the mobility performance of NR UEs operating at FR2, it is critical to incorporate the impact of antenna radiation patterns in simulations to more accurately model the impact of UE movements on the received/transmitted signal power. In addition to antenna radiation patterns, the number of antenna panels and their locations (front and/or back) also impact mobility performance.

In RAN2#105, we proposed a simulation model to be used when studying NR mobility in FR2 [3] based on studies in RAN1 and RAN4. In this paper, we use the same simulation model from [3] for the dense urban network in defined in Table 5.2.1.2-1 of TR 38.803 [4] (see Figure 2). In this layout, a micro gNB is randomly dropped on an edge of the cluster circle. All UEs associate and communicate with a micro gNB. A hexagonal grid with ISD of 200 m, 19 macro sites, 3 sectors per site model and wrap around is assumed. All cells in the network operate at 28 GHz.

Figure 2: Network layout for dense urban from TR 38.803

A summary of relevant parameters from TR 38.803 [4] is given in Table 1:

Table 1: Summary of Simulation Model for FR2

| Scenario (in TR38.803) | Indoor hotspots | Dense Urban | Urban Macro |
| --- | --- | --- | --- |
| Network Layout Model | Table 5.2.1.3-1 | Table 5.2.1.2-1 | Table 5.2.1.1-1 |
| Propagation Model | Section 5.2.2 |  |  |
| BS Antenna Model | Table 5.2.3.2.3-1 | Table 5.2.3.2.2-1 | Table 5.2.3.2.1-1 |
| UE antenna element pattern | Table 5.2.3.3-1 |  |  |
| UE Elevation distribution | Fixed at 90 |  |  |
| Indoor ratio | 100% | 0%(baseline), 20%, 100% |  |
| Blockage Modeling | No body blockage and hand grip modelling |  |  |
| Channel bandwidth | 200MHz | 200MHz | 200MHz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| BS max TX power | 23dBm | 43dBm | 33dBm |
| UE max TX power | 23dBm | 23dBm | 23dBm |
| UE min TX power | -40dBm | -40dBm | -40dBm |
| BS Noise figure | 9dB |  |  |
| UE Noise figure | 9dB |  |  |
| Minimum SINR (for outage) | -8dB |  |  |

### Simulation Results

We simulated the time of stay (ToS) at each cell for all UEs in the network. We assumed the UEs are stationary but rotating with a frequency of 0.2 Hz to highlight the impact of directional antenna on mobility in FR2. We simulated the network for 5 seconds in which every UE make one full rotation. Figure 3 shows the CDF of ToS. It is observed that for at least 60% of handovers the ToS is less than 2 seconds. Given the relatively short ToS, it is understood that time duration between configuring CHO and execution of CHO by UE can be very short assuming that 400 ms is needed for L3 filtering and some other latencies are also associated with configuring CHO such as neighbor cell preparation.

Figure 3: CDF of Time of Stay

Observation 1: For at least 60% of the handovers the time of stay is less than 2 seconds in dense urban FR2 networks.

Observation 2: Time duration between configuring CHO on a UE and execution of CHO by the UE can be very short.

## Early Data Forwarding in CHO

Based on observation 1, it is understood that if a serving cell starts early data forwarding after configuring CHO on UE in FR2 networks, the amount of unnecessary data forwarding is minimal since there is high probability that the UE is gone to another cell within few seconds.

Observation 3: If a serving cell starts early forwarding after configuring CHO on UE in FR2 networks, the amount of unnecessary data forwarding is minimal since there is high probability that the UE is gone to another cell within few seconds.

As part of NR Mobility enhancements WID [5], it is planned to reduce HO interruption time. We understand that early data forwarding is to reduce the HO interruption time. The impact of interruption time is more important in FR2 networks where UEs may make large number of HO attempts due to impact of directional antennae.

Observation 4: Early data forwarding reduces HO interruption time which is more severe in FR2 networks where UEs may make large number of HO attempts due to impact of directional antennae.

We note that FR2 networks are mostly deployed as small cells, we thus think the number of UEs served in a cell and configured by CHO is not large. Furthermore, we expect that in many deployments the Xn interface between FR2 cells is of high capacity. We therefore understand that FR2 networks are suitable candidates for deploying early data forwarding.

Observation 5: FR2 networks are suitable candidates for deploying early data forwarding.

We note that in order to improve mobility robustness and reliability by using CHO, the network may need to configure multiple neighboring cells. It is particularly important to prepare multiple cells in FR2 where RSRP may change quite rapidly due to UE rotation. We understand that it is up to the network to use early data forwarding and it seems useful if target cells can confirm whether they are willing to receive early packet forwarding for a UE when sending HO Req Ack message back to the serving cell. Given observation above we make following proposals:

Proposal 1: RAN3 specify early data forwarding for CHO.

Proposal 2: Early data forwarding is controlled by network. RAN3 to specify a mechanism by which target cells can confirm to the serving cell if they are willing to receive early data forwarding.

# Proposals (for approval)

In this paper, we presented simulation results showing the performance of conditional handover for NR UEs using directional antenna and operating in FR2. We discussed feasibility of implementing early data forwarding for CHO. Following observations and proposals are provided:

Observation 1: For at least 60% of the handovers the time of stay is less than 2 seconds in dense urban FR2 networks.

Observation 2: Time duration between configuring CHO on a UE and execution of CHO by the UE can be very short.

Observation 3: If a serving cell starts early forwarding after configuring CHO on UE in FR2 networks, the amount of unnecessary data forwarding is minimal since there is high probability that the UE is gone to another cell within few seconds.

Observation 4: Early data forwarding reduces HO interruption time which is more severe in FR2 networks where UEs may make large number of HO attempts due to impact of directional antennae.

Observation 5: FR2 networks are suitable candidates for deploying early data forwarding.

Proposal 1: RAN3 specify early data forwarding for CHO.

Proposal 2: Early data forwarding is controlled by network. RAN3 to specify a mechanism by which target cells can confirm to the serving cell whether they are willing to receive early data forwarding for CHO.

# References

- R3-193380, “Data forwarding timing on Conditional Handover”, NTT DOCOMO, 3GPP RAN3#105, August 2019

- R3-193934, “The Issues on Data Forwarding Specifics of CHO” CATT, 3GPP RAN3#105, August 2019

- R2-1901546,	“On NR Mobility in FR2”, Apple Inc., 3GPP RAN2 #105, Feb. 2019

- TR38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects,” 3GPP, September 2017

- RP-181433, “New WID: NR mobility enhancements,” Intel, 3GPP RAN #80, May 2018.