3GPP TSG-RAN3 Meeting #106	R3-196719

Reno, Nevada, US, 18-22 November 2019

Title: 	(TP for WWC BL CR for TS 29.413) Support for interfacing wireline 5G access networks to the 5GC

Source: 	Huawei, BT

Agenda item:	21.2.2

Document for:	Other

1. Introduction

The support of wireline 5G access networks to the 5GC was discussed at RAN3#105 meeting, with the following remaining issues:

- It is FFS the details of the RG level wireline access characteristics;

- Which reference point is actually impacted (N1, N2) by FN-RG authentication, and in which way (existing or new IE, message) necessary is FFS.

- A List of W-AGF may be included by the W-AGF for the SMF to select the UPF. This may not be visible explicitly on NGAP and explicit visibility of respective info in this message FFS.

- It is FFS whether a TNGF ID, a W-AGF, a TWIF ID needs to be explicitly defined or can be mapped e.g. on other existing (R)AN node IDs.

- It is FFS whether a User Location Information relevant for a TNGF, aW-AGF, a TWIF ID needs to be explicitly defined or can be mapped e.g. on other existing NGAP IEs, whether it needs to be defined in NGAP and whether a single IE wouldn’t be sufficient.

This contribution will give a further discussion on these issues and their potential impact on TS 38.413 and TS 29.413.

2. Discussion

The wireline 5G Access Network shall be connected to the 5G Core Network via a Wireline Access Gateway Function (W-AGF). As shown in Figure 1 for 5G-RG and Figure 2 for FN-RG, the W-AGF interfaces the 5G Core Network CP and UP functions via N2 and N3 interfaces, respectively.

Figure 1: 5G-RG connecting with 5GC architecture

Figure 2: FN-RG interfaces the 5G Core Network

The detailed impacts are given as follows, which should be captured in the TS 29.413 and TS 38.413.

- Global RAN Node ID

Currently, the Global RAN Node ID only specifies the 3GPP access (e.g. gNB, ng-eNB) and untrusted non-3GPP access (e.g. N3IWF). Since the wireline 5G access network is totally different than the 3GPP access network and the untrusted non-3GPP access network, the Global RAN Node ID should include the W-AGF associated ID during NG interface setup and configuration update procedure to support the wireline 5G access network.

As discussed in [1], there are several options to introduce W-AGF ID into TS 38.413. The option A (i.e. explicitly including the W-AGF ID in TS 38.413) can be considered as the clean solution.

The W-AGF ID should be clearly included in the Global RAN Node ID IE to support the wireline 5G access.

- QoS parameters

Since W-5GAN does not provide dynamic resource reservation, the GBR QoS flows cannot be catered for independently of non-GBR flows. In such case, the aggregate bitrate needs to reflect both GBR and non-GBR flows instead. These reflect the characteristics of the wireline access provided for the 5G-RG or FN-RG.

At last SA2#135 meeting, the RG-TMBR is introduced and it was agreed that the previously added definition of session-TMBR is not needed as described in [3].

•	Each RG is associated with the following aggregate rate limit QoS parameter:

  - 	per RG Total Maximum Bit Rate (RG-TMBR)

Hence, the Session-TMBR should be removed; instead, per RG Total Maximum Bit Rate (RG-TMBR) is introduced to limit the aggregate bitrate that can be provided across all GBR and non-GBR QoS flows of a RG. The AMF provides W-AGF with Subscribed RG-TMBR in Initial UE context NGAP message (e.g. in registration and Service Request procedures) instead of Subscribed UE-AMBR. And the RG-TMBR can be updated by the AMF.

The Session-TMBR should be removed from the baseline CR, and RG-TMBR is added in UE context setup and modification procedure.

In addition, In order to support the mapping between 5G QoS parameters and wireline access specific parameters by the W-AGF, the AMF may provide the RG Level Wireline Access Characteristics (RG-LWAC) to the W-AGF at the time of the RG registration. The RG-LWAC can be specified as a transparent data container.

The RG-LWAC provided by the AMF can be encoded by BBF and CableLabs.

- User location information

During the registration via Wireline Access Network (W-5GAN), the W-AGF shall forward the registration request received from the UE to the selected AMF within an N2 initial UE message. The message shall include the user location information, where to be used by the AMF to enforce the service area restrictions.

As agreed in [2] at last SA2#115 meeting, for the 5G-BRG and FN-BRG, the Global Line Identifier is used to describe the user location; for the 5G-CRG and FN-BRG, the Global Cable Identifier is used to describe the user location.

4.7.x Global Line Identifier For the scope of usage with 5GC, a Global Line Identifier (GLI) is specified in order to define a globally unique identifier of the line connecting the RG to the network. /**Skip the unrevelant**/ The Global Line Identifier is a variable length identifier encoded as defined in TS 23.003 and in BBF WT-456. 4.7.y Global Cable Identifier For the scope of usage with 5GC, a Global Cable Identifier (GCI) is specified in order to define a globally unique identifier of the line connecting the CRG to the network. /**Skip the unrevelant**/ The Global Cable Identifier is a variable length identifier encoded as defined in TS 23.003 and by Cablelabs in [x].

As discussed in [1], there are two options to introduce the User location information into TS 38.413. it is proposed to select option A (i.e. explicitly add the details in UE location information).  The exact encoding can be left to BBF and Cablelabs.

The Global Line Identifier (for 5G-BRG and FN-BRG) and the Global Cable Identifier (for 5G-CRG and FN-CRG) should be clearly added in User Location Information IE in TS 38.413.

- FN-RG authentication

As specified in [3], the authentication of FN-RG may be done by the W-AGF, as defined by BBF and Cablelabs.

#### 7.2.1.3 FN-RG Registration via W-5GAN /**Skip the unrevelant**/ 3. W-AGF performs initial registration on behalf of the FN-RG to the 5GC. The W-AGF sends a Registration Request to the selected AMF within an N2 initial UE message (NAS Registration Request, Line-ID/HFC-node-ID based ULI, Establishment cause, UE context request, Allowed NSSAI, Authenticated Indication). /**Skip the unrevelant**/ - The Authenticated Indication indicates to AMF and 5GC that the FN-RG has been authenticated by the access network.

The W-AGF provides an indication on N2 that the FN-RG has been authenticated. The W-AGF provides the Authenticated Indication in the Initial UE Message, which is used to indicate to AMF and 5GC that the FN-RG has been authenticated by the access network. As discussed in [1], the option A (i.e. added in the NGAP message) can be selected.

The Authenticated indication of the FN-RG should be carried in initial UE message.

- W-AGF identities parameter (for W-AGF and UPF coexistence)

As specified in [2], during the 5G-RG PDU session establishment via W-5GAN, the W-AGF may include the W-AGF identities parameter in the N2 Uplink NAS transport message and send it to the AMF in the 5GC.

#### 7.3.1.1 5G-RG PDU Session establishment via W-5GAN /**Skip the unrevelant**/ The W-AGF identities parameter may be included by the W-AGF and contains a list of Identifiers (i.e. FQDN or IP address) of N3 terminations at W-AGF and can be used by SMF in step 8 in TS 23.502 [3] clause 4.3.2.2.1 as input to select an UPF. ### 7.3.4 FN-RG related PDU Session Establishment via W-5GAN The W-AGF identities contains a list of Identifiers (i.e. FQDN or IP address) of N3 terminations at W-AGF and can be used by SMF in step 8 in TS 23.502 [3] clause 4.3.2.2.1 as input to select an UPF.

The W-AGF identities parameter contains a list of identifiers (i.e. FQDN or IP address) of N3 terminations at W-AGF, and it can be used by SMF as input to select an UPF.

W-AGF identities parameter should be provided by the W-AGF to the AMF in the Uplink NAS transport message.

In summary, there is no related IE descriptions in the corresponding messages, hence the W-AGF ID, Global Line Identifier, Global Cable Identifier, RG-LWAC, W-AGF identities and Authenticated Indication should be specified in TS 38.413, while the RG-TMBR should be an exception of UE-AMBR specified in TS 29.413.

Update the baseline CRs for TS 38.413 and TS 29.413 for the above issues.

3. Conclusion

This contribution discussed RAN impact to support the wireline 5G access networks connecting to 5GC. Based on the discussion in this paper, the following proposals are proposed.

- The W-AGF ID should be clearly included in the Global RAN Node ID IE to support the wireline 5G access.

The Session-TMBR should be removed from the baseline CR, and RG-TMBR is added in UE context setup and modification procedure.

The RG-LWAC provided by the AMF can be encoded by BBF and CableLabs.

The Global Line Identifier (for 5G-BRG and FN-BRG) and the Global Cable Identifier (for 5G-CRG and FN-CRG) should be clearly added in User Location Information IE in TS 38.413.

The Authenticated indication of the FN-RG should be carried in initial UE message.

W-AGF identities parameter should be provided by the W-AGF to the AMF in the Uplink NAS transport message.

Update the baseline CRs for TS 38.413 and TS 29.413 for the above issues.

4. Reference

- R3-196716 Email information on NG interface usage for Wireless-Wireline Convergence (WWC), Huawei.

- S2-1910357, Line ID uniqueness, Huawei, Hisilicon.

- 3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

- S2-1910377, Clarification of Session-TMBR, Huawei, Hisilicon, Ericsson, Nokia, Nokia Shanghai Bell.

Annex – TP

Change Begins

# 1	Scope

The present document describes the applicability of NG Application Protocol (NGAP) messages and procedures, defined in 3GPP TS 38.413 [2], to non-3GPP access. A general description for non-3GPP access can be found in 3GPP TS 23.501 [3], 3GPP TS 23.502 [4], and 3GPP TS 23.316 [x].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 23.502: "Procedures for the 5G System".

[5]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[x]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

# 3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G-RG	5G Residential Gateway

FN-RG	Fixed Network Residential Gateway

N3IWF	Non-3GPP InterWorking Function

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWIF	Trusted WLAN Interworking Function

W-AGF	Wireline Access Gateway Function

# 4	Principles for the use of NGAP for non-3GPP access

## 4.1	General

TS 23.501 [3] specifies the NGAP used between the Non-3GPP access network node and the AMF. The Non-3GPP access network node is either an Non-3GPP InterWorking Function (N3IWF), or a Trusted Non-3GPP Gateway Function (TNGF), or a Trusted WLAN Interworking Function (TWIF), or a Wireline Access Gateway Function (W-AGF). NGAP is used as specified in TS 38.413 [2] with clarifications or additions as specified in Clause 5.

# 5	Non-3GPP access

## 5.1	Use of the NGAP for non-3GPP access

The following NGAP procedures are used between the Non-3GPP access network node and the AMF:

-	PDU Session Management Procedures

-	PDU Session Resource Setup

-	PDU Session Resource Release

-	PDU Session Resource Modify

-	PDU Session Resource Notify

-	UE Context Management Procedures

-	Initial Context Setup

-	UE Context Release Request

-	UE Context Release

-	UE Context Modification

-	Transport of NAS Messages Procedures

-	Initial UE Message

-	Downlink NAS Transport

-	Uplink NAS Transport

-	NAS Non Delivery Indication

-	Reroute NAS Request

-	Interface Management Procedures

-	NG Setup

-	RAN Configuration Update

-	AMF Configuration Update

-	NG Reset

-	Error Indication

-	AMF Status Indication

-	Overload Start

-	Overload Stop

-	UE TNLA Binding Procedures

-	UE TNLA Binding Release

For the NGAP procedures used between the Non-3GPP access network node and the AMF, the Non-3GPP access network node fulfils the behaviour of the NG-RAN node as specified in clause 8 of TS 38.413 [2], with clarifications as specified in Clause 5.3. The text in clause 8 of TS 38.413 [2] referring to Uu should be understood as referring to the Y2 reference point as specified in TS 23.501 [3].

## 5.2	NGAP messages used for non-3GPP access

The list given below shows the NGAP messages, as specified in TS 38.413 [2] subclause 9.2 (tabular format) and 9.4

(ASN.1 notation) that are used between the Non-3GPP access network node and the AMF.

-	PDU SESSION RESOURCE SETUP REQUEST

-	PDU SESSION RESOURCE SETUP RESPONSE

-	PDU SESSION RESOURCE RELEASE COMMAND

-	PDU SESSION RESOURCE RELEASE RESPONSE

-	PDU SESSION RESOURCE MODIFY REQUEST

-	PDU SESSION RESOURCE MODIFY RESPONSE

-	PDU SESSION RESOURCE NOTIFY

-	INITIAL CONTEXT SETUP REQUEST

-	INITIAL CONTEXT SETUP RESPONSE

-	INITIAL CONTEXT SETUP FAILURE

-	UE CONTEXT RELEASE REQUEST

-	UE CONTEXT RELEASE COMMAND

-	UE CONTEXT RELEASE COMPLETE

-	UE CONTEXT MODIFICATION REQUEST

-	UE CONTEXT MODIFICATION RESPONSE

-	UE CONTEXT MODIFICATION FAILURE

-	INITIAL UE MESSAGE

-	DOWNLINK NAS TRANSPORT

-	UPLINK NAS TRANSPORT

-	NAS NON DELIVERY INDICATION

-	REROUTE NAS REQUEST

-	NG SETUP REQUEST

-	NG SETUP RESPONSE

-	NG SETUP FAILURE

-	RAN CONFIGURATION UPDATE

-	RAN CONFIGURATION UPDATE ACKNOWLEDGE

-	RAN CONFIGURATION UPDATE FAILURE

-	AMF CONFIGURATION UPDATE

-	AMF CONFIGURATION UPDATE ACKNOWLEDGE

-	AMF CONFIGURATION UPDATE FAILURE

-	NG RESET

-	NG RESET ACKNOWLEDGE

-	ERROR INDICATION

-	AMF STATUS INDICATION

-	OVERLOAD START

-	OVERLOAD STOP

-	UE TNLA BINDING RELEASE REQUEST

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

For the NGAP messages transferred between the Non-3GPP access network node and the AMF, the following exceptions to the specification in TS 38.413 [2] shall be applied:

PDU SESSION RESOURCE SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE;

PDU SESSION RESOURCE RELEASE COMMAND message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE.

PDU SESSION RESOURCE MODIFY REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE;

INITIAL CONTEXT SETUP REQUEST	message:

-	the following IEs shall be ignored, when received:

-	Core Network Assistance Information IE

-	Trace Activation IE

-	MobilityRestriction List IE

-	UE Radio Capability IE

-	Index to RAT/Frequency Selection Priority IE

-	Emergency Fallback Indicator IE

-	RRC Inactive Transition Report Request IE

-	UE Radio Capability for Paging IE

-	RG Level Wireline Access Characteristics IE: the information given within this IE between the W-AGF and the AMF shall be stored in the UE context by the W-AGF as specified in TS 23.316 [x].

-	Security Key IE: the information given within this IE between the TNGF and the AMF shall include the TNGF key as specified in TS 23.316 [x].

-	UE Aggregate Maximum Bit Rate IE: the information contained within this IE corresponds to the RG Total Maximum Bit Rate, as specified in TS 23.316 [x]. A W-AGF receiving the request message shall store and use the received RG Total Maximum Bit Rate for the concerned RG as specified in TS 23.316 [x].

UE CONTEXT RELEASE COMPLETE message:

-	the following IEs shall be ignored, when received:

-	Information on Recommended Cells and RAN Nodes for Paging IE

UE CONTEXT MODIFICATION REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	Index to RAT/Frequency Selection Priority IE

-	Core Network Assistance Information IE

-	Emergency Fallback Indicator IE

-	RRC Inactive Transition Report Request IE

-	if this is the first message received from a new AMF, the N3IWF shall identify the old AMF and the UE using the received RAN UE NGAP ID, release the UE-associated logical NG-connection to the old AMF and create a new UE-associated logical NG-connection to the new AMF

-	UE Aggregate Maximum Bit Rate IE: the information contained within this IE corresponds to the RG Total Maximum Bit Rate, as specified in TS 23.316 [x]. A W-AGF receiving the request message shall replace the previously provided value and use the received RG Total Maximum Bit Rate for the concerned RG as specified in TS 23.316 [x].

UE CONTEXT MODIFICATION RESPONSE message:

-	the following IEs shall be ignored, when received:

-	RRC State IE

INITIAL UE MESSAGE message:

-	RRC Establishment Cause IE: the information given within this IE is to indicate the establishment cause as specified in TS 23.502 [4].

-	Authenticated Indication IE: the information given within this IE between the W-AGF and the AMF is to indicate that the FN-RG has been authenticated by the wireline 5G access network as specified in TS 23.316 [x].

DOWNLINK NAS TRANSPORT message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	MobilityRestriction List IE

-	Index to RAT/Frequency Selection Priority IE

UPLINK NAS TRANSPORT message:

-	W-AGF Identity List IE: the information given within this IE between the W-AGF and the AMF contains a list of identifiers of N3 terminations at W-AGF as specified in TS 23.316 [x].

NG SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

RAN CONFIGURATION UPDATE message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

The Global RAN Node ID IE in the applicable NGAP messages between the N3IWF and the AMF includes the Global N3IWF ID as specified in TS 38.413 [2].

The Global RAN Node ID IE in the applicable NGAP messages between the W-AGF and the AMF includes the Global W-AGF ID as specified in TS 38.413 [2].

The User Location Information IE in the applicable NGAP messages between the N3IWF and the AMF includes the IP address and port number as specified in TS 38.413 [2].

The User Location Information IE in the applicable NGAP messages between the W-AGF and the AMF includes the Global Line Identifier or the Global Cable Identifier as specified in TS 38.413 [2].

The Security Key IE in the applicable NGAP messages includes the N3IWF key as specified in TS 33.501 [5].

The RAN UE NGAP ID IE in the applicable NGAP messages identifies the UE association over the NG interface within the N3IWF node, or the TNGF node, or the TWIF node, or the W-AGF node, as specified in TS 38.413 [2].

## 5.4	Handling of NGAP messages not specified to be applicable between the Non-3GPP access network node and AMF

If the Non-3GPP access network node or the AMF receive an NGAP message not listed in section 5.2 as being applicable between the Non-3GPP access network node and AMF, the receiving node shall act according to the criticality defined for the elementary procedure and ignore the message or discard the message and send an ERROR INDICATION message indicating that the procedure is not supported, as specified in in TS 38.413 [2].

End Change