3GPP TSG-RAN3 Meeting #106	R3-196833

Reno, Nevada, US, 18-22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.856 | CR | pCR | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (pCR TR 38.856) BL CR correction architectures alternatives discussion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-16 |  |  |  |  |  | Date: |  |  | 2019-11-1 |
| Category: | BF |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The descriptions of architecture alternatives does not include the general procedures, which is also the key feature for making the selection. - The drawbacks of each architecture alternatives are not highlighted, which is also the key point for making the selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add descriptions of impact on positioning procedures for architecture alternatives. - Add drawbacks of each architecture alternative. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unfair description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | xxx |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/**** Start Change ****/

# 5 		Local LMF in NG-RAN

Editor note: Specification impact of Local LMF including location of the LMF, potential new interface(s) (if any), impact on existing protocols, and coordination with the LMF in the 5GC

Editor note: Evaluation section (in case multiple solutions are captured) could be added

Editor note: The impact on positioning procedures should be evaluated and the procedure complexity should be considered when making the selection

## 5.1	Architecture

### 5.1.X 		Alternative 1

In architecture alternative 1, the LMC is an internal function of the NG-RAN node. In case of split gNB, the LMC is located in the gNB-CU-CP.

The interface between the LMC and the serving NG-RAN node is internal, and therefore minimizes the latency between the LMC and serving NG-RAN node. Functions of the NLs interface must be specified also for the NG-C interface.

Characteristics:

-	No new interfaces.

-	When UE positioning involves only TRPs within the NG-RAN node, positioning-related signalling is internal to the gNB.

-	To support location continuity in case of handover, LMC relocation to the target NG-RAN node can be enabled via enhancements to the XnAP Handover Preparation procedure. Further study is needed.

- Unclear location area management of the LMC: Deploying LMC on every NG-RAN may be redundant; deploying LMC on a part of NG-RAN may incur extra interface and extra procedures to XnAP/NGAP/NRPPa.

- Handover procedure may involve CN, such as AMF or LMF, to notify the change of LMC, e.g. NG Handover at pool border.

Procedures:

- LMC and NG-RAN: internal or XnAP procedures.

- LMC and UE: Uu procedures

- LMC and AMF/LMF: NGAP/NRPPa procedures

- Summary: No extra procedures required for UE positioning.

Figure 5.1.X-1  Alternative 1 – LMC as internal gNB function

### 5.1.Y 		Alternative 2

In architecture alternative #2, the LMC is a logical node within the split gNB connected to the gNB-CU-CP via the [FFS] interface.

This alternative requires a dedicated interface between the LMC and the serving NG-RAN node. The impacts to the NG-C interface are the same as alternative 1.

Characteristics:

-	New interface between the LMC and the gNB-CU-CP.

-	Allows LMC and gNB-CU-CP to be provided by different vendors.

-	Allows offloading of positioning support from a gNB-CU.

-	To support location continuity in case of handover, further study is needed.

- LMC can not be deployed at ng-eNB.

- Unclear location area management of the LMC: Deploying LMC on every NG-RAN may be redundant; deploying LMC on a part of NG-RAN may incur extra interface and extra procedures to XnAP/NGAP/NRPPa.

- Handover procedure may involve CN, such as AMF or LMF, to notify the change of LMC, and also the new interface may lead to higher positioning latancy during handover.

Procedures:

- LMC and NG-RAN: New interface, or new interface + XnAP.

- LMC and UE: new interface + Uu procedures

- LMC and AMF/LMF: new interface + NGAP/NRPPa procedures

- Summary: Extra procedures required for UE positioning, higheast latancy among 3 alternatives e.g. NG Handover at pool border

Figure 5.1.Y-1 Alternative 2 – LMC as logical node within split gNB

### 5.1.Z 		Alternative 3

In architecture alternative #3, the LMC is a new logical node in the NG-RAN, connected to NG-RAN nodes (gNBs and/or ng-eNBs) via the [FFS] interface.

This alternative requires a dedicated interface between the LMC and the serving NG-RAN node. The impacts to the NG-C interface are the same as alternative 1 and must be supported also by the new interface.

Characteristics:

-	New interface between the LMC and the NG-RAN node.

-	Allows LMC and NG-RAN nodes to be provided by different vendors.

-	Allows a single LMC to support multiple NG-RAN nodes (i.e. avoid introducing LMC in each individual NG-RAN node).

-	Allows offloading of positioning support from a gNB-CU.

### -	To support location continuity in case of handover when both source and target NG-RAN nodes are served by the same LMC, LMC relocation is not needed. However, when source and target NG-RAN nodes are served by different LMCs, support for location continuity requires further study.

### Handover procedure may involve CN, such as AMF or LMF, to notify the change of LMC e.g. NG Handover at pool border.

Procedures:

- LMC and NG-RAN: New interface

- LMC and UE: new interface + Uu procedures

- LMC and AMF/LMF: new interface + NGAP/NRPPa procedures

### Summary: Extra procedures required for UE positioning, latency between alternative 1 and 3.

Figure 5.1.Z-1 Alternative 3 – LMC as logical node in NG-RAN

/**** End of Change ****/