3GPP TSG-RAN WG3 Meeting #106	R3-196839

Reno, US, 18 – 22 November, 2019

Agenda item:	10.2.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	MRO for SN change failure

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses Mobility Robustness Optimisation for the SN change failure.

# 2	Discussion

In TR 37.816 [1] it is stated the RLF Indication and HO Report procedures can be reused for MRO of the SN change failure. The corresponding messages may be signalled between the MN and SN, and/or among the involved SNs.

The following observations taken from [2] have to be considered:

- Upon SCG failure the UE suspends SCG transmissions for all radio bearers and reports the SCG Failure Information to the MN, instead of triggering re-establishment.

- In all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN and the UE continues measurements based on configuration from the MN and the SN if possible. The SN measurements configured to be routed via the MN will continue to be reported after the SCG failure.

- The UE includes in the SCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN. The MN handles the SCG Failure Information message and may decide to keep, change, or release the SN/SCG.

In contrast to MCG failure or single connectivity failure scenarios, in case of SCG failure the UE remains connected with the MN and the MN receives the SCG Failure Information containing the UE’s measurements up to the moment of sending this message.

For MRO analysis we have two alternatives:

In section 2.1 we discuss the situation when the MRO analysis is done at the MN.

In section 2.2 we discuss the situation when the MRO analysis is done at the SN.

## 2.1	MRO analysis is performed at the MN

Remaining connected with the UE enables the MN to perform MRO root cause analysis after SN change failures. This is shown in the example flow below in Figure 1:

Figure 1: MRO handling for the SN Change failure while a SN initiated SN change is ongoing

The example flows below in Figure 2 show the cases when SCG failure occurs shortly after a successful SN change (SN change too early or to wrong cell):

Figure 2: MRO handling for the SN Change failure shortly after a successfully completed SN initiated SN change

Figure 3 below shows MRO handling for the case of a SCG failure that occurs after the UE has already stayed for a long period of time in the cell of the SN (SN change too late). To avoid ambiguities, it is assumed that “long time after successful handover” starts immediately after “short time after successful handover” has expired. Also, for MRO the source and target roles of a previously successfully performed handover are no longer of significance. Therefore T-SN is renamed to SN2 and SN1 may be any other SN. For MRO this is the same condition as after addition of a SN without a preceding SN change.

Figure 3: MRO handling for the SN Change failure after already staying for a long time in a cell

## 2.2	MRO analysis is performed at the SNs

In this section the same failure situations are handled as in the previous section. The difference is that the MRO root cause analysis is left to the SNs. There is one exception for the too late SN change. In this situation both MN and SN may have to optimize their thresholds for SN change, because actually both failed to initiate the SN change (at least if the problem was not a coverage problem).

Figure 4: MRO handling for the SN Change failure while a SN initiated SN change is ongoing

The following Figure 5 shows a SN change too early failure situation with MRO analysis in the SNs. The MN just forwards the measurements to the current SN using the “SN Failure Indication” message. This current SN is the T-SN of a preceding SN Change and, because the SN failure happened a short time after the successful handover, the T-SN again forwards the measurements to the S-SN using the Handover Report message. The S-SN performs the MRO root cause analysis and may adjust the parameters accordingly.

Figure 5: MRO handling for the SN Change failure shortly after a successfully completed SN initiated SN change

Figure 6 below shows the situation of a SN change failure after a long time of a preceding SN change. This is the same situation as after freshly adding a SN without having performed and preceding SN change. The MN forwards the measurement results using the SN Failure indication message, but different to the previous example in case of a too late SN change occurred at the SN both MN and SN should perform a MRO root cause analysis because each node failed to initiate the SN change (at least if it is not caused by a coverage problem).

Figure 6: MRO handling for the SN Change failure after already staying for a long time in a cell

## 2.3	Conclusions

The following conclusions can be made for MRO handling of the SN change failure:

Due to the fact of receiving SGC failure information from the UE and due to knowing about previously successfully completed SN changes the MN is already in the position to perform MRO root cause analysis for the SN change failure:

- In case of a MN initiated SN change neither an RLF Indication message nor a HO Report like message are needed.

- In case of SN initiated SN change or missing of a necessary SN change the signalling is different depending MRO analysis is done at the MN or is done at the SNs

  - In case the MRO analysis is done by the MN the MN identifies the SN which may needs to optimize the parameters for SN change using a Handover Report like message. The SN may ignore the MRO root cause provided by the Handover Report message and may use the received measurements to perform MRO root cause analysis.

  - In case the MRO analysis is done by the SNs the MN forwards the measurements to the currently serving SN using a SN Failure Indication. This indicates to the SN to perform root cause analysis.
If the SN node receiving the SN Failure Indication recognizes that there was a successful SN Change a short time ago then it forwards the measurements to the source SN for MRO root cause analysis and parameter optimization using a Handover Report message. If the Handover Report message contains MRO root cause the S-SN may ignore it and may use the forwarded measurements to perform an MRO root cause analysis.

- The two concepts 2a and 2b may be used simultaneously. Receiving a SN Failure indication always indicates that an MRO root cause analysis has to be made by SNs. The MN doesn’t need to determine to which SN (source or target) it has to send the message. The MN just sends it to the SN that is currently serving the UE. On the other hand, if the MN provides a root cause analysis and provides it within Handover Report to the SN the SN, if capable to perform MRO root cause analysis, is not obliged to use the MRO root cause provided by the MN but may use the result of its own MRO root cause analysis based on the forwarded measurement results.

# 3	Proposals

Following conclusion 3, that it is possible to support both the concepts 2a (MN performs provides MRO root cause and forwards measurements)  and 2b (MN just forwards the measurments to the currently serving SN) we have the following proposals::

Proposal 1: An RLF Indication like message e.g. SN Failure Indication is  is needed for scenarios where the MN does not perform MRO root cause analysis after or while SN initiated SN change and just forwards the received measurements to the currently serving SN.

Proposal 2: Introduce a Handover Report like message from MN to SN for optimizing the triggers of the SN initiated SN Change.

Proposal 3: A SN receiving a Handover Report like message providing a MRO root cause may, if capable, ignore that parameter and use the provided measurements to perform MRO root cause analysis.

# 4	References

[1]		TR 37.816 Study on RAN centric data collection and utilization for LTE and NR

[2]		TS 37.340 E-UTRA and NR Multi-connectivity, Stage 2