3GPP TSG-RAN WG3 Meeting #106	R3-196840

Reno (NV), USA, 18 – 22 November, 2019

Agenda item:	10.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP for SON BL CR for TS 38.423] Discussion and updates of slice load, beam load and TNL load reporting

Document for:	Text Proposal

# 1	Introduction

Load reporting per slice and beam was introduced in the stage 3 BL CRs at RAN3#105bis, associated with FFSs. The proposed reporting uses in both cases the Composite Available Capacity metric indicating relative capacity (percentage). An absolute reference therefore seems needed, and for this purpose we reiterate proposals from offline discussions at previous RAN3 meeting [1].

A TP based on BL CR for TS 38.423 [2] is provided in annex of this paper.

# 2	Discussion

## 2.1	Reporting of Slice Available Capacity

As per SA5 decision, slices may come with floating resource quotas as discussed at RAN3#105bis [1]:

Policy for split of radio resources between slices is defined in TS 28.541. The resulting allocation policy may allow resource usage based on statistical multiplexing, i.e. if a defined slice doesn’t need all its allocated resources, these unused resources, or part of them, may be used by other slices (“float quota”). The available resource in a slice may therefore depend on resource usage in other slice groups.

RRM policy as defined by SA5 is understood to be applicable at cell level.

The following principles to express per-slice available capacity are therefore proposed:

- The available capacity per slice is expressed as a percentage of the total cell capacity (Cell Capacity Class Value).

- The available capacity in a slice expresses the available resources for the slice according to the resource usage in the cell and the RRM policy for statistical multiplexing.

As a consequence, the sum of reported per-slice available capacity may exceed the total available cell capacity. It is therefore proposed to report:

- Total available cell capacity

- Available capacity per slice

Signalling support for the above was already captured in the BL CR, and we believe that the associated FFSs can be removed. Similarly an activation bit in the RESOURCE STATUS REQUEST message can be introduced for Slice Available Capacity.

Proposal 1: Remove FFSs relative to Slice Available Capacity and introduce activation bit in the RESOURCE STATUS REQUEST message.

## 2.2	Slice reporting in network sharing scenario

The BL CR [2] includes a ‘flat’ list of S-NSSAI, i.e. not structured per PLMNs. This is not aligned with existing NGAP and XnAP specification where S-NSSAI lists are provided per PLMN-ID, and in this way avoids inter-PLMN coordination of S-NSSAIs. We propose to align the BL CR on the existing NGAP and XnAP principle.

Proposal 2: Align the BL CR on the existing NGAP and XnAP principle for signaling of S-NSSAI lists.

## 2.3	Reporting of SSB Area Available Capacity

The NR radio design is highly complex, and RAN3’s current approach for reporting of SSB area (“beam”) available capacity is based on the possibility to identify, in the spatial domain, an SSB for the DL and UL traffic as we have discussed at RAN3#105bis in R3-195983.

However combined with frequency domain the NR design provides the possibility for highly complex configurations, e.g. where the beam structure under each SSB can be different as illustrated in Fig. 1 and Fig. 2 below.

Figure 1: Example of multiple SSBs in a carrier (from TS 38.300)

Figure 2: Use of different Beam Configuration for different BWPs

In this case reporting of available capacity per SSB area will be ambiguous or not possible. It should therefore be clarified that the Rel-16 specification is based on an assumption that all the bandwidth parts (BWP) in the cell use the same SSB- and beam design.

Proposal 3: Assume for Rel-16 SSB Area Available Capacity reporting that all BWPs within a cell use the same design in the spatial domain (SSB- and beam design).

In current BL CR, the procedural text (marked as FFS) includes mandate to report the SSB area available capacity when the cell supports multiple SSBs. But as described above this may not be possible for some physical layer configuration. The mandated reporting should therefore be replaced by optional reporting.

Proposal 4: Optional reporting of SSB area available capacity in cells supporting multiple SSBs.

Based on this assumption, we believe that Rel-16 SSB Area Available Capacity reporting should be flexible enough to handle radio deployments where the capacity available in a given SSB area may depend on the capacity consumed in other SSB areas. This can be considered as being similar to floating slice resource quotas (see section 2.1). The same solution could therefore be used, i.e. that the reported SSB area available capacity corresponds to available capacity taking into account the capacity consumption in other SSB areas within the cell. Based on this, the reported SSB area available capacity may be weighted according to the ratio of cell capacity class value.

Proposal 5: The reported SSB area available capacity corresponds to available capacity taking into account the capacity consumption in other SSB areas within the cell.

Proposal 6: Similar to slice available capacity, the SSB area available capacity is weighted according to the ratio of cell capacity class value.

## 2.4	Reporting of TNL Load

Proposals for TNL load reporting metrics, incorporated also in the XnAP TP below, can be found in [3].

In regard to TNL load reporting to an external peer to the gNB, the disaggregated architecture provides some challenges as to which information should be reported over Xn and X2 interfaces. Firstly, the TNL as defined in LTE corresponds to TNL towards the backhaul. That is, the S1-U interface. In contrast, the TNL reports that a gNB-CU-CP will receive correspond to fronthaul F1-U when received from a gNB-DU, and to S1-U/NG-U backhaul when received from a gNB-CU-UP. Therefore, there is an ambiguity as to what should be reported toward the external peer over X2/Xn, and whether this should correspond to the backhaul (as in LTE), or to the fronthaul. E.g., if only backhaul is reported (as in LTE), the MN or Source gNB may incorrectly be led to assume that the cell has enough TNL resources for the incoming UE. However, the fronthaul link may be already congested towards the selected cell and lead to poor performance or a rejection. A similar situation can occur in opposite direction (fronthaul not congested, backhaul congested). Therefore, given that both fronthaul (F1-U) and backhaul (NG-U) can be a potential bottleneck, it is most appropriate to report both towards the external peer.

Proposal 7: The TNL load reports toward an external peer over X2/Xn shall include both backhaul (S1-U/NG-U) and fronthaul links (F1-U) reported separately.

# 3	Conclusion

We have made the following proposals:

Proposal 1: Remove FFSs relative to Slice Available Capacity and introduce activation bit in the RESOURCE STATUS REQUEST message.

Proposal 2: Align the BL CR on the existing NGAP and XnAP principle for signaling of S-NSSAI lists.

Proposal 3: Assume for Rel-16 SSB Area Available Capacity reporting that all BWPs within a cell use the same design in the spatial domain (SSB- and beam design).

Proposal 5: The reported SSB area available capacity corresponds to available capacity taking into account the capacity consumption in other SSB areas within the cell.

Proposal 6: Similar to slice available capacity, the SSB area available capacity is weighted according to the ratio of cell capacity class value.

Proposal 7: The TNL load reports toward an external peer over X2/Xn shall include both backhaul (S1-U/NG-U) and fronthaul links (F1-U) reported separately.

A corresponding text proposal is included in annex of this paper.

# References

[1]		R3-196161, Load report per slice – summary of discussion, Nokia, Nokia Shanghai Bell, Huawei

[2]		R3-196314, SON BL CR to TS 38.423

[3]	R3-196844, [TP for SON BL CR for TS 38.463] Discussion and updates of TNL and HW load reporting, Nokia, Nokia Shanghai Bell

# Annex	- TP for SON BL CR to TR 38.323

<<< start of changes >>>

### 8.4.A	Resource Status Reporting Initiation

Editor’s note: The content of this section is FFS

#### 8.4.A.1	General

This procedure is used by an NG-RAN node1 to request the reporting of load measurements to another NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.4.A.2	Successful Operation

Figure 8.4.A.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from NG-RAN node1 to NG-RAN node2 to start a measurement, stop a measurement, add cells to report for a measurement or remove certain cells from a measurement.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects NG-RAN node2 shall perform measurements on. For each cell, the NG-RAN node2 shall include in the RESOURCE STATUS UPDATE message:

-	[This paragraph is FFS] the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the NG-RAN node2 is a gNB and if the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status List IE;

-	the NG TNL Available Capacity Indicator IE and the F1 TNL Available Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. [The following text in this paragraph is FFS] If the NG-RAN node2 is a gNB and if the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB, the Composite Available Capacity Group IE for such cell may include the SSB Area Capacity Value IE and the SSB Area Capacity Class Value List IE;

#### 8.4.A.3	Unsuccessful Operation

Figure 8.4.A.3-1: Resource Status Reporting Initiation, unsuccessful operation

If none of the requested measurements can be initiated, NG-RAN node2 shall send a RESOURCE STATUS FAILURE message.

#### 8.4.A.4	Abnormal Conditions

FFS

<<< next change >>>

#### 9.1.3.A	RESOURCE STATUS REQUEST

Editor’s note: The content of this section is FFS

This message is sent by an NG-RAN node1 to neighbouring NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  |  |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 |  |  |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporPartialStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 |  |  |
| Registration Request | M |  | ENUMERATED(start, stop, partial stop, add, …) | Type of request for which the resource status is required. |  |  |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW Load Ind Periodic [FFS], Fifth Bit = Number of Active UEs, Sixth Bit = Number of RRC connections, Seventh Bit = Slice Available Capacity. Other bits shall be ignored by the NG-RAN node2. |  |  |
| Cell To Report |  | 1 |  | Cell ID list to which the request applies. |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>Slice To Report List [FFS] |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Broadcast PLMNs |  | 1..<maxnoofsupportedPLMNs> |  |  |  |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |  |  |
| >>>>Slice To Report Item |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>>>>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Load Ind Periodic, Composite Available Capacity Periodic. |  |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSliceItems [FFS] | Maximum no. of signalled slice support items. Value is 1024. [FFS] |
| maxnoofsupportedPLMNs | Maximum no. of PLMNs supported by an NG-RAN node. Value is 12. |

<<< next change >>>

#### 9.1.3.B	RESOURCE STATUS RESPONSE

Editor’s note: The content of this section is FFS

This message is sent by the NG-RAN node2 to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

<<< next change >>>

#### 9.1.3.C	RESOURCE STATUS FAILURE

Editor’s note: The content of this section is FFS

This message is sent by the NG-RAN node2 to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 | Ignored by the receiver when the Complete Failure Cause Information IE is included. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

<<< next change >>>

#### 9.1.3.D	RESOURCE STATUS UPDATE

Editor’s note: The content of this section is FFS

This message is sent by NG-RAN node2 to neighbouring NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  |  |  |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>Radio Resource Status [FFS] | O |  | 9.2.2.yy |  |  |  |
| >>NG TNL Available Capacity | O |  | 9.2.2.xx |  |  |  |
| >>F1 TNL Available Capacity Indicator | O |  | 9.2.2.xx |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.zz |  |  |  |
| >>Slice Available Capacity | O |  | 9.2.2.jj |  |  |  |
| >>Hardware Load Indicator [FFS on the IE and on reporting per cell] | O |  | 9.2.2.ww |  |  |  |
| >>Number of Active UEs [FFS] | O |  | INTEGER (1..65536,...) |  |  |  |
| >>Number RRC Connections [FFS] | O |  | INTEGER (1..65536,...) |  |  |  |

| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| --- | --- |

<<< next change >>>

#### 9.2.2.x	NG-RAN CGI

This IE is used to globally identify a cell in NG-RAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RAT | M |  |  |  |
| >nr |  |  |  |  |
| >>NR CGI | M |  | 9.2.2.7 |  |
| >e-utra |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.2.2.8 |  |

#### 9.2.2.xx	TNL Available Capacity Indicator

The TNLAvailable Capacity Indicator IE indicates the status of the Transport Network Layer available capacity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL TNL Relative Available Capacity | M |  | INTEGER(0..100, …) | Indicates the amount of downlink transport network level available capacity. Value 100 corresponds to the nominal capacity. |
| DL TNL Absolute Available Capacity | M |  | INTEGER (1.. 16777,...) Max value is FFS | Available DL TNL capacity in Mbps |
| UL TNL Relative Available Capacity | M |  | INTEGER(0..100, …) | Indicates the amount of uplink transport network level available capacity. Value 100 corresponds to the nominal capacity. |
| UL TNL Available Capacity | M |  | INTEGER (1.. 16777,...) Max value is FFS | Available UL TNL capacity in Mbps |

#### 9.2.2.yy	Radio Resource Status

[The information is FFS]

The Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage |
| UL GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage |
| DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage |
| UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage |
| DL Total PRB usage | M |  | INTEGER (0..100) | Per cell DL Total PRB usage |
| UL Total PRB usage | M |  | INTEGER (0..100) | Per cell UL Total PRB usage |
| DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| SSB Area Radio Resource Status List |  | 0..<maxnoofSSBAreas> |  |  |
| > SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage |
| > SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage |
| > SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage |
| > SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage |
| > SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage |
| > SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.2.2.zz	Composite Available Capacity Group

The Composite Available Capacity Group IE indicates the overall available resource level per cell and per SSB area in the cell in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | Composite Available Capacity 9.2.2.a | For the Downlink | - |  |
| Composite Available Capacity Uplink | M |  | Composite Available Capacity 9.2.2.a | For the Uplink | - |  |

#### 9.2.2.a	Composite Available Capacity

The Composite Available Capacity IE indicates the overall available resource level in the cell in either Downlink or Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Capacity Class Value | O |  | 9.2.2.b |  | - |  |
| Capacity Value | M |  | 9.2.2.c | ‘0’ indicates no resource is available, Measured on a linear scale. | - |  |

#### 9.2.2.b	Cell Capacity Class Value

The Cell Capacity Class Value IE indicates the value that classifies the cell capacity with regards to the other cells. The Cell Capacity Class Value IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity Class Value | M |  | INTEGER (1..100,...) | Value 1 shall indicate the minimum cell capacity, and 100 shall indicate the maximum cell capacity. There should be a linear relation between cell capacity and Cell Capacity Class Value. |  |  |

#### 9.2.2.c	Capacity Value

[The information per SSB area is FFS]

The Capacity Value IE indicates the amount of resources per cell and per SSB area that are available relative to the total NG-RAN resources. The capacity value should be measured and reported so that the minimum NG-RAN resource usage of existing services is reserved according to implementation. Each of the IEs Capacity Value and SSB Area Capacity Value can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity with respect to the whole cell. Capacity Value should be measured on a linear scale. |  |  |
| SSB Area Capacity Value |  | 0..<maxnoofSSBAreas> |  |  |  |  |
| > SSB Area Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . SSB Area Capacity Value should be measured on a linear scale. | - |  |

#### 9.2.2.jj	Slice Available Capacity

The Slice Available Capacity IE indicates the amount of resources per network slice that are available relative to the total NG-RAN resources. The Slice Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Broadcast PLMNs |  | 1..<maxnoofsupportedPLMNs> |  |  |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |  |  |
| >Slice Available Capacity |  | 1..< maxnoofSliceItems > |  |  |  |  |
| >> Slice Available Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity. Slice Capacity Value should be measured on a linear scale. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofsupportedPLMNs | Maximum no. of PLMNs supported by an NG-RAN node. Value is 12. |
| maxnoofSliceItems [FFS] | Maximum no. of signalled slice support items. Value is 1024. [FFS] |

<<< end of changes >>>