3GPP TSG-RAN WG3 Meeting #106	R3-196852

Reno (NV), USA, 18 – 22 November, 2019

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP for TS 38.423] Introduction of SRS-RSRP configuration

Document for:	Text Proposal

# 1	Introduction

This TP is based on R3-192212 introducing changes for CLI in XnAP, and shows implementation of proposal 1 in [1].

# References

[1]		R3-196850, Network signalling related to SRS-RSRP measurements, Nokia, Nokia Shanghai Bell

# Annex	- TP for TR 38.423

<<< start of changes >>>

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID |
| NR CGI | M |  | 9.2.2.7 |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| CHOICE NR-Mode-Info | M |  |  |  |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>Intended TDD DL-UL Configuration NR | O |  | 9.2.2.x |  |
| >>>Cell SRS Configuration | O |  | OCTET STRING | Contains the CellSRSConfiguration inter-node message defined in TS 38.331 [10]. |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. |
| Connectivity Support | M |  | 9.2.2.28 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

<<< end of changes >>>