3GPP TSG-RAN3 Meeting #106	R3-196892

Reno, Nevada, US, 18-22 November 2019

Title: 	(TP for MDT BL CR for TS 38.423): Support of MDT for RRC_INACTIVE UEs

Source: 	Huawei

Agenda item:	10.3.2

Document for:	Other

1. Introduction

At last meeting of RAN2, RAN2 has the following agreements for MDT for inactive UE.

3	If the signalling based logged MDT received by the NG-RAN when UE is in RRC_INACTIVE:

⁻	The NG-RAN stores the logged MDT configuration in the UE context.

⁻	When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE.

⁻	When the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the logged MDT configuration to the new NG-RAN.

This agreement should be confirmed by RAN3.

4	If the management based logged MDT received by the NG-RAN when UE is in RRC_INACTIVE,

⁻	No requirement for the NG-RAN to store the logged MDT configuration in the UE context

⁻	When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE.

⁻	When the UE resumes the RRC connection in another NG-RAN, the source NG-RAN will not propagate the management based logged MDT configuration. The source NG-RAN should inform the target NG-RAN of UE consents.

RAN2 expects RAN3 to confirm those agreements. In this contribution, we will discuss the MDT processing for RRC_INACTIVE UEs based on RAN2 agreements.

2. Discussion

2.1 Receiving signalling based MDT configuration when UE is in RRC_INACTIVE

RAN2 achieves the following agreements for signalling based logged MDT:

3	If the signalling based logged MDT received by the NG-RAN when UE is in RRC_INACTIVE:

⁻	The NG-RAN stores the logged MDT configuration in the UE context.

⁻	When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE.

⁻	When the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the logged MDT configuration to the new NG-RAN.

This agreement should be confirmed by RAN3.

According to the TS 38.413, the NG-RAN can inform the AMF of the RRC state in RRC Inactive Transition Report message if the NG-RAN supports the reporting and receives the reporting request from AMF, i.e. the 5GC knows whether the UE is in RRC_INACTIVE. In this case, we think the AMF will not send the MDT configuration.

In case the RRC state reporting is not activated for the UE and the AMF does not know whether the UE is in RRC_INACTIVE, the NG-RAN can store the MDT configuration and propagate it to the target NG-RAN. Therefore, the NG-RAN does not need to page the UE in this case.

Although, RAN2 agreements only apply to signalling based logged MDT, we think those agreements can be applicable for signalling based immediate MDT as well.

It is proposed RAN3 to agree on the following proposal:

If signalling based MDT received by the NG-RAN when UE is in RRC_INACTIVE:

- The NG-RAN stores both the logged MDT and immediate MDT configuration in the UE context.

- When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN node can configure the logged and immediate MDT configuration for the UE.

- When the UE resumes RRC connection in a new NG-RAN node, the last serving NG-RAN node can propagate the logged MDT and immediate MDT configuration to the new NG-RAN node.

2.2 Receiving management based MDT configuration when UE is in RRC_INACTIVE

RAN2 achieves the following agreements for management based logged MDT:

4	If the management based logged MDT received by the NG-RAN when UE is in RRC_INACTIVE,

⁻	No requirement for the NG-RAN to store the logged MDT configuration in the UE context

⁻	When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE.

⁻	When the UE resumes the RRC connection in another NG-RAN, the source NG-RAN will not propagate the management based logged MDT configuration. The source NG-RAN should inform the target NG-RAN of UE consents.

For the management based logged MDT, the NG-RAN node will select UEs according to its configuration received from OAM. The last serving gNB does not know whether the UE is in the MDT area scope or not. In our understanding, if the network wants to configure the management based logged MDT for inactive UE, the network should configure according to configuration received by the new gNB, i.e. the NG-RAN node will not propagate the management based logged MDT configuration.

Therefore we think the NG-RAN node does not need to require to page the RRC_INACTIVE UE when the NG-RAN node receives the MDT configuration from OAM. When UE resume in the last serving NG-RAN node, the NG-RAN node can reselect this UE based on the UE selection rules and then configures the MDT. When UE resumes in a new NG-RAN, the new NG-RAN node can configure the MDT configuration based on the logged MDT configuration received from the OAM.

For management based immediate MDT, we think there is no need for the NG-RAN node to store the immediate MDT configuration in the UE specific context. However, the MDT configuration should be kept at the NG-RAN ndoe level untill receiving the deactivation command from OAM. There is also no needed to propogate the management based immediate MDT configuration from last serving NG-RAN node to new serving NG-RAN node.

It is proposed RAN3 to agree on the following proposals:

For the management based logged MDT received by the NG-RAN when UE is in RRC_INACTIVE:

- The NG-RAN does not store the logged MDT in the UE context

- When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE.

- When the UE resumes the RRC connection in another NG-RAN, the source NG-RAN will not propagate the management based logged MDT.

- The source NG-RAN should inform the target NG-RAN of UE consents.

- Proposal 3: For the management based immediate MDT received by the NG-RAN:

- The NG-RAN keeps the management based immediate MDT configuration until receiving deactivation command from OAM.

- The NG-RAN selects only active UEs to perform management based immediate MDT task.

3. Conclusion

In this contribution, we discussed the MDT handling for RRC_INACTIVE UEs. We has the following proposals:

- If signalling based MDT received by the NG-RAN when UE is in RRC_INACTIVE:

- The NG-RAN stores both the logged MDT and immediate MDT configuration in the UE context.

- When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN node can configure the logged and immediate MDT configuration for the UE.

- When the UE resumes RRC connection in a new NG-RAN node, the last serving NG-RAN node can propagate the logged MDT and immediate MDT configuration to the new NG-RAN node.

For the management based logged MDT received by the NG-RAN when UE is in RRC_INACTIVE:

- The NG-RAN does not store the logged MDT in the UE context

- When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE.

- When the UE resumes the RRC connection in another NG-RAN, the source NG-RAN will not propagate the management based logged MDT.

- The source NG-RAN should inform the target NG-RAN of UE consents.

Proposal 3: For the management based immediate MDT received by the NG-RAN:

- The NG-RAN keeps the management based immediate MDT configuration until receiving deactivation command from OAM.

- The NG-RAN selects only active UEs to perform management based immediate MDT task.

We also provides the corresponding TPs in section 4 for these proposals.

4. TPs for MDT BL CR for TS 38.423 (based on R3-196889)

Start Change

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Index to RAT/Frequency Selection Priority IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message the target NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23]. In particular, the new NG-RAN shall, if supported:

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [23];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to "Immediate MDT and Trace", initiate the requested trace session and MDT session as described in TS 32.422 [23];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to "Immediate MDT Only", initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN shall ignore Interfaces To Trace IE, and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the new NG-RAN may use it to propagate the MDT Configuration as described in TS 37.320 [38];

-	if the Trace Activation IE includes the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [38].

-	if the Trace Activation IE includes the WLAN Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [38].

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Logged MDT only”, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN shall ignore Interfaces To Trace IE, and Trace Depth IE;

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

If the old NG-RAN node is not able to identify the UE context by means of the UE Context ID, or if the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message is not valid, or, if it decides not to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message.

If the old NG-RAN node decides to keep the UE context in case of periodic RNAU, it shall store the Allocated C-RNTI IE and the Access PCI IE in the UE Context ID IE, as described in TS 38.300 [9].

If the Old NG-RAN node to New NG-RAN node Resume Container IE is included in the RETRIEVE UE CONTEXT FAILURE message, the new NG-RAN node should transparently forward the content of this IE to the UE as described in TS 38.300 [9].

#### 8.2.4.4	Abnormal Conditions

Void.

Next Change

#### 9.2.3.xx4	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Immediate MDT and Trace,…) |  | - | - |
| CHOICE Area Scope of MDT | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>>NR CGI | M |  | 9.2.2.7 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | OCTET STRING (2) | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAC | M |  | OCTET STRING (2) | Tracking Area Code |  |  |
| >>>PLMN Identity | M |  | 9.2.2.4 |  |  |  |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>M1 Configuration | O |  | 9.2.3.xx4 |  |  |  |
| >>M2 Configuration | O |  | ENUMERATED (true, …) |  |  |  |
| >>M3 Configuration | O |  | 9.2.3.xx5 |  |  |  |
| >>M4 Configuration | O |  | 9.2.3.xx6 |  |  |  |
| >>M5 Configuration | O |  | 9.2.3.xx7 |  |  |  |
| >>M6 Configuration | O |  | 9.2.3.xx8 |  |  |  |
| >>M7 Configuration | O |  | 9.2.3.xx9 |  |  |  |
| >>M8 Configuration | O |  | 9.2.3.xx10 |  |  |  |
| >>M9 Configuration | O |  | 9.2.3.xx11 |  |  |  |
| >>MDT Location Information | O |  | 9.2.3.xx12 |  |  |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | FFS |  |  |  |
| >>Logging duration | M |  | FFS |  |  |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.xx10 |  |  |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.xx11 |  |  |  |
| >>Logging MDT Type Indicator | M |  | ENUMERATED (only out-of-coverage, only in-coverage, out-of coverage and in-coverage) |  |  |  |
| >>Logging MDT NR Frequencies Configuration | O |  | FFS |  |  |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.xx3 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

#### 9.2.3.xx5	MDT Configuration- EUTRA

The IE defines the MDT configuration parameters of EUTRA.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED(Immediate MDT only, Immediate MDT and Trace,…) |  | - | - |
| CHOICE Area Scope of MDT | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>>E-UTRA CGI | M |  | 9.2.2.8 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | OCTET STRING (2) | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAC | M |  | OCTET STRING (2) | Tracking Area Code |  |  |
| >>>PLMN Identity | M |  | 9.2.2.4 |  |  |  |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>M1 Configuration | O |  | 9.2.3.xx6 |  |  |  |
| >>M2 Configuration | O |  | ENUMERATED (true, …) |  |  |  |
| >>M3 Configuration | O |  | 9.2.3.xx7 |  |  |  |
| >>M4 Configuration | O |  | 9.2.3.xx8 |  |  |  |
| >>M5 Configuration | O |  | 9.2.3.xx9 |  |  |  |
| >>M6 Configuration | O |  | 9.2.3.xx10 |  |  |  |
| >>M7 Configuration | O |  | 9.2.3.xx11 |  |  |  |
| >>M8 Configuration | O |  | 9.2.3.xx12 |  |  |  |
| >>M9 Configuration | O |  | 9.2.3.xx13 |  |  |  |
| >>MDT Location Information | O |  | 9.2.3.xx14 |  |  |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | FFS |  |  |  |
| >>Logging duration | M |  | FFS |  |  |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.xx15 |  |  |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.xx16 |  |  |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.xx3 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

Next Change

#### 9.2.3.xx6	M1 Configuration

This IE defines the parameters for M1 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, A2event-triggered periodic, …) |  | - | - |
| M1 Threshold Event A2 | C-ifM1A2trigger |  |  | Included in case of event-triggered or event-triggered periodic reporting for measurement M1. | - | - |
| >CHOICE Threshold | M |  |  |  | - | - |
| >>RSRP |  |  |  |  |  | - |
| >>>Threshold RSRP | M |  | FFS | This IE is defined in TS 38.331 [10]. | - | - |
| >>RSRQ |  |  |  |  |  | - |
| >>>Threshold RSRQ | M |  | FFS | This IE is defined in TS 38.331 [10]. | - | - |
| >>SINR |  |  |  |  |  |  |
| >>>Threshold SINR | M |  | FFS | This IE is defined in TS 38.331 [10]. | - | - |
| M1 Periodic reporting | C-ifperiodicMDT |  |  | Included in case of periodic or event-triggered periodic reporting for measurement M1. | - | - |
| >Report interval | M |  | FFS | This IE is defined in TS 38.331 [10]. | - | - |
| >Report amount | M |  | FFS | Number of reports. | - | - |

#### 9.2.3.xx7	M3 Configuration

This IE defines the parameters for M3 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M3 Collection Period | M |  | FFS |  |

#### 9.2.3.xx8	M4 Configuration

This IE defines the parameters for M4 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M4 Collection Period | M |  | FFS |  |
| M4 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.2.3.xx9	M5 Configuration

This IE defines the parameters for M5 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M5 Collection Period | M |  | FFS |  |
| M5 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.2.3.xx10	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | FFS |  |
| M6 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.2.3.xx11	M7 Configuration

This IE defines the parameters for M7 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | FFS | Unit: minutes |
| M7 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.2.3.xx12	M8 Configuration

This IE defines the parameters for Bluetooth measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bluetooth Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| Bluetooth Measurement Configuration Name List |  | 0..1 |  |  |
| >Bluetooth Measurement Configuration Name Item IEs |  | 1 .. <maxnoofBluetoothName> |  |  |
| >>Bluetooth Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..248)) |  |
| BT RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 measurement as defined in 37.320 [38]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofBluetoothName | Maximum no. of Bluetooth local name used for Bluetooth measurement collection. Value is 4. |

#### 9.2.3.xx13	M9 Configuration

This IE defines the parameters for WLAN measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WLAN Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| WLAN Measurement Configuration Name List |  | 0..1 |  |  |
| >WLAN Measurement Configuration Name Item IEs |  | 1 .. <maxnoofWLANName> |  |  |
| >>WLAN Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..32)) |  |
| WLAN RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 as defined in 37.320 [38]. |
| WLAN RTT | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M9 as defined in 37.320 [38]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofWLANName | Maximum no. of WLAN SSID used for WLAN measurement collection. Value is 4. |

#### 9.3.1.xx14	MDT Location Information

This IE defines the MDT Location Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [38]. First Bit = GNSS Second Bit = E-CID information. Other bits are reserved for future use and are ignored if received. Value “1” indicates “activate” and value “0” indicates “do not activate”. |

9.3.1.x15	Bluetooth Measurement Configuration

This IE defines the parameters for Bluetooth measurement collection.

Editor note: the definition of this IE may need to be updated and pending to RAN2 discussion.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bluetooth Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| Bluetooth Measurement Configuration Name List |  | 0..1 |  |  |
| >Bluetooth Measurement Configuration Name Item IEs |  | 1 .. <maxnoofBluetoothName> |  |  |
| >>Bluetooth Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..248)) |  |
| BT RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 measurement as defined in 37.320 [31]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofBluetoothName | Maximum no. of Bluetooth local name used for Bluetooth measurement collection. Value is 4. |

9.2.1.x16	WLAN Measurement Configuration

This IE defines the parameters for WLAN measurement collection.

Editor note: the definition of this IE may need to be updated and pending to RAN2 discussion.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WLAN Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| WLAN Measurement Configuration Name List |  | 0..1 |  |  |
| >WLAN Measurement Configuration Name Item IEs |  | 1 .. <maxnoofWLANName> |  |  |
| >>WLAN Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..32)) |  |
| WLAN RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 as defined in 37.320 [31]. |
| WLAN RTT | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M9 as defined in 37.320 [31]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofWLANName | Maximum no. of WLAN SSID used for WLAN measurement collection. Value is 4. |

End Change