3GPP TSG-RAN3 Meeting #106	R3-196918

Reno, NV, USA, 18-22 November 2019

Title: 	(TP for NR_IIOT BL CR for TS 38.300): Higher layer duplication

Source: 	Huawei

Agenda item:	17.2.4.1

Document for:	Discussion

1. Introduction

For higher layer duplication solution 1, at the last RAN3#105, solution#1 based multi-connectivity for higher-layer redundancy transmission was further discussed with the following way forward.

- Support the solution by using RSN;

- If the PDU session pair information can be provided to NG-RAN node, enhancements can be made to gain more flexibility.

Also a LS is sent to SA2 for the following question in [2]

- RAN3 sees benefit if 5GC can provide the PDU session pair information to gain more flexibility in NG-RAN node, but RAN3 is not sure if the 5GC would be able to provide this information to NG-RAN node without adding too much complexity.

Hence before SA2 make replies, it is better to keep the stage 2 descriptions as it is.

While, For higher layer duplication solution 4, there is a FFS in baseline CR for TS 38.300.

- FFS whether redundant data forwarding tunnel is needed during mobility.

- Currently there is no common understanding whether to support the data forwarding. Also the lossless handover feature will ensure in itself that no packet can be lost during the forwarding. Hence at this stage this is not supported in this release, and this FFS can be removed. Also several editorial changes are needed.

- Remove the FFS in Baseline CR for TS 38.300.

2. Reference

- R3-194794 LS on Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity, RAN3

3. TP for BL CR for 38.300

## 16.1	URLLC

#### /********Skip the unchanged*********/

16.1.x.1 Redundant user plane paths based on dual connectivity

UE may initiate two redundant PDU Sessions over the 5G network. The 5GS sets up the user plane paths of the two redundant PDU sessions to be disjointed. When PDU session setup or modification is initiated, the RAN can configure dual connectivity in one NG-RAN node or two NG-RAN nodes for the two redundant PDU sessions to ensure the user plane paths disjoint according to redundancy information received from 5GC. The RAN shall ensure that the resources of the data radio bearers for the two redundant PDU sessions are isolated. If the RAN cannot satisfy the disjoint user plane requirements, the redundant PDU sessions may be kept or not kept according to the RAN local configuration. The redundancy information is transferred to the target NG-RAN node in case of handover.

16.1.x.2 Redundant data transmission via single UPF and single RAN node

Two NG-U tunnels are setup between single UPF and single NG-RAN node for redundant transmission of the QoS flows when PDU session setup or modification is initiated. The two NG-U tunnels are transferred via disjointed transport layer paths. The 5GC provides the indicator per QoS flow to the NG-RAN for the redundant transmission. For downlink, the NG-RAN node eliminates the duplicated packets per QoS flow. For uplink the NG-RAN node replicates the packet and transmits them via the two NG-U tunnels. The indicator per QoS flow for redundant transmission is transferred to the target NG-RAN node in case of handover.