3GPP TSG-RAN WG3 meeting #106	 R3-196992

Reno, NV, USA, 18– 22 November, 2019	revision of R3-195463

Agenda Item:	13.3.1

Source: 	Huawei

Title: 		Routing in IAB network

Document for:		Discussion and Decision

- Introduction

In RAN2 #107 meeting, the following agreements about routing in IAB network are achieved [1]:

For routing and bearer mapping of a packet retrieved from RLC layer, the IAB-node needs to be configurable with the following mappings:

BAP routing ID in BAP header  Egress link (routing table)

Ingress RLC channel Egress RLC channel (bearer mapping)

For the selection/addition of a BAP routing ID as well as routing and bearer mapping for a packet retrieved from upper layers, the IAB-node and IAB donor needs to be configurable with the following mappings:

(FFS) Upper layer information  BAP Routing ID to be added in BAP header

BAP routing ID in BAP header  Egress link

Upper layer information (FFS)   Egress RLC channel

In RAN3 #105 meeting, we have the following agreements about routing related configuration for DL [2]

On the DL, the IAB-donor DU is configurable with information that allows deriving the BAP routing ID from IP header information for F1-U, F1-C and non-F1 traffic.

On the DL, the IAB-donor is configurable with information that allows deriving the BAP address from the destination IP address.

In RAN3 #105bis meeting, the following agreement is achieved for DL routing [3].

configuration of downlink bearer mapping and routing should be performed by F1-AP

path id is derived from IP header and mapping provided by CU

In the DL, for BAP path id derivation on the donor DU: IP address, IPv6 flow level and/or DS/DSCP can be used; all of these fields are optional in F1AP message to configure routing

In this contribution, we are going to analyse more details about routing for wireless backhaul links in an IAB network, based on the above agreements.

- Discussion

Based on the discussion progress about IAB routing, there are still several issues to be addressed, and we will discuss them in the following part.

Issue 1: Which BAP routing ID to be added in BAP header for UL

For the UL transmission, the access IAB node should decide which BAP routing ID will be added to a given BAP SDU according to configurations received from the IAB-donor-CU. Each BAP routing ID includes a BAP address, and may also include a path ID. Therefore, the IAB-donor-CU should configure one or more BAP routing ID for upstream transmission to the access IAB node. To limit the occupied bits of BAP path ID, we assume that the path ID is unique to a given destination BAP address. Thus, the configuration of available upstream BAP routing ID for access IAB node can be split into two parts, one is to configure the destination BAP address, and another is to configure how to choose the BAP path ID with a given destination BAP address.

- BAP address configuration for upstream

The agreements about the definition of BAP address is “Each BAP address defines a unique destination (unique for IAB network of one Donor, either an IAB access node, or the IAB donor)”[4]. For upstream destination, it has not been clarified whether the BAP address should identify an IAB donor CU or the IAB donor DU.

We think using the BAP address to identify the IAB donor DU is reasonable since the BAP layer based routing will be performed across the wireless BH links, IP based routing will be used for the intra donor routing. Otherwise, if the BAP address is used to identify the IAB donor CU, each IAB donor DU will be responsible to forward packets with different BAP address carried in upstream packets, and multiple IAB donor DUs will share same BAP address only if these DUs connects to a same IAB donor CU-UP. In such case, it is meaningless for the IAB donor DU to check whether the BAP address of upstream packet is same as the one configured to it or not. To be honest, this is workable, but will defy the BAP layer receiving procedure of IAB node, and will cause split receiving operation for BAP layer specification. In addition, such way will prevent the IAB donor CU to do load balancing among different IAB donor DUs.

Observation 1: If assuming the BAP address in the BAP header is to identify the destination donor CU, BAP PDU with different BAP addresses may be transmitted via one donor DU.

Observation 2：For upstream transmission, the BAP address should be used to identify the destination IAB donor DU since the BAP layer routing is only used in the area of wireless BH links.

Therefore, in the following analysis of this paper, we assume that the BAP address in upstream packets is used to identify the IAB donor DU.

Proposal 1: RAN3 confirms that the BAP address carried in upstream packets is used to identify the destination IAB donor DU, rather than the IAB donor CU.

The following two options are proposed for the upstream BAP address configuration.

Option 1: Configure destination BAP address for upstream transmission per access IAB node. This is a simple way, where IAB donor CU only configures one destination BAP address for upstream transmission, with the assumption that the appointed IAB donor DU of the destination BAP address has the connection to all the CU-UPs the access IAB node connects to. In such a way, the IAB donor CU can do global load balancing for uplink transmission among multiple IAB donor DUs, through assigning different destination BAP address to different access IAB nodes. The granularity of such global load balancing is per IAB node.

Option 2: Allow the access IAB node to choose different destination BAP addresses in BAP layer according to upper layer information, e.g. the destination IP address.  In this way, the IAB donor CU can configure more than one UL destination BAP address for an access IAB node. To an access IAB node, each UL BAP address is assigned with one or more destination IP addresses of upstream packet. IAB donor CU will ensure that a destination IP address only be mapped to one UL BAP address in the configuration for an access IAB node. This option enables the IAB donor CU to do load balancing in a more flexible way, compared to option 1.

Proposal 2: For upstream, the BAP address to be added in BAP header is configured based on the destination IP address in the BAP SDU.

- Configuration of upstream BAP path ID for access IAB node

In addition to the BAP address configuration, the IAB donor CU may optionally configure one or multiple BAP path IDs towards each BAP address. If more than one path IDs are configured to a given BAP address, the access IAB node needs to decide which path ID to be added in an upstream packets. IAB donor CU may control such decisions in access IAB nodes in a centralized way, through e.g. assigning a priority level value to each allocated path ID, or configure a mapping from upper layer information to the path ID.

The priority level based solution is not as flexible as the upper layer info based solution for the access IAB node, since the access IAB node will always select the path ID with the highest priority unless this path suffers from BH RLF, while the upper layer info based solution enables the access IAB node to choose different paths for different packets towards a same destination BAP address. The upper layer information can be the identifier of UE DRB (i.e. identified by the Destination IP address + GTP-TEID). As to the CP signalling, the similar information can be used as in bearer mapping configuration, e.g. F1-AP message type.

If no BAP path ID is configured for a packet towards a BAP address, the default value will be added to BAP header by the access IAB node and the IAB node can select any next hop to transmit this packet. In some scenarios, if multiple BAP path IDs are configured towards same BAP address in the routing configuration information, and no preference is configured by the IAB-donor-CU, then the access IAB node can determine a preferred one by itself.

Proposal 3: For upstream, the path ID to be added in BAP header is configured based on destination IP address + GTP-TEID or F1-C message type in the BAP SDU, in case multiple paths towards the same destination are configured.

Issue 2: How to handle the RLF case

According to section 2.1, the access IAB node will add the BAP routing ID for UL packets and IAB-donor-DU will add BAP routing IDs for DL packets, and thus, it is possible that the access IAB node/IAB-donor-DU adds a preferred BAP path ID to a packet, but some BH links along this path may suffer from RLF. To ensure lossless transmission, upon RLF, the intermediate IAB node can forward the packets through other candidate paths. For example, as shown in Figure 1, the link between IAB node 3 and IAB node 1 is failed, but there are still some packets with BAP path ID 2 in IAB node 3, IAB node 3 can send those packets with BAP path ID 2 to the IAB donor via IAB node 2.

When the intermediate IAB node needs to choose another next hop link different from the one indicated by the entry of the BAP path ID and BAP address, the IAB node may look up the routing table and find another entry with same BAP address by implementation or take the priority level into consideration if the priority level of each path is configured.

Furthermore, it is also possible for the access IAB node or the IAB donor to select other paths if it knows that some link along the path with the highest priority level is suffering from RLF. For example, the access IAB node or the IAB donor DU may detect that the first hop of the path is failure, or receive RLF notification from intermediate IAB nodes. In such case, the selection of BAP path ID from other inferior candidate paths to same BAP address can also be based on implementation or according to the configured priority level of each path.

Figure 1. An example of BH link RLF

Proposal 4: If the path with highest priority (or indicated by path ID in BAP header) suffers BH RLF, BAP layer selects the other alternative path belonging to the same BAP address by implementation or based on the priority if configured.

Issue 3: How to handle the load balancing

In RAN2 #106 meeting, the agreements about routing show that “For load balancing still FFS what is decided locally and/or decided by the Donor.”  In fact, both the global load balancing and local balancing are easy to be achieved through implementation.

First, it is easy for the IAB-donor-CU to do global load balancing through configuring how to choose the path ID according to upper layer information or assigning preferred path ID with highest priority to the IAB-donor-DU and IAB nodes.

In addition, for the source node, i.e. the access IAB node or the IAB-donor-DU, if it is aware that the CU preferred path suffers from congestion, it can choose an alternative path with second priority level if configured or totally by the local decision of source node through implementation, and add the alternative BAP path ID to the BAP SDU.

Moreover, for intermediate IAB nodes, if the path indicated by the BAP path ID is congested or will overload, an intermediate IAB node can choose an alternative path towards a same BAP address also, either according to the configured priority level of each path or local decision by implementation.

Proposal 5: It is up to the IAB donor implementation whether the path selection for local load balancing is allowed, i.e. the donor decides whether to configure the preferred path in redundant paths case.

Issue 4: Whether the BAP path ID can be changed by intermediate IAB node

Based on the analysis in issue 2 and issue 3, it is obvious that re-routing may occurs in intermediate IAB node for some special reasons, e.g. BH link suffers RLF, load balancing. It means that an intermediate IAB node may select another candidate path which is different from the path indicated by the BAP path ID carried in BAP header of a received packet towards the same destination BAP address. A new issue for the re-routing case is that whether the intermediate IAB node changes BAP path ID carried in BAP header or not when it select another path. There are three options to handle such case, as listed in the following.

Option 1. Keep the original BAP path ID

In case of path switching in RLF, if the BAP path ID is not changed, all the following other intermediate IAB nodes in the new path may be confused about the BAP path ID in BAP header, since this BAP path ID is different from their already configured path IDs towards the corresponding BAP address. There are two possible cases for subsequent IAB nodes in the new forwarding path after re-routing. To be frank, both of them are abnormal cases.

Figure 2. An example of re-routing case

The first one is that some subsequent IAB nodes may be configured with BAP path ID which is conflicted with the unchanged BAP path ID carried in rerouted packets. For example, as shown in Figure 2, the upstream path IAB7IAB6IAB5IAB1IAB donor DU is identified by BAP path ID 1, and the UL routing configuration to IAB node 6 will including the routing entry of this BAP path ID 1. For IAB 4, we can see there are two UL paths to the IAB donor DU, the first one IAB4IAB2IAB donor DU is named as BAP path ID 1, while the second one IAB4IAB3IAB donor DU is named as BAP path ID 2. Such naming case is possible because the red dashed path and the greed dashed path are not overlapped except the same destination node. If some link in the red dashed path, e.g. the BH link between IAB 5 and IAB 6 suffers RLF, the IAB 6 can do rerouting for packets received from IAB 7 if the BAP path ID=1. Obviously, the IAB 6 will reroute these packets to IAB4 without change the BAP path ID. When IAB 4 receives these rerouted packets, IAB 4 will forward these packet to IAB2 after checking routing table, because IAB 4 is configured with a routing entry of BAP address=IAB donor DU and BAP path ID=1.  Such case is still workable for upstream data forwarding, but has disadvantage of causing overburden to the green dashed path, although the orange dashed path can also be used by IAB 4 to forward the re-routed packets received from IAB 6. Otherwise, more restrictions should be considered when allocating the BAP path ID to avoid such confliction when re-routing occurs.

The second abnormal case is that some subsequent IAB nodes is not able to select an entry for the unchanged BAP path ID carried in rerouted packets. To tackle with such abnormal case, some particular BAP behavior should be specified. For example, the BAP layer should support ignoring the BAP path ID if no routing entry is matched, and chooses any configured next hop node among the routing entries which have same destination BAP address as the BAP address carried in the packets.

Option 2a. The intermediate IAB node changes the BAP path ID to identify the actual path, which is selected when perform re-routing.

For example, the upstream paths from IAB 6 to the IAB donor DU are named as

BAP path ID=1: IAB6IAB5IAB1IAB donor DU

BAP path ID=2: IAB6IAB5IAB2IAB donor DU

BAP path ID=3: IAB6IAB4IAB2IAB donor DU

BAP path ID=4: IAB6IAB4IAB3IAB donor DU

When the BH link between IAB 6 and IAB 5 suffers RLF, the IAB 6 do rerouting for packets with BAP path ID=1, it can choose either the path identified by BAP path ID=3 or 4, and change the BAP path ID carried in the BAP header of the re-routed packets accordingly. This option will avoid the confliction case caused by option 1.

Option 2b. The intermediate IAB node changes the BAP path ID to the default value, e.g. all zero value.

Besides, another simple solution is enable the intermediate IAB node reset the BAP path ID to default value, e.g. all zero value, if re-routing is performed. And such option also need to specify the particular behaviour of BAP layer to handle with the default BAP path ID value, e.g. choose any configured entry towards the destination BAP address.

Based on the above analysis, option 1 may cause some confliction or require more restriction for allocating BAP path ID, thus we prefer option 2a and option 2b.

Proposal 6: If the path indicated by the path ID in the BAP header is not available (e.g. BH RLF), BAP resets the path ID in BAP header as one of the actual path to be re-routed, or resets the path ID to the default value, e.g. all zero values.

Issue 5: RLF handling of routing for the IAB node connecting to multiple donor DUs

In RAN2 106 meeting, the following agreements about routing in IAB network was achieved [4].

Each BAP address can have one or multiple entries in the routing table to enable local route selection. Multiple entries is for load balancing, re-routing at RLF. For load balancing still FFS what is decided locally and/or decided by the Donor.

To avoid data loss in IAB network, packets need to be re-routed in the RLF case. As analysed in section 2.2, if multiple entries are configured to a node (either the source node in wireless backhaul link, or intermediate IAB nodes) for a given BAP address, it is possible for this node to choose an alternative routing path if the path indicated by the carried BAP path ID suffers RLF.

As described in section 2.1, we assume the upstream destination BAP address identifies the IAB donor DU instead of IAB donor CU. Therefore, multiple uplink routing entries relates to a given BAP address will provide multiple paths to an IAB-donor-DU. To provide link redundancy in case of BH link RLF, it is possible for an IAB node to connect to multiple IAB-donor-DUs, and have one or more paths towards each IAB-donor-DU.

For example, as shown in Figure 3, IAB node 5 can connect to 3 different IAB-donor-DUs belong to a same IAB donor. There are two different paths between the IAB-donor-DU 3 and the IAB node 5, i.e. IAB-donor-DU 3↔ IAB node 1 ↔ IAB node 3↔ IAB node 5, and IAB-donor-DU 3↔ IAB node 2 ↔ IAB node 4↔ IAB node 5. Only one path exists between the IAB-donor-DU 1 and the IAB node 5, i.e. IAB-donor-DU 1↔ IAB node 1 ↔ IAB node 3↔ IAB node 5. Similarly, there is only one path between the IAB-donor-DU 2 and the IAB node 5, i.e. IAB-donor-DU 2↔ IAB node 2 ↔ IAB node 4↔ IAB node 5.

Figure 3. Scenario of BH link redundancy for IAB networks

Based on the example shown in Figure 3, if some link of one path between the IAB-donor-DU 3 and the IAB node 5 is RLF, another alternative UL path can be selected by the IAB node 5 for re-routing. If some link between IAB node 5 and the IAB-donor-DU 1/2 is failure, no alternative path can be used by IAB node 5 for UL re-routing. In fact, if the IAB-donor-CU is not CP-UP split, each IAB-donor-DU can forward the UL packet to the IAB-donor-CU. Even in the case that the IAB-donor-CU consists of one IAB donor-CU-CP and multiple IAB-donor-CU-UPs, the IAB-donor-CU has connections to each IAB-donor-DU, if we can make the following assumption 1, any IAB-donor-DU of an IAB donor can forward the UL UP packet to the right IAB-donor-CU-UP, and forward the UL CP packet to the IAB-donor-CU-CP.

Assumption 1: In an IAB donor, each IAB-donor-DU can connect to all the IAB-donor-CU-UPs.

With the above assumption, when some link of the preferred path to an IAB-donor-DU is failure, inter IAB-donor-DU re-routing may be used for robust UL transmission.

Proposal 7: RAN3 should confirm the assumption 1, i.e. in an IAB donor, each IAB-donor-DU can connect to all the IAB-donor-CU-UPs.

If the assumption 1 is confirmed, the upstream data can be transmitted via any IAB-donor-DU to the target CU-UP. If the path to the configured or preferred IAB node DU fails, IAB node can switch to another path to the other alternative IAB donor DU within the same donor CU. The alternative IAB donor DU has different BAP address with the original one in the BAP header. In the above path failure case, IAB node can select any next hop link to perform routing, which is not based on the BAP address in BAP header and routing table. This can be considered as the inter donor DU path switch or path redundancy.

Proposal 8: If assumption 1 is confirmed, IAB node can select any alternative IAB-donor-DU as the destination, if the path to the IAB-donor-DU identified by the BAP address in BAP header fails.

If the assumption 1 is not valid, RAN2 and RAN3 may need some standard efforts to support the inter IAB-donor-DU path switching or redundant, since which donor DUs are connected to the same CU-UP needs to be known by IAB node. Otherwise, if such inter-donor-DU path switch is not supported, those upstream packets has to be stored at intermediate IAB node until the path to the given destination IAB-donor-DU identified by the BAP address is recovered.

Issue 6: What kind of signalling is used for DL routing configuration

RAN3 agreed that the F1AP message will be used to configure the DL routing in RAN3 #105bis meeting. There are two kinds of F1AP messages in current specification, i.e. non-UE associated F1AP message, and UE associated F1AP message. In fact, both of the two kinds F1AP messages can be used to configure the routing table for IAB-DU or IAB donor DU.

For each DU, i.e. either the IAB-DU or the IAB-donor-DU, the granularity of BAP layer is node level, rather than per child node level. The DL routing configuration is also a node level configuration. Therefore, using non-UE associated signaling is more reasonable to carry the DL routing configuration for IAB node or IAB-donor DU.

Observation 3 : The granularity of BAP layer is node level, rather than per child node level, using non-UE associated signaling for routing configuration is more straightforward and reasonable.

Besides, the DL configuration of routing table is more essential for each parent DU (e.g. IAB  node DU, IAB donor DU, etc.) when compared to some child node specific configuration, since the traffics of all descendent IAB nodes and UEs connected to them will rely on the routing configuration. Therefore, such routing configuration should be forwarded with higher priority than a normal child node specific configuration. About the bearer mapping for F1-C signaling, RAN2 has agreed that “R2 assumes to support prioritization and separate BH RLC channel between non UE-associated signaling and UE-associated signaling, impact FFS.” [1]. And it seems that most companies agree that the non UE-associated signalling should be treated with higher priority. Consequently, using non-UE associated signalling to configure the routing table for parent DU is a better choice.

Observation 4 : DL configuration of routing table is more essential for each parent DU (e.g. IAB  node DU, IAB donor DU, etc.) when compared to some child node specific configuration.

Furthermore, considering that the IAB donor CU may need to update the routing configuration for IAB donor DU and/or IAB node, for the case that topology change or load balancing, using non-UE associated signaling will ensure the whole routing entries of the routing table being updated in a synchronized way. For example, as shown in Figure 4, a simple case of IAB topology update is given. IAB node 3 changes its parent node from IAB node 1 to IAB node 2.

Figure 4. IAB network topology update

Therefore, the IAB donor CU need to update the routing configuration in IAB donor DU, i.e. update the next hop node for IAB node 3 from IAB node 1 to IAB node 2. If UE associated F1AP signaling is used for routing configuration, two separate F1AP messages are necessary in such case. One is the IAB node 1 associated F1AP signaling, to remove the old routing entry regards to IAB node 3 in IAB donor DU. Another one is the IAB node 2 associated F1AP signaling, used to add the new routing entry regards to IAB node 2 in IAB donor DU. Apparently, the F1AP signaling corresponds to different child nodes will be transmitted separately, then there is a risk that the parent DU will suffer from routing entry confliction before it receives all the routing update signalings. For example, if the IAB node 2 associated F1AP signaling arrives at the IAB donor DU at first, and IAB node 1 associcated F1AP signaling arrives at the IAB donor DU latter, in such time interval, the IAB donor DU will have two entries to the IAB node 3. However, the old one is not available at all, but some packets towards IAB node 3 may still be forwarded by the IAB donor DU to IAB node 1.  Using non-UE associated signagling for routing configuration doses not have such problem, and will using less signaling when compared to UE-associated signaling based solution.

Observation 5 : Using UE associated F1AP signaling has a risk of causing routing entry confliction, and requires more number of signalings for routing configuration update scenarios.

Proposal 9: Using non-UE associated F1AP signalling to carry the DL routing configuration.

- Conclusion

This paper mainly discusses remain issues about routing for IAB networks, then we draw the following observation and proposals:

Observation 1: If assuming the BAP address in the BAP header is to identify the destination donor CU, BAP PDU with different BAP addresses may be transmitted via one donor DU.

Observation 2：For upstream transmission, the BAP address should be used to identify the destination IAB donor DU since the BAP layer routing is only used in the area of wireless BH links.

Observation 3 : The granularity of BAP layer is node level, rather than per child node level, using non-UE associated signaling for routing configuration is more straightforward and reasonable.

Observation 4 : DL configuration of routing table is more essential for each parent DU (e.g. IAB  node DU, IAB donor DU, etc.) when compared to some child node specific configuration.

Observation 5 : Using UE associated F1AP signaling has a risk of causing routing entry confliction, and requires more number of signalings for routing configuration update scenarios.

Proposal 1: RAN3 confirms that the BAP address carried in upstream packets is used to identify the destination IAB donor DU, rather than the IAB donor CU.

Proposal 2: For upstream, the BAP address to be added in BAP header is configured based on the destination IP address in the BAP SDU.

Proposal 3: For upstream, the path ID to be added in BAP header is configured based on destination IP address + GTP-TEID or F1-C message type in the BAP SDU, in case multiple paths towards the same destination are configured.

Proposal 4: If the path with highest priority (or indicated by path ID in BAP header) suffers BH RLF, BAP layer selects the other alternative path belonging to the same BAP address by implementation or based on the priority if configured.

Proposal 5: It is up to the IAB donor implementation whether the path selection for local load balancing is allowed, i.e. the donor decides whether to configure the preferred path in redundant paths case.

Proposal 6: If the path indicated by the path ID in the BAP header is not available (e.g. BH RLF), BAP resets the path ID in BAP header as one of the actual path to be re-routed, or resets the path ID to the default value, e.g. all zero values.

Proposal 7: RAN3 should confirm the assumption 1, i.e. in an IAB donor, each IAB-donor-DU can connect to all the IAB-donor-CU-UPs.

Proposal 8: If assumption 1 is confirmed, IAB node can select any alternative IAB-donor-DU as the destination, if the path to the IAB-donor-DU identified by the BAP address in BAP header fails.

Proposal 9: Using non-UE associated F1AP signalling to carry the DL routing configuration.

- Reference

- Report of the 3GPP RAN WG2 #107 meeting.

- Chairman notes of the 3GPP RAN WG3 #105 meeting.

- Chairman notes of the 3GPP RAN WG3 #105bis meeting.

- Report of the 3GPP RAN WG2 #106 meeting.