3GPP TSG-RAN WG3 Meeting #106	R3-197251

# Reno, USA,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0012 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of transmission and measurement functionality in NRPPa |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 1/11/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Functionality for support of generic transmission / measurement control is missing in NRPPa |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce procedures for initiating measurements (Measurement Request), initiate transmisisons (Transmission Request), request basic TRP list (TRP Initial Information Exchange), and request detailed TRP information (TRP Information Exchange) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for transmission / measurement functionality in rel-16 positioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TBD |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| TRP Initial Information Exchange | TRP INITIAL INFORMATION REQUEST | TRP INITIAL INFORMATION RESPONSE | TRP INITIAL INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement Request | MEASUREMENT INITIATION REQUEST | MEASUREMENT INITIATON RESPONSE | MEASUREMENT INITIATION FAILURE |
| Transmission Request | TRANSMISSION INITIATION REQUEST | TRANSMISSION INITIATION RESPONSE | TRANSMISSION INITIATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |

>>> NEXT CHANGE <<<

### 8.2.X	Measurement Request

#### 8.2.X.1	General

The purpose of the Measurement request procedure is to allow the LMF to request the NG-RAN node to initiate measurements.

#### 8.2.X.2	Successful Operation

Figure 8.2.X.2-1: Measurement Request procedure, successful operation

The LMF initiates the procedure by sending a MEASUREMENT INITIATION REQUEST message. The NG-RAN node responds with MEASUREMENT INITIATION RESPONSE message that contains the requested measurement information.

#### 8.2.X.3	Unsuccessful Operation

Figure 8.2.X.3-1: Measurement Request procedure, unsuccessful operation

If the NG-RAN node cannot perform the requested measurements, the NG-RAN node shall respond with a MEASUREMENT INITIATION FAILURE message.

### 8.2.Y	TRP Initial Information Exchange

#### 8.2.Y.1	General

The purpose of the TRP Initial Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide the list of TRPs that it hosts.

#### 8.2.Y.2	Successful Operation

Figure 8.2.Y.2-1: TRP Initial Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INITIAL INFORMATION REQUEST message. The NG-RAN node responds with TRP INITIAL INFORMATION RESPONSE message that contains the requested TRP information.

#### 8.2.Y.3	Unsuccessful Operation

Figure 8.2.Y.3-1: TRP Initial Information Exchange procedure, unsuccessful operation

If the NG-RAN node cannot provide the requested information, the NG-RAN node shall respond with a TRP INITIAL INFORMATION FAILURE message.

### 8.2.Z	TRP Information Exchange

#### 8.2.Z.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for some or all TRPs hosted by the NG-RAN node.

#### 8.2.Z.2	Successful Operation

Figure 8.2.Z.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with TRP INFORMATION RESPONSE message that contains the requested TRP information.

#### 8.2.Z.3	Unsuccessful Operation

Figure 8.2.Z.3-1: TRP Information Exchange procedure, unsuccessful operation

If the NG-RAN node cannot provide the requested information, the NG-RAN node shall respond with a TRP INFORMATION FAILURE message.

>>> NEXT CHANGE <<<

#### 9.1.1.A	MEASUREMENT INITIATION REQUEST

This message is sent by LMF to initiate measurements at a TRP.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | 9.2.A |  | YES | reject |
| TRP ID | M |  | 9.2.C |  | YES | reject |
| Measurement Configuration | M |  | 9.2.D |  | YES | reject |

#### 9.1.1.B	MEASUREMENT INITIATION RESPONSE

This message is sent by NG-RAN node to indicate that the requested measurement is successfully initiated at the TRP.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | 9.2.A |  | YES | reject |
| RAN Measurement ID | M |  | 9.2.B |  | YES | reject |
| Measurement Result | M |  | 9.2.E |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.C	MEASUREMENT INITIATION FAILURE

This message is sent by NG-RAN node to indicate that the requested measurement cannot be initiated.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | 9.2.A |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.D	TRP INITIAL INFORMATION REQUEST

This message is sent by LMF to request basic TRP configuration from a RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |

#### 9.1.1.E	TRP INITIAL INFORMATION RESPONSE

This message is sent by NG-RAN node to convey basic TRP configuration to a LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Configuration | M |  | 9.2.F |  | YES | reject |

#### 9.1.1.F	TRP INITIAL INFORMATION FAILURE

This message is sent by NG-RAN node to indicate that the requested TRP information cannot be provided to a LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |

#### 9.1.1.G	TRP INFORMATION REQUEST

This message is sent by LMF to request detailed TRP configuration from a RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  |  |  |  |  |  |
| TRP Information List |  | 0 .. 1 |  |  |  |  |
| >TRP Information Item |  | 1 .. <maxnoTRPs> |  |  |  |  |
| >> TRP ID | M |  | 9.2.C |  |  |  |
| TRP Information Type List |  |  |  |  |  |  |
| >TRP Information Item |  | 1 .. <maxnoInfoTypes> |  |  |  |  |
| >>TRP Parameters | M |  | ENUMERATED (pci, FFS, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65,535. |
| maxnoInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 63. |

#### 9.1.1.H	TRP INFORMATION RESPONSE

This message is sent by NG-RAN node to convey detailed TRP configuration to a LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Parameter Configuration List | M | 1 .. <maxnoTRPs> |  |  |  |  |
| >TRP Parameter Configuration Item |  |  |  |  |  |  |
| >>TRP ID | M |  | 9.2.C |  |  |  |
| >> TRP Configuration List | M |  | 9.2.G |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65,535. |

#### 9.1.1.I	TRP INFORMATION FAILURE

This message is sent by NG-RAN node to indicate that the requested TRP information cannot be provided to a LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |

>>> NEXT CHANGE <<<

### 9.2.A	LMF Measurement ID

The LMF Measurement ID IE is used to associate all the procedures related to the same measurement initiated by the LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| LMF Measurement ID | M |  | INTEGER(1..65535,…) |  |

### 9.2.B	RAN Measurement ID

The RAN Measurement ID IE is used to identify all the procedures related to the same measurement in the NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| RAN Measurement ID | M |  | INTEGER(1..65535,…) |  |

### 9.2.C	TRP ID

The TRP ID IE is used to identify a TRP uniquely within a NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Identifier | M |  | INTEGER(1..65535,…) | Identifies TRP within NG-RAN node |
| Associated Cell ID | O |  | NG-RAN CGI 9.2.6 | Cell ID associated with the TRP |

### 9.2.D	Measurement Configuration

The Measurement Configuration IE contains the measurement configuration to be conveyed to the TRP(s),

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Details are FFS |  |  |  |  |

### 9.2.E	Measurement Result

The Measurement Result IE conveys measurement results to the LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Details are FFS |  |  |  |  |

### 9.2.F	TRP Configuration

The TRP Configuration IE contains basic TRP configuration for TRPs within a NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP List |  | 1 |  |  |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  |
| >> TRP ID | M |  | 9.2.C |  |
| >> TRP Type | M |  | ENUMERATED (tx-only, rx-only, tx-and-rx,, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65,535. |

### 9.2.G	Parameter Configuration List

The Parameter Configuration List IE contains the requested TRP configuration for TRPs within a NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Parameter Configuration List |  | 1 |  |  |
| >Parameter Configuration Item |  | 1 .. <maxnoInfoTypes> |  |  |
| >> CHOICE Parameter Item | M |  |  |  |
| >>> PCI | M |  |  |  |
| >>> Param1 | M |  |  |  |
| >>> Param2 | M |  |  |  |
| >>> Param3 | M |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 63. |