3GPP TSG-RAN WG3 #106				R3-197312

Reno, NV, USA, 18-22 November 2019

Agenda Item:	14.3.2

Source:	Ericsson

Title: 	Removal of Connection Establishment Indication

Document for:	discussion and approval

# Introduction

In the last meeting, there was a debate on whether to remove or not the use of Connection Establishment Indication (CEI) for capability retrieval after the NGAP connection is established. The discussion seems to stem from a previous LTE debate. Two companies proposed to remove it, while two others proposed to define a new CEI procedure for NGAP.

After investigation, it seems this discussion originates from RAN3#100, where this was discussed widely for LTE case and some compromise was made so that backward-compatibility problems can be avoided:

- Support using CEI procedure for capability fetch after S1 UE association is established,

- Support using DL NAS Transfer for capability fetch.

Now in 5GS, we have the possibility to use the DL NAS Transport for capability fetching. Therefore, there is no need for the CEI to obtain and report UE capbilities.

Proposal: Remove the editor’s note on CEI.

# TP for BL CR#0173

START OF CHANGES

### 8.3.a	Connection Establishment Indication

#### 8.3.a.1	General

The purpose of the Connection Establishment Indication procedure is to enable the AMF to complete the establishment of the UE-associated logical NG-connection. The procedure uses UE-associated signalling. This procedure applies only if the NG-RAN node is an ng-eNB.

#### 8.3.a.2	Successful Operation

Figure 8.3.a.2-1: Connection Establishment Indication procedure. Successful operation.

The AMF initiates the procedure by sending a CONNECTION ESTABLISHMENT INDICATION message to the NG-RAN node.

If the UE-associated logical NG-connection is not established, the AMF shall allocate a unique AMF UE NGAP ID to be used for the UE and include it in the CONNECTION ESTABLISHMENT INDICATION message.

If the UE Radio Capability IE is included in the CONNECTION ESTABLISHMENT INDICATION message, the NG-RAN node shall store this information in the UE context, and use it as defined in TS 38.300 [8].

If the End Indication IE is included in the CONNECTION ESTABLISHMENT INDICATION message and set to "no further data", the NG-RAN node shall consider that there are no further NAS PDUs to be transmitted for this UE.

If the S-NSSAI IE is contained in the CONNECTION ESTABLISHMENT INDICATION message, the NG-RAN node shall store this information in the UE context, and use it as specified in TS 23.501 [9].

If the Allowed NSSAI IE is contained in the CONNECTION ESTABLISHMENT INDICATION message, the NG-RAN node shall store this information in the UE context, and use it as specified in TS 23.501 [9].

#### 8.3.a.3	Unsuccessful Operation

Not applicable.

#### 8.3.a.4	Abnormal Conditions

Not applicable.

END OF CHANGES