3GPP TSG-RAN3 Meeting #106	R3-197319

Reno, U.S., 18th November – 22nd November 2019

Agenda Item:	20.2.3

Source: 	Ericsson

Title:  	Clarifications on configuration exchange for NR V2X

Document for:	Discussion and approval

# Introduction

The last meeting the following agreements were made with respect to resource coordination exchange in NR V2X:

WA: the sidelink resource pool is configured in the gNB-DU by OAM

For mode 1 (all types) DU is responsible for SL resource allocation

DU is responsible for LTE V2X sidelink resource allocation Mode 3 (SPS scheduling)

V2X frequency and bandwidth information exchanged between RAN nodes shall be supported

A TP in R3-196197 was agreed for the F1AP BL CR, with an editor’s note on whether the added IEs on LTE and NR sidelink info list and their content are needed or not.

In this paper we would like to highlight some important aspects, with respect to the resource coordination for NR V2X and the agreements made above.

# Discussion

It is important to emphasize that since it has been agreed that the DU is responsible for the SL resource allocation and that the resource pool for LTE and NR will be configured in the DU by OAM, then the DU can already know what are the LTE and NR carries/frequencies configured for the sidelink resource pools by itself.

Observation 1: Taking into account the last meeting’s agreements, the gNB-DU can already know what are the LTE and NR carries/frequencies configured for the V2X resource pools

Therefore, there is no need for configuration exchange between the CU and the DU, since the DU can be capable of deducing of the RAT carriers and frequencies from its own configuration. We propose then to remove the added IEs in F1AP BL CR.

Proposal 1 : Remove LTE V2X Sidelink Info List (FFS) NR Sidelink Info List (FFS) from the F1AP BL CR

Regarding the X2 and Xn TPs, in R3-196195 and R3-196196, respectively, we would like to note that the agreed TPs are breaking one of RAN3’s common good principles to not exchange capability information at interface setup messages, which is considered as a bad practice. Therefore, companies should discuss whether there is a cleaner way to support V2X frequency and bandwidth information exchanged between RAN nodes. For instance, a container could be introduced instead to exchange the capability information.

Proposal 2 : RAN3 to discuss whether a container could be introduced to exchange V2X frequency and bandwidth information between RAN nodes, instead of explicit signaling at interface setup messages

Furthermore, for supporting SL resource coordination exchange between NG-RAN nodes, a question that comes often in RAN3 is whether to leverage UE or Cell associated signaling? Such question seems to forget the long discussion that happened in LTE V2X and the outcome of it.

For recollection, Cell-associated signaling was discussed at length during LTE V2X and in the end it was considered not beneficial and even harmful for the network. In fact, V2X usage is local: if no additional info is known about, e.g., which UEs are using which sidelink resources at which position, signaling this over Xn will give a “pessimistic” view to the receiving node. The receiving node might use this info to, e.g., schedule such resources less aggressively when in reality there’s little or no interference caused, thereby leading to a suboptimal radio resource allocation at the receiving node and in a waste of network signaling resources. In addition, vehicles will move around a lot, hence the “snapshot” exchanged exchange over Xn will become very soon obsolete.

At the end, this is still considered a useless functionality also for NR V2X, which will cause a sub-optimization of network resources.

Proposal 3 : RAN3 to agree not exchanging the V2X resource pool over X2/Xn since it is not beneficial for the network resources

Regarding UE-associated signaling with UE-dedicated resource allocation, they obviously cannot be configured by OAM. Such UE resources information, which can be achieved either via mode-1 or mode-2 configuration, contain knowledge of the amount of UEs which are interested in SL operations, neighbour cell deployment and served V2X UE positions. It is therefore desirable that such resources are not interfered by other concurrent transmission in neighboring cells in order to avoid any disruption of V2X services or ending up with SL configuration discontinuity.

Observation 2 : It is needed to support UE-specific SL resource coordination for NR-V2X operation between NG-RAN nodes.

Therefore, the container that was proposed in Proposal 2 can also be used to carry UE-dedicated info which could optimize not only resource coordination, but also mobility and cell (re)selection mechanisms.

Observation 3: UE-specific info can be exchanged within a container between neighbour RAN nodes to facilitate resource coordination as well as for mobility and cell reselection purposes

# Conclusion

Based on the above discussion, we made the following observations:

Observation 1: Taking into account the last meeting’s agreements, the gNB-DU can already know what are the LTE and NR carries/frequencies configured for the V2X resource pools

Observation 2 : It is needed to support UE-specific SL resource coordination for NR-V2X operation between NG-RAN nodes

Observation 3: UE-specific info can be exchanged within a container between neighbour RAN nodes to facilitate resource coordination as well as for mobility and cell reselection purposes

Thus, we propose the following:

Proposal 1 : Remove LTE V2X Sidelink Info List (FFS) NR Sidelink Info List (FFS) from the F1AP BL CR

Proposal 2 : RAN3 to discuss whether a container could be introduced to exchange V2X frequency and bandwidth information between RAN nodes, instead of explicit signaling at interface setup messages, as well as UE dedicated info

Proposal 3 : RAN3 to agree not exchanging the V2X resource pool over X2/Xn since it is not beneficial for the network resources

We provide the TP below to remove the added changes

# TP for F1AP BL CR

------------------------------------------------------------CHANGE START------------------------------------------------------------

Editor’s note: whether the added IEs and their content are needed or not is FFS

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per TA. | YES | ignore |
| CHOICE NR-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>> NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>> Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. | - |  |
| RANAC | O |  | RAN Area Code 9.3.1.57 |  | YES | ignore |
| Extended Served PLMNs List |  | 0..1 |  | This is included if more than 6 Served PLMNs is to be signalled. | YES | ignore |
| >Extended Served PLMNs Item |  | 1 ..<maxnoofExtendedBPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per TA. | - |  |
| Cell Direction | O |  | 9.3.1.78 |  | YES | ignore |
| Cell Type | O |  | 9.3.1.87 |  | YES | ignore |
| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR-1> |  | This IE corresponds to the PLMN-IdentityInfoList IE in SIB1 as specified in TS 38.331 [8]. The PLMN Identities and associated information contained in this IE are provided in the same order as broadcast in SIB1. | YES | ignore |
| >PLMN Identity List | M |  | Available PLMN List 9.3.1.65 |  | - |  |
| >Extended PLMN Identity List | O |  | Extended Available PLMN List 9.3.1.76 |  | - |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | - |  |
| >NR Cell Identity | M |  | BIT STRING (36) |  | - |  |
| >RANAC | O |  | RAN Area Code 9.3.1.57 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR-1 | Maximum no. of PLMN Ids.broadcast in an NR cell minus 1. Value is 11. |