3GPP TSG-RAN3 Meeting #106	R3-197321

Reno, U.S., 18th November – 22nd November 2019

Agenda Item:	31.2.2

Source: 	Ericsson

Title:  	Reporting of NR cells as Inter-RAT measurements for E-CID in LPPa

Document for:	Discussion and approval

# Introduction

In RAN3#105bis, the following agreement was made:

RAN3 intends to support reporting of NR cells as inter-RAT measurements for E-CID in LPPa in TEI16

This paper provides clarifications on the proposed CR in [1]

# Discussion

In last meeting, a company wanted that the coding of EN-DC NR cells measurement report for LPPa is done on purpose. However, some doubts were mentioned on the necessity on the differentiation of NR cell type, since current inter-RAT measurement IE in LPPa makes no assumption about the RAN type: it just “cares” about the RAT, no matter whether the NR is from SA, or EN-DC, etc. The E-SMLC will anyway have a database of cell identifiers and NR ARFCNs, etc.

Proposal 1: Differentiation is unnecessary on whether the NR comes from SA or NSA; this is just assistance information to improve E-CID accuracy

Proposal 1bis: Add NR cells codepoint to the inter-RAT structure inside the Inter-RAT Measurement Quantities Item IE for E-CID. No need for a dedicated IE

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by E-SMLC to initiate E-CID measurements.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED(OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…) |  | - | - |
| Inter-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >Inter-RAT Measurement Quantities Item | M |  | ENUMERATED(GERAN, UTRAN,…, NR) |  |  |  |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | - |  |

Observation 1 : the NR codepoint refers to the RAT type. This could be clarified in the semantics, if needed

Regarding the max number of NR Results in the Inter-RAT Measurements result, a company highlighted the fact that 8 might be too small, 16 might be more reasonable; however, our understanding is that the limitation comes from the LTE RRC specification that has been frozen by RAN2. Further changes would thus have to be discussed in the next release.

Proposal 2 : Further changes on Maximum no. of NR cells that can be reported in E-CID may be discussed in Release-17

# Conclusion

Based on the discussion above, we propose to agree on the CR in [1]:

Proposal 1: Differentiation is unnecessary on whether the NR comes from SA or NSA; this is just assistance information to improve E-CID accuracy, if needed

Proposal 1bis: Add NR cells codepoint to the inter-RAT structure inside the Inter-RAT Measurement Quantities Item IE for E-CID. No need for a dedicated IE

Observation 1 : the NR codepoint refers to the RAT type. This could be clarified in the semantics

Proposal 2 : Further changes on Maximum no. of NR cells that can be reported in E-CID may be discussed in Release-17

# Reference

[1] R3-197294, “Inter-RAT Measurement of NR Cells for E-CID”, RAN3#106