3GPP TSG-RAN WG3 #106	R3-197401

Reno, Nevada , US, 18th November – 22nd November 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Information on Load distribution over Xn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  |  | Date: |  | 2019-11-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A description of load information that takes into account the spatial distribution of load has been deemed as necessary in RAN3. Support for signalling over the Xn interface of the load information per SSB area is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To describe how load information that take into account spatial distribution of load are exchanged over the Xn interface |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to represent load in a spatial way and consequently it is not possible to make the load balancing function work correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.A, 8.4.B, 9.3.1.A, 9.3.1.B, 9.3.1.D, 9.2.xx(new), 9.2.yy(new), 9.2.zz(new), 9.2.a(new), 9.2.b(new), 9.2.c (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS… CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============ Start of 1st change ==============

### 8.4.A	Resource Status Reporting Initiation

Editor’s note: The content of this section is FFS

#### 8.4.A.1	General

This procedure is used by an NG-RAN node1 to request the reporting of load measurements to another NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.4.A.2	Successful Operation

Figure 8.4.A.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from NG-RAN node1 to NG-RAN node2 to start a measurement, stop a measurement, add cells to report for a measurement or remove certain cells from a measurement.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects NG-RAN node2 shall perform measurements on. For each cell, the NG-RAN node2 shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the NG-RAN node2 is a gNB and if the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the required SSB Area Radio Resource Status List IE;

-	the NG TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the NG-RAN node2 is a gNB and if the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB, the Composite Available Capacity Group IE for such cell shall include the required SSB Area Capacity Value IE, providing the SSB area capacity with respect to the Cell Capacity Class Value. [The following text in this paragraph is FFS] If the NG-RAN node2 is a gNB and if the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, the Composite Available Capacity Group IE for such cell shall include the Slice Capacity Value, providing the slice capacity with respect to the Cell Capacity Class Value.

#### 8.4.A.3	Unsuccessful Operation

Figure 8.4.A.3-1: Resource Status Reporting Initiation, unsuccessful operation

If none of the requested measurements can be initiated, NG-RAN node2 shall send a RESOURCE STATUS FAILURE message.

#### 8.4.A.4	Abnormal Conditions

FFS

### 8.4.B	Resource Status Reporting

Editor’s note: The content of this section is FFS

#### 8.4.B.1	General

This procedure is initiated by NG-RAN node2 to report the result of measurements admitted by NG-RAN node2 following a successful Resource Status Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.4.B.2	Successful Operation

Figure 8.4.B.2-1: Resource Status Reporting, successful operation

The NG-RAN node2 shall report the results of the admitted measurements in RESOURCE STATUS UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Resource Status Reporting Initiation procedure.

#### 8.4.B.3	Unsuccessful Operation

Not applicable.

#### 8.4.B.4	Abnormal Conditions

FFS

============ End of 1st change ==============

============ Here is the 2nd change ==============

#### 9.1.3.A	RESOURCE STATUS REQUEST

Editor’s note: The content of this section is FFS

This message is sent by an NG-RAN node1 to neighbouring NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  |  |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 |  |  |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporPartialStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 |  |  |
| Registration Request | M |  | ENUMERATED(start, stop, partial stop, add, …) | Type of request for which the resource status is required. |  |  |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW Capacity Ind Periodic [ Fifth Bit = Number of Active UEs, Sixth Bit = Number of RRC connections. Other bits shall be ignored by the NG-RAN node2. |  |  |
| Cell To Report |  | 1 |  | Cell ID list to which the request applies. |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | YES | Ignore |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | EACH | Ignore |
| >>Slice To Report List [FFS] |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>>>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Capacity Ind Periodic, Composite Available Capacity Periodic. |  |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems [FFS] | Maximum no. of signalled slice support items. Value is 1024. [FFS] |

#### 9.1.3.B	RESOURCE STATUS RESPONSE

Editor’s note: The content of this section is FFS

This message is sent by the NG-RAN node2 to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.C	RESOURCE STATUS FAILURE

Editor’s note: The content of this section is FFS

This message is sent by the NG-RAN node2 to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 | Ignored by the receiver when the Complete Failure Cause Information IE is included. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.D	RESOURCE STATUS UPDATE

Editor’s note: The content of this section is FFS

This message is sent by NG-RAN node2 to neighbouring NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| NG TNL capacity Indicator | O |  | 9.2.xx |  |  |  |
| Hardware Load Indicator [FFS] | O |  | 9.2.ww |  |  |  |
| Cell Measurement Result |  | 1 |  |  |  |  |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>Radio Resource Status [FFS] | O |  | 9.2.yy |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.zz |  |  |  |
| >>Slice Available Capacity [FFS] | O |  | 9.2.jj |  |  |  |
| >>Number of Active UEs [FFS] | O |  | INTEGER (1..65536,...) |  |  |  |
| >>Number RRC Connections [FFS] | O |  | INTEGER (1..65536,...) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

============ The 2nd change ends here ==============

============ We have one more change ==============

### 9.2.xx	NG TNL Capacity Indicator

The NG TNL Capacity Indicator IE indicates the offered and available capacity of the NG Transport Network.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL NGTNL Offered Capacity | M |  | INTEGER (1.. 16777216,...) | Maximum capacity offered by the transport portion of the NG-gNB in kbps |
| DL NGTNL Available Capacity | M |  | INTEGER (1.. 16777216,...) | Available capacity over the transport portion of the NG-gNB in kbps |
| UL NGTNL Offered Capacity | M |  | INTEGER (1.. 16777216,...) | Maximum capacity offered by the transport portion of the NG-gNB in kbps |
| UL NGTNL Available Capacity | M |  | INTEGER (1.. 16777216,...) | Available capacity over the transport portion of the NG-gNB in kbps |

### 9.2.yy	Radio Resource Status

[The information is FFS]

The Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage |
| UL GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage |
| DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage |
| UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage |
| DL Total PRB usage | M |  | INTEGER (0..100) | Per cell DL Total PRB usage |
| UL Total PRB usage | M |  | INTEGER (0..100) | Per cell UL Total PRB usage |
| DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| SSB Area Radio Resource Status List |  | 0..<maxnoofSSBAreas> |  |  |
| > SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage |
| > SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage |
| > SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage |
| > SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage |
| > SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage |
| > SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

### 9.2.zz	Composite Available Capacity Group

The Composite Available Capacity Group IE indicates the overall available resource level per cell and per SSB area in the cell in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | Composite Available Capacity 9.2.a | For the Downlink | - |  |
| Composite Available Capacity Uplink | M |  | Composite Available Capacity 9.2.a | For the Uplink | - |  |

### 9.2.a Composite Available Capacity

The Composite Available Capacity IE indicates the overall available resource level in the cell in either Downlink or Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Capacity Class Value | O |  | 9.2.b |  | - |  |
| Capacity Value | M |  | 9.2.c | ‘0’ indicates no resource is available, Measured on a linear scale. | - |  |

### 9.2.b Cell Capacity Class Value

The Cell Capacity Class Value IE indicates the value that classifies the cell capacity with regards to the other cells. The Cell Capacity Class Value IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity Class Value | M |  | INTEGER (1..100,...) | Value 1 shall indicate the minimum cell capacity, and 100 shall indicate the maximum cell capacity. There should be a linear relation between cell capacity and Cell Capacity Class Value. |  |  |

### 9.2.c Capacity Value

The Capacity Value IE indicates the amount of resources per cell and per SSB area that are available relative to the total NG-RAN resources. The capacity value should be measured and reported so that the minimum NG-RAN resource usage of existing services is reserved according to implementation. The Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity with respect to the whole cell. Capacity Value should be measured on a linear scale. |  |  |
| SSB Area Capacity Value |  | 0..<maxnoofSSBAreas> |  |  |  |  |
| > SSB Area Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . SSB Area Capacity Value should be measured on a linear scale. | - |  |

### 9.2.jj Slice Available Capacity

[The information is FFS]

The Slice Available Capacity IE indicates the amount of resources per network slice that are available relative to the total NG-RAN resources. The Slice Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Slice Available Capacity |  | 1..< maxnoofSliceItems > |  |  |  |  |
| > Slice Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . Slice Capacity Value should be measured on a linear scale. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems [FFS] | Maximum no. of signalled slice support items. Value is 1024. [FFS] |

// **************************** That is all for today ****************************