3GPP TSG-RAN WG3 Meeting #106	R3-197427

Reno (NV), USA, 18 – 22 November, 2019

Agenda item:	10.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP for SON BL CR for TS 36.423] TNL Load reporting updates

Document for:	Text Proposal

# Introduction

This TP includes updates to the SON BL CR for TS 36.423 as proposed in [1] (proposal 7) and [2] (proposals 1 and 2).

# References

[1]	R3-196840, [TP for SON BL CR for TS 38.423] Discussion and updates of slice load, beam load and TNL load reporting, Nokia, Nokia Shanghai Bell

[2]	R3-196844, [TP for SON BL CR for TS 38.463] Discussion and updates of TNL and HW load reporting, Nokia, Nokia Shanghai Bell

# TP for BL CR for TS 36.423

<<< First change >>>

#### 9.1.2.x4	EN-DC RESOURCE STATUS UPDATE

Editor’s note: The content of this section is FFS

This message is sent by en-gNB to neighbouring MeNB to report the results of the requested measurements.

Direction: en-gNB  eNB.

[Reporting per cell is FFS for TNL Load Indicator, Hardware Load Indicator]

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| MeNB Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by MeNB | YES | reject |
| en-gNB Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by en-gNB | YES | reject |
| Cell Measurement Result |  | 1 |  |  |  |  |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | 9.2.111 |  |  |  |
| >>Radio Resource Status [FFS] | O |  | 9.2.yy |  |  |  |
| >> S1 TNL Available Capacity Indicator | O |  | TNL Available Capacity Indicator 9.2.xx |  |  |  |
| >> F1 TNL Available Capacity Indicator | O |  | TNL Available Capacity Indicator 9.2.xx |  |  |  |
| >>Composite Available Capacity Group[FFS] | O |  | 9.2.zz |  |  |  |
| >>Hardware Load Indicator [FFS] | O |  | 9.2.ww |  |  |  |
| >> Number of Active UEs [FFS], | O |  | INTEGER (1..65536,...) |  |  |  |

<<< next change >>>

### 9.2.xx	TNL Available Capacity Indicator

The TNL Available Capacity Indicator IE indicates the status of the Transport Network Layer available capacity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL TNL Relative Available Capacity | M |  | INTEGER(0..100, …) | Indicates the amount of downlink transport network level available capacity. Value 100 corresponds to the nominal capacity. |
| DL TNL Absolute Available Capacity | M |  | INTEGER (1.. 16777216,...) Max value is FFS | Available DL TNL capacity in Mbps. |
| UL TNL Relative Available Capacity | M |  | INTEGER(0..100, …) | Indicates the amount of uplink transport network level available capacity. Value 100 corresponds to the nominal capacity. |
| UL TNL Available Capacity | M |  | INTEGER (1.. 16777216,...) Max value is FFS | Available UL TNL capacity in Mbps. |

<<< End of changes >>>