3GPP TSG-RAN3 Meeting #106	R3-196712

Reno, Nevada, US, 18-22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0289 | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of E-RAB ID in modification procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the existing PDU session modification procedure, the E-RAB ID is optional but without any procedural texts. When the E-RAB ID is included, the NG-RAN shall replace the stored value by the received one. But it is not fully clear what will happen when the E-RAB ID is not included in the PDU Session Resource Modify Request Transfer IE. There are two different understandings as follows: - Understanding 1: keep the existing E-RAB ID. - Understanding 2: remove the E-RAB ID. It should be clarified which understanding is correct NG-RAN node behaviour. As specified in TS 23.502, when the AMF decides to revoke some EBI(s), e.g. when the AMF receives a new EBI allocation request but there is no EBI available, the AMF may decide to revoke EBI(s) for another PDU Session, the AMF initiates a PDU Session Modification as described in clauses 4.3.3.2 or 4.3.3.3 and includes EBI list to be revoked in the Nsmf_PDUSession_UpdateSMContext Request. The SMF releases the indicated EBI(s) for the PDU Session. Hence, when the E-RAB ID is not included in the PDU Session Resource Modify Request Transfer IE, it should be interpreted that the NG-RAN node should remove the E-RAB ID for the PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedural texts for the above understanding 2. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the PDU session resource modify function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear about the NG-RAN’s behaviour when the E-RAB ID is not included in the PDU Session Resource Modify Request Transfer IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.2.3	PDU Session Resource Modify

<Unchanged Text Omitted>

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

<Unchanged Text Omitted>

For each PDU session included in the PDU Session Resource Modify Request List IE:

-	For each QoS flow included in the QoS Flow Add or Modify Request List IE, based on the QoS Flow Level QoS Parameters IE, the NG-RAN node may establish, modify or release the DRB configuration and may change allocation of resources on NG or Uu accordingly. The NG-RAN node shall associate each QoS flow accepted to setup or modify with a DRB of the PDU session. The associated DRB for the QoS flow accepted to modify may not change. If the E-RAB ID IE is included in the QoS Flow Add or Modify Request List IE, the NG-RAN node shall replace the previously stored E-RAB ID for the concerned QoS flow and use it as sepcfied in TS 38.300 [8]. If the E-RAB ID IE is not received, the NG-RAN node shall consider that no E-RAB ID is mapped for the concerned QoS flow.

<Unchanged Text Omitted>