3GPP TSG-RAN3 Meeting #106	R3-196929

Reno, Nevada, US, 18-22 November 2019	Revision of R3-195802

Title: 	(TP for NR_IIOT BL CR for TS 38.473): TSC Assistance Information

Source: 	Huawei

Agenda item:	17.3.2

Document for:	Discussion and Decision

1. TP for BL CR

8.3.1	UE Context Setup

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, and DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

<Unchanged Text Omitted>

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [21].

If the TSC Traffic Characteristics IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received TSC Traffic Characteristics and use it for scheduling process for the concerned UE as specified in TS 23.501 [21].

<Unchanged Text Omitted>

8.3.4	UE Context Modification (gNB-CU initiated)

8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU

<Unchanged Text Omitted>

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace the received UL PDU Session Aggregate Maximum Bit Rate and use it as specified in TS 23.501 [21].

If the TSC Traffic Characteristics IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace the TSC Traffic Characteristics and use it as specified in TS 23.501 [21].

<Unchanged Text Omitted>

9.3.1.45	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow or to a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.49 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.47 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.48 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.46 | This IE shall be present for GBR QoS Flows only. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Details in TS 23.501 [21]. This IE applies to non-GBR flows only and shall be ignored otherwise. | - |  |
| PDU Session ID | O |  | INTEGER (0 ..255) | As specified in TS 23.501 [21]. | YES | ignore |
| UL PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.22 | The PDU session Aggregate Maximum Bit Rate Uplink which is associated with the involved PDU session. | YES | ignore |
| TSC Traffic Characteristics | O |  | 9.3.1.x | This IE indicates the TSC traffic characteristics as described in TS 23.501 [12]. This IE may be present in case of GBR QoS flows and shall be ignored otherwise. | YES | ignore |

<Unchanged Text Omitted>

9.3.1.47	Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink.

| IE/Group Name |  |  | Presence |  | Range | IE type and reference |  |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QoS Priority Level |  |  | M |  |  | INTEGER (1..127) |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| Packet Delay Budget |  |  | M |  |  | 9.3.1.51 |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| Packet Error Rate |  |  | M |  |  | 9.3.1.52 |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| 5QI |  |  | O |  |  | INTEGER (0..255,...) |  |  | This IE contains the dynamically assigned 5QI as specified in TS 23.501 [21]. |  | - |  |  |
| Delay Critical |  |  | C-ifGBRflow |  |  | ENUMERATED (delay critical, non-delay critical) |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| Averaging Window |  |  | C-ifGBRflow |  |  | 9.3.1.53 |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| Maximum Data Burst Volume |  |  | O |  |  | 9.3.1.54 |  |  | For details see TS 23.501 [21]. This IE shall be included if the Delay Critical IE is set to “delay critical” and shall be ignored otherwise. |  | - |  |  |
| CN Packet Delay Budget | O |  | 9.3.1.z | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

Next Change

9.3.1.49	Non Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name |  | Presence |  |  | Range | IE type and reference |  |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5QI |  | M |  |  |  | INTEGER (0..255,...) |  |  | This IE contains the standardized or pre-configured 5QI as specified in TS 23.501 [21] |  | - |  |  |
| Priority Level |  | O |  |  |  | INTEGER (1..127) |  |  | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |  | - |  |  |
| Averaging Window |  | O |  |  |  | 9.3.1.53 |  |  | This IE applies to GBR QoS Flows only. For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |  | - |  |  |
| Maximum Data Burst Volume |  | O |  |  |  | 9.3.1.54 |  |  | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. If the 5QI refers to a non-delay critical QoS flow the IE shall be ignored. |  | - |  |  |
| CN Packet Delay Budget | O |  | 9.3.1.z | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

Next Change

9.3.1.x	TSC Traffic Characteristics

This IE provides the traffic characteristics of TSC QoS flows.

Editor’s Note:	It is FFS whether to include this IE within the QoS Flow Level QoS Parameters IE, or at the same level as the QoS Flow Level QoS Parameters IE. The IE may be present in case of GBR QoS flows and is ignored otherwise.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Assistance Information Downlink | O |  | TSC Assistance Information 9.3.1.y |  |
| TSC Assistance Information Uplink | O |  | TSC Assistance Information 9.3.1.y |  |

9.3.1.y	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | INTEGER (0..4095000) | Periodicity, in unit of 1us as specified in TS 23.501 [9]. |
| Burst Arrival Time | O |  | OCTET STRING (SIZE(4)) | Burst Arrival Time as specified in TS 23.501 [9]. |

#### 9.3.1.z	CN Packet Delay Budget

This IE indicates the CN Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Packet Delay Budget Downlink | O |  | INTEGER (0..5115) | Core Network Packet Delay Budget Downlink, in unit of 0.1ms is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. |
| CN Packet Delay Budget Uplink | O |  | INTEGER (0..5115) | Core Network Packet Delay Budget Uplink, in unit of 0.1ms is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. |

<Unchanged Text Omitted>