3GPP TSG-RAN3 Meeting #106	R3-196930

Reno, Nevada, US, 18-22 November 2019 	Revision of R3-195803

Title: 	(TP for NR_IIOT BL CR for TS 38.463): TSC Assistance Information

Source: 	Huawei

Agenda item:	17.3.2

Document for:	Discussion and Decision

- Annex-TP for BL CR

9.3.1.26	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - | - |
| >Non-dynamic 5QI |  |  |  |  | - | - |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.27 |  | - | - |
| >Dynamic 5QI |  |  |  |  | - | - |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - | - |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.29 |  | - | - |
| GBR QoS Flow Information | O |  | 9.3.1.30 | This IE shall be present for GBR QoS Flows only. | - | - |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [20]. This IE applies to Non-GBR flows only and shall be ignored otherwise. | - | - |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU Session. | - | - |
| Paging Priority Indicator (PPI) | O |  | 9.3.1.55 |  | - | - |
| RDI | O |  | ENUMERATED (enabled, …) | Indicates whether Reflective QoS flow to DRB mapping should be applied. | - | - |
| Redundant QoS Flow Information | O |  | ENUMERATED (true, …) | This IE indicates that this QoS flow is requested for the redundant transmission. | YES | Ignore |
| TSC Traffic Characteristics | O |  | 9.3.1.x | This IE indicates the TSC traffic characteristics as described in TS 23.501 [20]. This IE may be present in case of GBR QoS flows and shall be ignored otherwise. | YES | ignore |

Editor’s note: FFS whether to put the Redundant QoS Flow Information at the same level as QoS Flow Level QoS Parameters.

Next of Text Proposal for TS 38.463

#### 9.3.1.27	Non Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255, …) | This IE contains the standardized or pre-configured 5QI as specified in TS 23.501 [20]. |
| Priority Level | O |  | 9.3.1.51 | For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. |
| Averaging Window | O |  | 9.3.1.49 | This IE applies to GBR QoS Flows only. For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. If the 5QI refers to a non-delay critical QoS flow the IE shall be ignored. |
| CN Packet Delay Budget | O |  | 9.3.1.z | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. |

#### 9.3.1.28	Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | 9.3.1.51 | For details see TS 23.501 [20]. |
| Packet Delay Budget | M |  | 9.3.1.47 | For details see TS 23.501 [20]. |
| Packet Error Rate | M |  | 9.3.1.48 | For details see TS 23.501 [20]. |
| 5QI | O |  | INTEGER (0..255,…) | This IE contains the dynamically assigned 5QI as specified in TS 23.501 [20]. |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (delay critical, non-delay critical) | For details see TS 23.501 [20]. |
| Averaging Window | C-ifGBRflow |  | 9.3.1.49 | For details see TS 23.501 [20]. |
| Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [20]. This IE shall be included if the Delay Critical IE is set to “delay critical” and shall be ignored otherwise. |
| CN Packet Delay Budget | O |  | 9.3.1.z | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

9.3.1.26	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS Flow.

| IE/Group Name |  |  | Presence |  | Range |  | IE type and reference | Semantics description |  |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics |  |  | M |  |  |  |  |  |  |  | - |  |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor |  |  | M |  |  |  | 9.3.1.27 |  |  |  | - |  |  |
| >Dynamic 5QI |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor |  |  | M |  |  |  | 9.3.1.28 |  |  |  | - |  |  |
| NG-RAN Allocation and Retention Priority |  |  | M |  |  |  | 9.3.1.29 |  |  |  | - |  |  |
| GBR QoS Flow Information |  |  | O |  |  |  | 9.3.1.30 | This IE shall be present for GBR QoS Flows only. |  |  | - |  |  |
| Reflective QoS Attribute |  |  | O |  |  |  | ENUMERATED (subject to, …) | Details in TS 23.501 [20]. This IE applies to Non-GBR flows only and shall be ignored otherwise. |  |  | - |  |  |
| Additional QoS Flow Information |  |  | O |  |  |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU Session. |  |  | - |  |  |
| Paging Priority Indicator (PPI) |  |  | O |  |  |  | 9.3.1.55 |  |  |  | - |  |  |
| RDI |  |  | O |  |  |  | ENUMERATED (enabled, …) | Indicates whether Reflective QoS flow to DRB mapping should be applied. |  |  | - |  |  |
| Redundant QoS Flow Information | O |  | ENUMERATED (true, …) | This IE indicates that this QoS flow is requested for the redundant transmission. | YES | ignore |
| TSC Traffic Characteristics |  |  | O |  |  |  | 9.3.1.x | This IE indicates the TSC traffic characteristics as described in TS 23.501 [20]. This IE may be present in case of GBR QoS flows and shall be ignored otherwise. |  |  | YES |  | ignore |

#### 9.3.1.x	TSC Traffic Characteristics

This IE provides the traffic characteristics of TSC QoS flows.

Editor’s Note:	It is FFS whether to include this IE within the QoS Flow Level QoS Parameters IE, or at the same level as the QoS Flow Level QoS Parameters IE. The IE may be present in case of GBR QoS flows and is ignored otherwise.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Assistance Information Downlink | O |  | TSC Assistance Information 9.3.1.y |  |
| TSC Assistance Information Uplink | O |  | TSC Assistance Information 9.3.1.y |  |

#### 9.3.1.y	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | INTEGER (0..4095000) | Periodicity, in unit of 1us as specified in TS 23.501 [9]. |
| Burst Arrival Time | O |  | OCTET STRING (SIZE(4)) | Burst Arrival Time as specified in TS 23.501 [9]. |

#### 9.3.1.z	CN Packet Delay Budget

This IE indicates the CN Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Packet Delay Budget Downlink | O |  | INTEGER (0..5115) | Core Network Packet Delay Budget Downlink, in unit of 0.1ms is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. |
| CN Packet Delay Budget Uplink | O |  | INTEGER (0..5115) | Core Network Packet Delay Budget Uplink, in unit of 0.1ms is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. |