3GPP TSG-RAN3 Meeting #106	R3-197231

Reno, Nevada, US, 18-22 November 2019

Title: 	F1 impacts for NR-DC power control

Source: 	Huawei

Agenda item:	11.4

Document for:	Approval

1. Introduction

In the last RAN1#98 and #98bis meeting, RAN1 has agreed to support the semi-static power sharing and dynamic power sharing for NR-DC. In this contribution, we will discuss the impacts on the specifications of RAN3.

Agreements in RAN1#98:

- Considering the following two alternatives for semi-static power sharing with PMCGPMCG+ PSCG≤PTotalNN- DC - Alt.1: For the uplink transmission in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa. - If such overlapping with UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCGin MCG such that pMCG≤PMCG; - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and pSCG can be up to PSCG' . - Alt.1-1: PMCG' and PSCG' are configured by RRC signaling. - Alt.1-2: PMCG' and PSCG' are determined by RAN4 requirement. Alt.2: For the uplink transmission in MCG and in SCG, UE limits its actual transmission power pMCG to be up to PMCGand pSCG to be up to PSCG

Agreements in RAN1#98bis:

- Adopt Alt.1-2 and Alt.2 for semi-static power sharing for NR-NR DC. - Alt.1-2 is only subject to configured maximum transmission power defined by RAN4 - Configuration between Alt.1-2 and Alt.2 is supported. - FFS: add more clarification - FFS: applied for synchronous DC only or applied for both synchronous and asynchronous DC (which may be the same or different for Alt.1-2 and Alt. 2)

- Support dynamic power sharing - If there is no overlapping transmission, maximum power on CG i is determined by RAN4 spec without considering P_CG_i. - If there is overlapping transmission, maximum power on CG i is limited to P_CG_i. - Note: “look-ahead” operation is included as a UE capability below - In case of power limitation, MCG is prioritized over SCG and reuse CA rule within each CG - Optional UE capability to indicate the support of dynamic power sharing operation - Separate optional UE capability to indicate the support of ’look-ahead’ operation on condition that UE indicates support of dynamic power sharing operation.

3. Discussion

In (NG) EN-DC, for FR1 in MCG +FR1 in SCG, the semi-static power sharing is supported. RAN2 agreed that the MN sends the max power for NR (i.e. the p-maxNR-FR1), the max power for E-UTRA (i.e. the p-maxEUTRA), the max power of UE (i.e. the p-maxUE-FR1) to the SN in the CG-ConfigInfo. After the SN addition, the SN can also request a new max power value (using requestedP-MaxFR1) for the SCG in the CG-Config. In RAN3, the DU can request the requestedP-MaxFR1 for the SCG in the CG-Config.

9.3.1.26	DU to CU RRC Information

This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CellGroupConfig | M |  | OCTET STRING | CellGroupConfig, as defined in TS 38.331 [8]. |  |  |
| MeasGapConfig | O |  | OCTET STRING | MeasGapConfig as defined in TS 38.331 [8]. For EN-DC/NGEN-DC operation, includes the gap for FR2, as requested by the gNB-CU via MeasConfig IE. For NG-RAN,NE-DC and MN for NR-NR DC, includes the gap(s) for FR1 and/or FR2, as requested by the gNB-CU via MeasConfig IE and according to the requested gap type (per-UE or per-FR). |  |  |
| Requested P-MaxFR1 | O |  | OCTET STRING | requestedP-MaxFR1, as defined in TS 38.331 [8]. For EN-DC operation, this IE should be included. |  |  |
| DRX Long Cycle Start Offset | O |  | INTEGER (0..10239) | Identical to the value of the drx-LongCycleStartOffset IE within the DRX-Config as defined in TS 38.331. This field is not used in NR-DC. |  |  |
| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. For EN-DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination. | YES | ignore |
| Selected FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [8]. For EN-DC operation, this IE should be included so that gNB-CU is informed of the selected FeatureSet. | YES | ignore |
| Ph-InfoSCG | O |  | OCTET STRING | PH-TypeListSCG, as defined in TS 38.331[8].For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in SN. | Yes | ignore |
| Requested BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Band Combination. | YES | ignore |
| Requested FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Feature Set. | YES | ignore |
| Requested P-MaxFR2 | O |  | OCTET STRING | requestedP-MaxFR2, as defined in TS 38.331 [8]. For NR-DC operation, this IE should be included. | YES | ignore |
| DRX Config | O |  | OCTET STRING | DRX-Config, as defined in TS 38.331 [8]. This field is only used in NR-DC. | YES | ignore |
| PDCCH BlindDetectionSCG | O |  | OCTET STRING | pdcch-BlindDetectionSCG, as defined in TS 38.331[8]. This IE is used between the MgNB-DU and the MgNB-CU. | YES | ignore |
| Requested PDCCH BlindDetectionSCG | O |  | OCTET STRING | requestedPDCCH-BlindDetectionSCG, as defined in TS 38.331[8]. This IE is used between the SgNB-DU and the SgNB-CU. | YES | ignore |
| Ph-InfoMCG | O |  | OCTET STRING | PH-TypeListMCG, as defined in TS 38.331[8]. For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in MCG. | YES | ignore |

Observation 1: In EN-DC, the DU can request the new max power value (using requestedP-MaxFR1) in the DU to CU RRC Information.

In the last RAN1 meeting, RAN1 has agreed to support the semi-static power sharing and dynamic power sharing.

The agreements of RAN1 do not explicitly say for which band combination(s) the agreements are adopted. However, according to the following contents in the LS R2-1903001 of RAN1, there is no joint power limit for the FR1 in MCG + FR2 in SCG case and the FR2 in MCG + FR1 in SCG case need not be supported. Therefore, the only case where RAN1 agreements on joint power limit (listed in introduction of this document) can apply is the FR1 in MCG +FR1 in SCG and FR2 in MCG +FR2 in SCG case.

RAN1 discussed the issue of uplink power control for synchronous and asynchronous NR-NR dual connectivity. For the case of asynchronous NR-NR dual connectivity, RAN1 made the following agreement: Agreements: - For Rel. 16 UEs and asynchronous NN-DC operation, where MCG has serving cells only in FR1 and the SCG has serving cells only in FR2, the uplink power control is performed independently across cell groups - This is under the assumption that for NR Rel. 16, no joint power limit across FR1 and FR2 is defined by RAN4. In addition, RAN1 has not identified any use case to support the scenario, where SCG is fully in FR1 and MCG is fully in FR2 for either synchronous or asynchronous NR-NR dual connectivity operation. However, if supported, RAN1 has not identified other RAN1 specification impacts other than the UE capability and the power control aspect mentioned below: - If synchronous and asynchronous NR-NR dual connectivity with SCG fully in FR1 and MCG fully in FR2 are supported, uplink power control is performed independently across the two cell groups.

Observation 2: RAN1 agreed to support the semi-static power sharing in MCG +FR1 in SCG and FR2 in MCG+ FR2 in SCG in R16 NR-DC.

In our understanding, R16 NR-DC can reuse the principles of EN-DC, the SN can request the new max power in the SCG. Therefore the DU can request the new max power in the DU to CU RRC Information. For the FR1+FR1, we think RAN3 can reuse the existing parameter Requested P-MaxFR1.RAN3 only need to add the NR-DC case in the semantics description. In the R15, for the FR2 in the SCG, the SCG does not request the new max power. Therefore RAN3 decide to clarify in the semantics that Requested P-MaxFR2 is not used in in R15 in R3-196169. RAN3 need to recover the semantics in R16.

The modification in R3-196169:

| Requested P-MaxFR2 | O |  | OCTET STRING | This IE is not used in this version of the specification | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 1: Add the NR-DC case in the semantics description of Requested P-MaxFR1. Remove the note for Requested P-MaxFR2 IE of R3-196169 in R16. CR for F1 is provided in the Annex.

The proposed changes to TS 38.473 is in the Annex, and could be turned to CR if agreeable.

4. Conclusion

In this contribution, we discussed the impacts of F1 for R16 NR-DC power control. We have the following proposals:

Observation 1: In EN-DC, the DU can request the new max power value (using requestedP-MaxFR1) in the DU to CU RRC Information.

Observation 2: RAN1 agreed to support the semi-static power sharing in MCG +FR1 in SCG and FR2 in MCG+ FR2 in SCG in R16 NR-DC.

Proposal 1: Add the NR-DC case in the semantics description of Requested P-MaxFR1. Remove the note for Requested P-MaxFR2 IE of R3-196169 in R16. CR for F1 is provided in the Annex.

5. Annex – 38.473

3GPP TSG-RAN3 Meeting #106	R3-19xxxx

Reno, Nevada, US, 18-22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR |  | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | F1 impacts for NR-DC power control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently in EN-DC, the DU is able to request the new max power value (using requestedP-MaxFR1) in the DU to CU RRC Information. In R16, RAN1 has agreed to support the semi-static power sharing in MCG +FR1 in SCG and FR2 in MCG+ FR2 in SCG for NR-DC. Corresponding changes need to be applied over F1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the NR-DC case in the semantics description of Requested P-MaxFR1. - Remove the note for Requested P-MaxFR2 IE of R3-196169 in R16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The semi-static power sharing and dynamic power sharing for NR-DC cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.26	DU to CU RRC Information

This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CellGroupConfig | M |  | OCTET STRING | CellGroupConfig, as defined in TS 38.331 [8]. |  |  |
| MeasGapConfig | O |  | OCTET STRING | MeasGapConfig as defined in TS 38.331 [8]. For EN-DC/NGEN-DC operation, includes the gap for FR2, as requested by the gNB-CU via MeasConfig IE. For NG-RAN,NE-DC and MN for NR-NR DC, includes the gap(s) for FR1 and/or FR2, as requested by the gNB-CU via MeasConfig IE and according to the requested gap type (per-UE or per-FR). |  |  |
| Requested P-MaxFR1 | O |  | OCTET STRING | requestedP-MaxFR1, as defined in TS 38.331 [8]. For EN-DC and NR-DC operation, this IE should be included. |  |  |
| DRX Long Cycle Start Offset | O |  | INTEGER (0..10239) | Identical to the value of the drx-LongCycleStartOffset IE within the DRX-Config as defined in TS 38.331. This field is not used in NR-DC. |  |  |
| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. For EN-DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination. | YES | ignore |
| Selected FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [8]. For EN-DC operation, this IE should be included so that gNB-CU is informed of the selected FeatureSet. | YES | ignore |
| Ph-InfoSCG | O |  | OCTET STRING | PH-TypeListSCG, as defined in TS 38.331[8].For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in SN. | Yes | ignore |
| Requested BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Band Combination. | YES | ignore |
| Requested FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Feature Set. | YES | ignore |
| Requested P-MaxFR2 | O |  | OCTET STRING | requestedP-MaxFR2, as defined in TS 38.331 [8]. For NR-DC operation, this IE should be included. | YES | ignore |
| DRX Config | O |  | OCTET STRING | DRX-Config, as defined in TS 38.331 [8]. This field is only used in NR-DC. | YES | ignore |
| PDCCH BlindDetectionSCG | O |  | OCTET STRING | pdcch-BlindDetectionSCG, as defined in TS 38.331[8]. This IE is used between the MgNB-DU and the MgNB-CU. | YES | ignore |
| Requested PDCCH BlindDetectionSCG | O |  | OCTET STRING | requestedPDCCH-BlindDetectionSCG, as defined in TS 38.331[8]. This IE is used between the SgNB-DU and the SgNB-CU. | YES | ignore |
| Ph-InfoMCG | O |  | OCTET STRING | PH-TypeListMCG, as defined in TS 38.331[8]. For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in MCG. | YES | ignore |