3GPP TSG-RAN WG3 #106	R3-197404

Reno, Nevada , US, 18th November – 22nd November 2019

Agenda Item:	10.2.2.1

Source:	Ericsson,

Title:	Discussion on load information for MLB

Document for:	Discussion, Decision

# Introduction

In RAN3#105bis it was agreed that Composite Available Capacity (CAC) shall be exchanged over F1, Xn and X2, and that Transport Network Load (TNL) should be exchanged over F1 and E1. However, for other types of load metrics, such as PRB usage, and hardware (HW) load a decision has not been finalized. In additions to these, new metrics, such as number of active UEs and number of UEs in RRC_CONNECTED mode have been suggested to be introduced for load balancing and load sharing in NR. The following table from the RAN3#105bis Chairman Notes captures the mapping between load metrics and RAN interface and the current state of the discussion:

|  | F1 | E1 | Xn | X2 |
| --- | --- | --- | --- | --- |
| CAC | X |  | X | X |
| PRB Usage | X? |  | X? | X? |
| TNL load | X | X | X | X (from en-gNB) |
| HW load | X? | X? | X? | X? |
| Number of active UEs | X? |  | X? | X? |
| number of UEs in RRC_CONNECTED |  |  | X? |  |

In addition to this, RAN3 is still discussing the granularity of load information in terms of load per SSB area, per network slice, and per band.

This contribution discusses the remaining details of load information exchange over F1, E1, Xn and X2 interfaces as well as what granularity should be supported for each type of load information.

# Discussion

## CAC

RAN3 has agreed to use cell-level CAC for load balance and load sharing over Xn, X2 (for EN-DC) and F1. However, it was left for FFS to determine whether a finer granularity of CAC should be supported, namely

- CAC per SSB coverage area

- Slice Available Capacity

The current proposal is to define both metrics in a similar way, specifically by using the Cell Capacity Class Value (CCCV) together with either an SSB Area Capacity Value (CV) or a Slice Capacity Value, respectively.

### CAC per SSB area

The details of the CAC solution per SSB area solution have now been captured as FFS in text proposals to 38.423, and 38.473 [2][3][5]. Since RAN3 has extensively discussed the benefits of load information capturing the spatial distribution of load and, at the RAN3#105bis meeting, RAN3 has “acknowledged the need of reporting spatial load distribution of cells; a solution is needed”, we propose to agree on the current solution for CAC per SSB area over Xn, X2 and F1.

- RAN3 to agree on CAC per SSB area to be exchanged over Xn, X2, and F1 as captured in the text proposals [6][7][9].

### CAC per network slice

While at RAN3#105bis a possible solution describing Slice Available Capacity has been captured in [2][5], the meaning of the Slice Available Capacity and how it relates to the cell-level CAC needs to be further clarified.

For instance, a network slice can be configured to use dedicated resources but can also be allocated shared resources when its dedicated resources are fully utilized and there are unused resources in the cell. As such, it is unclear whether the Slice Available Capacity should refer exclusively to the radio resources dedicated to the network slice or it should also consider resources that are allocated to the slice in an ad-hoc basis. In order to define this metric in an unequivocal and interoperable way, clarifications on the concept would be needed.

- The Slice Available Capacity should remain FFS until RAN3 clarifies the meaning this capacity and how it relates to the cell-level CAC.

## TNL capacity indicator

At RAN3#105bis is was agreed to move from a TNL load indicator IE to a TNL capacity indicator IE as the latter is more beneficial and better interoperable for load balancing algorithms.

In LTE, both HW Load and TNL Load are reported using four values (Low, Medium, High, Overload). While such coarse granularity is generally undesirable for load balancing functionalities, in a multi-vendor scenario each of the four value may be triggered by a different load threshold for different vendor equipment and therefore lead to a non-homogeneous understanding of what Low, Medium, High, load is across the network.

To solve this issue, it has been suggested to consider a finer reporting granularity, e.g. in the range [0..100]. However, this relative measure of TNL capacity is also inadequate for proper load balancing algorithms as it hides the effective capacity offered by the gNB, namely the basic question is what does the value mean in terms of available resources.

For instance, consider the scenario wherein a gNB sends a RESOURCE STATUS REQUEST to two neighboring gNBs, each with a maximum TNL capacity of 1Gbps and 10Gbps, respectively. And assume that the gNB receives a RESOURCE STATUS UPDATE message from the two neighboring gNBs, each reporting a TNL available capacity indicator of 50. The gNB could then incorrectly interpret this information as if the same TNL capacity were available at both neighboring gNBs.

- Expressing the TNL capacity with a relative indicator in the range [0…100] is not enough to correctly assess the effective TNL available capacity.

One possible solution to this issue is to express the TNL capacity in absolute value, e.g. in kbps. However, for the purpose of load balancing algorithms, it would be beneficial to additionally indicate the maximum offered TNL capacity, so that it is possible to understand the level of load of each cell and therefore select the best target cell in terms of capacity available and overall load. The same representation of the TNL capacity indicator should then be adopted for both S1 TNL capacity indicator and the gNB-CU-CP TNL capacity indicator to harmonize the overall solution.

- The TNL capacity indicator and the gNB-CU-CP TNL capacity indicator should be expressed in absolute value, e.g., in kbps, and comprise two values:

  - TNL available capacity

  - TNL offered capacity

From RAN3#105bis, it was left FFS whether the TNL capacity indicator should be reported on a per cell basis. Given that the capacity of the transport network is typically not split on a per cell basis, we propose not to report TNL capacity indicator on a per cell basis, but to report it on a per node basis.

- TNL capacity indicator should not be reported on a per cell basis, but on a per node basis.

- RAN3 to agree to the text proposal for TNL capacity indicator in [6][7][8][9].

## Hardware capacity indicator

Similar to what was agreed for the TNL Capacity Indicator, it would be beneficial to move from a HW load indication to an indication of HW capacity instead.

- Consider expressing the HW state in terms of HW processing capacity instead of HW load.

Secondly, it would beneficial for load balancing algorithms to move from a coarse indication of HW capacity (e.g., Medium, Low, High), to a finer granularity.

Over the E1 interface, since the main functionality of the CU-UP is to handle UP traffic, it is reasonable to express the HW capacity in terms of offered capacity and available capacity. This allows for example selection of the most appropriate gNB-CU-UP in a gNB that has multiple gNB-CU-Ups available. In this case, the HW capacity could be indicated in absolute terms, e.g., in kbps.

For the HW capacity over X1, X2 and F1, one could consider using a relative measure in a range [0…100]. However, on one hand, the meaning of HW capacity (or, equivalently, HW load) is less clear in the case of CU and DU. On the other hand, similar to the argument described for the TNL case, using a relative measure of HW capacity would leave it unclear with respect to what a measure is taken. Therefore, it is not clear how such metriccan properly be used for the purpose of load balancing and load sharing. Therefore, the usefulness and the need to exchange a measure of HW capacity (or, equivalently, HW load) over F1, X2, Xn is less obvious and should be kept FFS until it is clarified.

- The HW capacity indicator over E1 should be expressed in absolute value, e.g., in kbps, and comprise two values:

  - Offered Throughput

  - Available Throughput

- RAN3 should clarify the meaning of HW capacity (or, equivalently, HW load) over Xn, X2, and F1.

- RAN3 to agree to the text proposal for HW capacity indicator in [6][7][8][9].

## Number of active UEs and UEs in RRC_CONNECTED mode

At RAN3#105bis, some companies proposed to consider the number of active UEs and/or the number of UE in RRC_CONNECTED mode as additional types of load information. While the proposal is interesting, a thoroughly analysis of how these metrics could be used for load balancing and their potential benefit over other metrics is still lacking. For instance, some companies noticed that the number of active UEs is very dynamic and can be zero in many cases, other companies noticed that some UEs, while being active or in RRC_CONNECTE mode, only have signalling connection and do not contribute to the cell load (in terms of, e.g., PRB utilization, etc.). Therefore, it is not obvious from the discussion held at the RAN3#105bis meeting whether this is a good metric for cell load and how it could be utilized. From our perspective, given that RAN3 has already agreed to introduce at least CAC per cell, load per cell and more, we should further discuss and evaluate the benefits of adding number of active UEs and number of UEs in RRC_CONNECTED compared to the currently available load information.

- Further discuss and evaluate the benefits of adding number of active UEs and number of UEs in RRC_CONNECTED compared to the currently agreed load information.

## MLB stage-2 description

Some aspects of the RAN3 MLB solutions are mature enough to be captured in a stage-2 description for TS 38.300. We therefore propose to capture the text proposal in Appendix to this document in TS 38-300.

- RAN3 to capture the stage-2 description in the appendix in TS 38.300.

# Conclusions

In this contribution, the following observations are captured:

- Expressing the TNL capacity with a relative indicator in the range [0…100] is not enough to correctly assess the effective TNL available capacity.

In this contribution, the following proposals are captured:

- RAN3 to agree on CAC per SSB area to be exchanged over Xn, X2, and F1 as captured in the text proposals [6][7][9].

- The Slice Available Capacity should remain FFS until RAN3 clarifies the meaning this capacity and how it relates to the cell-level CAC.

- The TNL capacity indicator and the gNB-CU-CP TNL capacity indicator should be expressed in absolute value, e.g., in kbps, and comprise two values:

  - TNL available capacity

  - TNL offered capacity

- TNL capacity indicator should not be reported on a per cell basis.

- RAN3 to agree to the text proposal for TNL capacity indicator in [6][7][8][9].

- Consider expressing the HW state in terms of HW processing capacity instead of HW load.

- The HW capacity indicator over E1 should be expressed in absolute value, e.g., in kbps, and comprise two values:

  - Offered Throughput

  - Available Throughput

- RAN3 should clarify the meaning HW capacity (or, equivalently, HW load) over Xn, X2, and F1.

- RAN3 to agree to the text proposal for HW capacity indicator in [6][7][8][9].

- Further discuss and evaluate the benefits of adding number of active UEs and number of UEs in RRC_CONNECTED compared to the currently agreed load information.

- RAN3 to capture the stage-2 description in the appendix in TS 38.300.

# References

- Chairmain notes, 3GPP TSG-RAN WG3 #105bis Chongqing, China, 14-18 October 2019.

- R3-196164, “(TP for SON BL CR for 38.423) Support for information on Spatial load distribution over Xn”, RAN3#105bis, October 2019.

- R3-196269, “(TP for SON BL CR for 38.423) Support on MLB for EN-DC scenario” , RAN3#105bis, October 2019.

- R3-196268, “(TP for SON BL CR for 38.463) MLB for E1” , RAN3#105bis, October 2019.

- R3-196267, “(TP for SON BL CR for 38.473) MLB”, RAN3#105bis, October 2019.

- R3-197401 “(TP for SON BL CR for 38.423) Support for information on Spatial load distribution over Xn”, RAN3#106, Reno, Nevada , US, 18-22 November 2019.

- R3-197405, “(TP for SON BL CR for 38.423) Support on MLB for EN-DC scenario”. RAN3#106, Reno, Nevada , US, 18-22 November 2019.

- R3-197402, “(TP for SON BL CR for 38.463) Addition on load information over E1”. RAN3#106, Reno, Nevada , US, 18-22 November 2019.

- R3-197403, “(TP for SON BL CR for 38.473) Support for information on load distribution over F1”. RAN3#106, Reno, Nevada , US, 18-22 November 2019.

# Appendix – TP for MLB Stage-2 description for 38.300

--------------------------------------------------Start of the change---------------------------------------------------

## 15.X	Self-optimisation

### 15.X.1	Support for Mobility Load Balancing

Editor’s note: This section captures the stage 2 descriptions for Mobility Load Balancing

#### 15.X.1.1	General

Editor’s note: the content of this section is FFS.

The objective of mobility load balancing is to distribute cell load evenly among cells and across areas of cells covered by different SSBs or to transfer part of the traffic from congested cell or from congested areas of cells covered by different SSBs, or to offload users from one cell or carrier or RAT or areas of cells covered by different SSBs to achieve network energy saving, This can be done by means of optimization of cell reselection/handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

Both intra-RAT and intra-system inter-RAT load balancing scenarios should be supported.

In general, support for mobility load balancing consists of one or more of following functions:

-	 Load reporting;

- Load balancing action based on handovers;

-	 Adapting handover and/or reselection configuration.

#### 15.X.1.2 Load reporting

Editor’s note: the content of this section is FFS.

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

Editor’s note: load related information and its definition are FFS.

The following load related information should be supported which consists of,

-	Radio resource usage (per cell and per SSB area: DL/UL/SUL GBR PRB usage, DL/UL/SUL non-GBR PRB usage, DL/UL/SUL total PRB usage);

-	TNL load indicator (UL/DL /SUL TNL available capacity);

-	Cell Capacity Class value (per cell and per SSB area UL/DL relative capacity indicator);

- Capacity value (per cell and per SSB area UL/DL available capacity for load balancing as percentage of total cell capacity).

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

--------------------------------------------------End of the change-----------------------------------------------------