3GPP TSG-RAN3 Meeting #106	R3-197599

Reno, Nevada, US, 18th November – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resuming SCG in RRC Resume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has agreed to support resuming a suspended SCG configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the section describing the suspension and resumption of the SCG in MR-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to support resuming a suspended SCG configuration, as is already agreed in RAN2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.12	Activity Notification

### 10.12.2	MR-DC with 5GC

The Activity Notification function is used to report user plane activity within SN resources or to report a RAN Paging Failure event to the SN. It can either report inactivity or resumption of activity after inactivity was reported. In MR-DC with 5GC the Activity Reporting is provided from the SN only. The MN may take further actions. RAN Paging Failure Reporting is provided from the MN only.

MR-DC with 5GC Activity Notification

Figure 10.12.2-1: Support of Activity Notification in MR-DC with 5GC

1.	The SN notifies the MN about user data inactivity.

2.	The MN decides further actions that impact SN resources (e.g. send UE to RRC_INACTIVE, bearer reconfiguration). In the case shown, MN takes no action.

3.	The SN notifies the MN that the (UE or PDU Session or QoS flow) is no longer inactive.

MR-DC with 5GC with RRC_INACTIVE – SCG configuration released in SN

The Activity Notification function may be used to enable MR-DC with 5GC with RRC_INACTIVE operation. The MN node may decide, after inactivity is reported from the SN and also MN resources show no activity, to send the UE to RRC_INACTIVE. Resumption to RRC_CONNECTED may take place after activity is reported from the SN for SN terminated bearers.

Figure 10.12.2-2: Support of Activity Notification in MR-DC with 5GC with RRC_Inactive – SCG configuration released in SN

Figure 10.12.2-2 shows how Activity Notification function interacts with NG-RAN functions for RRC_INACTIVE and SN Modification procedures in order to keep the higher layer MR-DC configuration established for UEs in RRC_INACTIVE, including NG and Xn interface C-plane, U-plane and bearer contexts established while lower layer MCG and SCG resources are released. When the UE transits successfully back to RRC_CONNECTED, lower layer MCG and SCG resources are established afterwards by means of RRC Connection Reconfiguration.

Editors Note: RRC related details are FFS.

Editors Note: Figure 10.12.2-2 depicts the case where the UE when coming back to RRC_CONNECTED consumes radio resources from the same SN and MN as before it was sent to RRC_INACTIVE. Whether further scenarios need to be depicted needs further discussions.

1.	The SN notifies the MN about user data inactivity for SN terminated bearers.

2.	The MN decides to send the UE to RRC_INACTIVE.

3/4.	The MN triggers the MN initiated SN Modification procedure, requesting the SN to release lower layers.

5.	The UE is sent to RRC_INACTIVE.

6-8.	After a period of inactivity, upon activity notification from the SN, the UE returns to RRC_CONNECTED.

9/10.	The MN triggers the MN initiated SN Modification procedure to re-establish lower layers. The SN provides configuration data within an SN RRC configuration message.

11-14.	The RRCConnectionReconfiguration procedure commences.

MR-DC with 5GC with RRC_INACTIVE - SCG configuration suspended in SN

The Activity Notification function may be used to enable MR-DC with 5GC with RRC_INACTIVE operation. The MN node may decide, after inactivity is reported from the SN and also MN resources show no activity, to send the UE to RRC_INACTIVE, while keeping the SCG configuration. Resumption to RRC_CONNECTED may take place after activity is reported from the SN for SN terminated bearers.

Figure 10.12.2-x: Support of Activity Notification in MR-DC with 5GC with RRC_Inactive - SCG configuration suspended in SN

Figure 10.12.2-x shows how Activity Notification function interacts with NG-RAN functions for RRC_INACTIVE and SN Modification procedures in order to keep the full MR-DC configuration established for UEs in RRC_INACTIVE. When the UE transits successfully back to RRC_CONNECTED, lower layer MCG and SCG configurations are restored or reconfigured afterwards by means of RRC (Connection) Resume.

Editors Note: RRC related details are FFS.

Editors Note: Figure 10.12.2-x depicts the case where the UE when coming back to RRC_CONNECTED consumes radio resources from the same SN and MN as before it was sent to RRC_INACTIVE. Whether further scenarios need to be depicted needs further discussions.

1.	The SN notifies the MN about user data inactivity for SN terminated bearers.

2.	The MN decides to send the UE to RRC_INACTIVE.

3/4.	The MN triggers the MN initiated SN Modification procedure, requesting the SN to suspend lower layers.

5.	The UE is sent to RRC_INACTIVE.

6-7.	After a period of inactivity, upon activity notification from the SN, the MN decides to return the UE to RRC_CONNECTED.

8/9.	The MN triggers the MN initiated SN Modification procedure to resume the SCG lower layers. If the SCG configuration needs to be updated, the SN provides the configuration data within an SN RRC configuration message.

10-12.	The RRC(Connection)Resume procedure commences, where the UE is instructed to resume both the MCG and the SCG. If the SCG configuration is to be updated, the new configuration is provided in the RRC(Connection)Resume message.

13.	The MN informs the SN that the UE has completed the reconfiguration procedure successfully. via SN Reconfiguration Complete message, including the SN RRC response message, if received from the UE.13.13.

14.	If instructed, the UE performs synchronisation towards the PSCell of the SN.