3GPP TSG-RAN WG3 #107-e																		   R3-200265

24 February-6 March 2020															 Revision of R3-196639

E-Meeting

Agenda item:			14.3.2

Source:					ZTE, Huawei

Title:	(TP to BL CR#0120 Introduction of NB-IoT Paging and eDRX aspects for TS38.413): Removal of NB-IoT UE Identity Index IE

Document for:		Approval

- Introduction

Since it has already been agreed in RAN2#108 meeting that 5G-S-TMSI is used as UE Paging Identity for PO determination for NB-IoT UE connection to 5GC as follows, it is not useful to introduce NB-IoT UE Identity Index IE as UE Paging Identity in Ng interface.

RAN2#108 agreements: 5G-S-TMSI is used for PO determination for eMTC and NB-IoT UE connection to 5GC. 5G-S-TMSI mod 16384 is used as the UE_ID for PO calculation for eMTC and NB-IoT UE connection to 5GC.

So, we suggest to remove the NB-IoT UE Identity Index IE by this TP.

- Text Propose (TS38.413 15.6.0)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

--------Start of the First Change----

8.5.1	Paging

8.5.1.1	General

The purpose of the Paging procedure is to enable the AMF to page a UE in the specific NG-RAN node.

8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

The AMF initiates the Paging procedure by sending the PAGING message to the NG-RAN node.

At the reception of the PAGING message, the NG-RAN node shall perform paging of the UE in cells which belong to tracking areas as indicated in the TAI List for Paging IE.

If the Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 38.304 [12] and TS 36.304 [29].

For each cell that belongs to any of the tracking areas indicated in the TAI List for Paging IE, the NG-RAN node shall generate one page on the radio interface.

If the Paging Priority IE is included in the PAGING message, the NG-RAN node may use it according to TS 23.501 [9].

If the UE Radio Capability for Paging IE is included in the PAGING message, the NG-RAN node may use it to apply specific paging schemes.

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present, according to TS 38.300 [8].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be used for paging the UE according to TS 38.300 [8].

If the Paging Origin IE is included in the PAGING message, the NG-RAN node shall transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the NB-IoT Paging eDRX Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it according to TS 36.304 [29]. If the NB-IoT Paging Time Window IE is included in the NB-IoT Paging eDRX Information IE, the NG-RAN node shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [29]. The NG-RAN node should take into account the reception time of the PAGING message on the NG interface to determine when to page the UE.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.2.4.1	PAGING

This message is sent by the AMF and is used to page a UE in one or several tracking areas.

Direction: AMF  gNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Paging Identity | M |  | 9.3.3.18 |  | YES | ignore |
| Paging DRX | O |  | 9.3.1.90 |  | YES | ignore |
| TAI List for Paging |  | 1 |  |  | YES | ignore |
| >TAI List for Paging Item |  | 1..<maxnoofTAIforPaging> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Paging Priority | O |  | 9.3.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Paging Origin | O |  | 9.3.3.22 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.3.1.69 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.3.1.yyy |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is 16. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

SKIPPED ASN.1 UNCHANGED

LocationReportingRequestType,

MaskedIMEISV,

MessageIdentifier,

MobilityRestrictionList,

NAS-PDU,

NASSecurityParametersFromNGRAN,

NB-IoT-DefaultPagingDRX,

NB-IoT-Paging-eDRXInfo,

NewSecurityContextInd,

NGRAN-CGI,

NGRAN-TNLAssociationToRemoveList,

NGRANTraceID,

SKIPPED ASN.1 UNCHANGED

FROM NGAP-IEs

SKIPPED ASN.1 UNCHANGED

id-MobilityRestrictionList,

id-NAS-PDU,

id-NASC,

id-NASSecurityParametersFromNGRAN,

id-NB-IoT-DefaultPagingDRX,

id-NB-IoT-Paging-eDRXInfo,

id-NewAMF-UE-NGAP-ID,

id-NewGUAMI,

id-NewSecurityContextInd,

SKIPPED ASN.1 UNCHANGED

FROM NGAP-Constants;

SKIPPED ASN.1 UNCHANGED

PagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UEPagingIdentity		CRITICALITY ignore	TYPE UEPagingIdentity	PRESENCE mandatory	}|

{ ID id-PagingDRX			CRITICALITY ignore	TYPE PagingDRX			PRESENCE optional		}|

{ ID id-TAIListForPaging		CRITICALITY ignore	TYPE TAIListForPaging	PRESENCE mandatory	}|	{ ID id-PagingPriority		CRITICALITY ignore	TYPE PagingPriority		PRESENCE optional		}|

{ ID id-UERadioCapabilityForPaging	CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional		}|	{ ID id-PagingOrigin			CRITICALITY ignore	TYPE PagingOrigin		PRESENCE optional		}|

{ ID id-AssistanceDataForPaging	CRITICALITY ignore	TYPE AssistanceDataForPaging		PRESENCE optional		}|

{ ID id-NB-IoT-Paging-eDRXInfo	CRITICALITY ignore	TYPE NB-IoT-Paging-eDRXInfo	PRESENCE optional	}

,

...

}

SKIPPED ASN.1 UNCHANGED

9.4.5	Information Element Definitions

#### -- N

SKIPPED ASN.1 UNCHANGED

N3IWF-ID ::= CHOICE {

n3IWF-ID				BIT STRING (SIZE(16)),

choice-Extensions		ProtocolIE-SingleContainer { {N3IWF-ID-ExtIEs} }

}

N3IWF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NAS-PDU ::= OCTET STRING

NASSecurityParametersFromNGRAN ::= OCTET STRING

NB-IoT-DefaultPagingDRX ::= ENUMERATED {rf128, rf256, rf512, rf1024, ... }

NB-IoT-Paging-eDRXCycle ::= ENUMERATED {hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, ...}

NB-IoT-Paging-TimeWindow ::= ENUMERATED {s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, ... }

NB-IoT-Paging-eDRXInfo ::= SEQUENCE {

nB-IoT-Paging-eDRXCycle NB-IoT-Paging-eDRXCycle,

nB-IoT-Paging-TimeWindow NB-IoT-Paging-TimeWindow OPTIONAL,

...

}

NetworkInstance ::= INTEGER (1..256, ...)

NewSecurityContextInd ::= ENUMERATED {

true,

...

}

SKIPPED ASN.1 UNCHANGED

### 9.4.7	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

SKIPPED ASN.1 UNCHANGED

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation							ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance									ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-NB-IoT-Paging-eDRXInfo								ProtocolIE-ID ::= xxx

id-NB-IoT-DefaultPagingDRX								ProtocolIE-ID ::= xxz

SKIPPED ASN.1 UNCHANGED

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>