3GPP TSG-RAN WG3 Meeting #107-e	R3-200434

# E-meeting, 24th February – 6th March, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | N/A | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR for TS 38.423) Correction on MLB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2020-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Miscellaneous Corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add SSB Index into each item in SSB area list. 2. Change the presence of every cell list IEs into optional. 3. Add IEs to indicate SUL PRB usage and SUL PDCCH CCE usage. 4. Some other miscellaneous corrections, such as correcting wrong value ranges, inconsistent presence, etc. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IEs for MLB remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.A, 9.1.3.D, 9.2.2.yy, 9.2.2.c, 9.2.2.m, 9.2.2.o (new), 9.3.4, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.423 CR N/A TS 38.473 CR N/A |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.1.3.A	RESOURCE STATUS REQUEST

Editor’s note: The content of this section is FFS

This message is sent by an NG-RAN node1 to neighbouring NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  |  |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 |  |  |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporPartialStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 |  |  |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. |  |  |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW Capacity Ind Periodic, Fifth Bit = Number of Active UEs, Sixth Bit = RRC connections. Other bits shall be ignored by the NG-RAN node2. |  |  |
| Cell To Report |  | 0..1 |  | Cell ID list to which the request applies. |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | YES | Ignore |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | EACH | Ignore |
| >>>>SSB Index | M |  | SSB Index 9.2.2.o |  |  |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>>>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Capacity Ind Periodic, Composite Available Capacity Periodic. |  |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems [FFS] | Maximum no. of signalled slice support items. Value is 1024. [FFS] |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.1.3.D	RESOURCE STATUS UPDATE

Editor’s note: The content of this section is FFS

This message is sent by NG-RAN node2 to neighbouring NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Hardware Load Indicator [FFS] | O |  | 9.2.2.ww |  |  |  |
| Cell Measurement Result |  | 0..1 |  |  |  |  |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | 9.2.3.25 |  |  |  |
| >>Radio Resource Status [FFS] | O |  | 9.2.2.yy |  |  |  |
| >>NG TNL Load Indicator [It is FFS if reported per cell] | O |  | 9.2.2.xx |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.zz |  |  |  |
| >>Slice Available Capacity [FFS] | O |  | 9.2.2.jj |  |  |  |
| >>Number of Active UEs [FFS] | O |  | INTEGER (0..65535,...) |  |  |  |
| >> RRC Connections | O |  | 9.2.2.kk |  |  |  |

| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| --- | --- |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.2.2.yy	Radio Resource Status

[The information is FFS]

The Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Radio Resource Status Type | M |  |  |  |
| >ng-eNB |  |  |  |  |
| >>DL GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage |
| >>UL GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage |
| >>DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage |
| >>UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage |
| >>DL Total PRB usage | M |  | INTEGER (0..100) | Per cell DL Total PRB usage |
| >>UL Total PRB usage | M |  | INTEGER (0..100) | Per cell UL Total PRB usage |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >gNB |  |  |  |  |
| >>SSB Area Radio Resource Status List |  | 1 |  |  |
| >>>SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | SSB Index 9.2.2.o |  |
| >>>>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage |
| >>>>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage |
| >>>>SSB Area SUL GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL GBR PRB usage |
| >>>>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage |
| >>>>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage |
| >>>>SSB Area SUL non-GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL non-GBR PRB usage |
| >>>>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage |
| >>>>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage |
| >>>>SSB Area SUL Total PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL Total PRB usage |
| >>>>SSB Area DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>>>SSB Area UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>>>SSB Area SUL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.2.2.c	Capacity Value

The Capacity Value IE indicates the amount of resources per cell and per SSB area that are available relative to the total NG-RAN resources. The capacity value should be measured and reported so that the minimum NG-RAN resource usage of existing services is reserved according to implementation. The Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity with respect to the whole cell. Capacity Value should be measured on a linear scale. |  |  |
| SSB Area Capacity Value List |  | 0..1 |  |  |  |  |
| >SSB Area Capacity Value Item |  | 1..<maxnoofSSBAreas> |  |  |  |  |
| >>SSB Index | M |  | SSB Index 9.2.2.o |  | - |  |
| >>SSB Area Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . SSB Area Capacity Value should be measured on a linear scale. | - |  |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.2.2.m Number of RRC Connections

The Number of RRC Connections IE indicates the absolute number of UEs in RRC_CONNECTED mode.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number of RRC Connections | M |  | INTEGER (0..65535,...) |  |  |  |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.2.2.o SSB Index

The SSB Index IE identify an SSB area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Index | M |  | INTEGER (0..63,...) |  |  |  |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.3.4	PDU Definitions

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- RESOURCE STATUS REQUEST

--

-- **************************************************************

ResourceStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusRequest-IEs}},

...

}

ResourceStatusRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID		CRITICALITY ignore	TYPE Measurement-ID					PRESENCE conditional}|-- This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add".

{ ID id-RegistrationRequest					CRITICALITY reject	TYPE RegistrationRequest				PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics			PRESENCE optional}|

{ ID id-CellToReport							CRITICALITY ignore	TYPE CellToReport					PRESENCE optional}|

{ ID id-ReportingPeriodicity				CRITICALITY ignore	TYPE ReportingPeriodicity			PRESENCE optional},

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- RESOURCE STATUS UPDATE

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusUpdate-IEs}},

...

}

ResourceStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-HardwareLoadIndicator		CRITICALITY reject	TYPE HardwareLoadIndicator			PRESENCE optional}|--FFS

{ ID id-CellMeasurementResult		CRITICALITY ignore	TYPE CellMeasurementResult		PRESENCE optional},

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

9.3.5	Information Element definitions

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- C

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

CellToReport ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellToReport-Item

CellToReport-Item	::= SEQUENCE {

cell-ID									Target-CGI,

sSBToReport-List                        SSBToReport-List								OPTIONAL,

sliceToReport-List                      SliceToReport-List								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellToReport-Item-ExtIEs} }	OPTIONAL,

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- D

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

DL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- N

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

NumberofActiveUEs::= INTEGER (0..65535,...)

NoofRRCConnections ::= INTEGER (0..65535,...)

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

NG-eNB-RadioResourceStatus	::= SEQUENCE {

dL-GBR-PRB-usage							DL-GBR-PRB-usage,

uL-GBR-PRB-usage							UL-GBR-PRB-usage,

dL-non-GBR-PRB-usage						DL-non-GBR-PRB-usage,

uL-non-GBR-PRB-usage						UL-non-GBR-PRB-usage,

dL-Total-PRB-usage						DL-Total-PRB-usage,

uL-Total-PRB-usage						UL-Total-PRB-usage,

dL-scheduling-PDCCH-CCE-usage			DL-scheduling-PDCCH-CCE-usage					OPIONAL,

uL-scheduling-PDCCH-CCE-usage			UL-scheduling-PDCCH-CCE-usage					OPIONAL,

iE-Extensions							ProtocolExtensionContainer { { NG-eNB-RadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

NG-eNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- S

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

SSBAreaCapacityValue-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaCapacityValue-List-Item

SSBAreaCapacityValue-List-Item	::= SEQUENCE {

ssb-Index							SSB-Index,

ssbAreaCapacityValue	INTEGER (0..100),

iE-Extensions						ProtocolExtensionContainer { { SSBAreaCapacityValue-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaCapacityValue-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSBAreaRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaRadioResourceStatus-List-Item

SSBAreaRadioResourceStatus-List-Item	::= SEQUENCE {

ssb-Index							SSB-Index,

ssb-Area-DL-GBR-PRB-usage	DL-GBR-PRB-usage,

ssb-Area-UL-GBR-PRB-usage	UL-GBR-PRB-usage,

ssb-Area-SUL-GBR-PRB-usage	UL-GBR-PRB-usage	OPIONAL,

ssb-Area-dL-non-GBR-PRB-usage		DL-non-GBR-PRB-usage,

ssb-Area-uL-non-GBR-PRB-usage		UL-non-GBR-PRB-usage,

ssb-Area-SUL-non-GBR-PRB-usage		UL-non-GBR-PRB-usage							OPIONAL,

ssb-Area-dL-Total-PRB-usage			DL-Total-PRB-usage,

ssb-Area-uL-Total-PRB-usage			UL-Total-PRB-usage,

ssb-Area-SUL-Total-PRB-usage		UL-Total-PRB-usage								OPIONAL,

ssb-DL-scheduling-PDCCH-CCE-usage	DL-scheduling-PDCCH-CCE-usage					OPIONAL,

ssb-UL-scheduling-PDCCH-CCE-usage	UL-scheduling-PDCCH-CCE-usage					OPIONAL,

ssb-SUL-scheduling-PDCCH-CCE-usage	UL-scheduling-PDCCH-CCE-usage					OPIONAL,

iE-Extensions						ProtocolExtensionContainer { { SSBAreaRadioResourceStatus-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaRadioResourceStatus-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSB-Index::= INTEGER (0..63,...)

SSBToReport-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBToReport-List-Item

SSBToReport-List-Item	::= SEQUENCE {

ssb-Index							SSB-Index,

iE-Extensions						ProtocolExtensionContainer { { SSBToReport-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBToReport-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- U

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

UL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

//////////////////////////////////////////////////////////TP end//////////////////////////////////////////////////////////