3GPP TSG-RAN WG3 #107-e 																		R3-200510

24 February-6 March 2020 E-Meeting

Title: 	(TP for NR_Mob_enh BL CR for TS 38.300): Early forwarding transfer for DAPS handover

Source: 	Huawei

Agenda item:	15.4.1.1

Document for:	Other

- Introduction

At last meeting, a new class 2 procedure early forwarding transfer was proposed to convey the DL PDCP SN status to the target node for DAPS handover.

However, there is still FFS on whether to introduce the new procedure or to reuse the current SN Status Transfer procedure. This contribution proposes RAN3 to agree on the new procedure for DL PDCP SN transfer for DAPS handover.

- Discussion

Basically, the following benefits are foreseen for using the new early forwarding transfer procedure:

- There is no need and impossible to transfer the accurate UL PDCP SDU receiving status from the source node to the target node before the UE switch the Uu link in case of DAPS handover;

- The new early forwarding transfer procedure can be used to inform the target node to discard some DL packets which are delivered to the UE successfully in the meantime of DAPS handover execution.

- Avoid any impact to legacy SN Status Transfer procedure, because the legacy SN Status Transfer procedure conveys both the DL and UL PDCP SN status;

Proposal: To use the new defined procedure e.g. EARLY FORWARDING TRANSFER to indicate the DL PDCP SN value for the first forward DL PDCP SDU and update the BLCRs accordingly.

Besides, the following redundant sentence are removed from the TS 38.300 and the TS 36.300 BLCRs, because it appears twice in two places:

“The source gNB does not stop assigning SNs to downlink PDCP SDUs until it sends the SN STATUS TRANSFER message to the target gNB in step 8b.”

3. Conclusion

In this paper, we discuss about early forwarding transfer for DAPS handover and we have the following simple proposal:

Proposal: To use the new defined procedure e.g. EARLY FORWARDING TRANSFER to indicate the DL PDCP SN value for the first forward DL PDCP SDU and update the BLCRs accordingly.

The corresponding TP to TS 38.300 BLCR is provided in the ANNEX. The rest TPs for other BLCRs are provided in the reference [1] ~ [3].

3. Reference

[1] R3-200511	(TP for LTE_feMob BL CR for TS 36.300): Early forwarding transfer for DAPS handover.

[2] R3-200512	(TP for NR_Mob_enh BL CR for TS 38.423): Early forwarding transfer for DAPS handover.

[3] R3-200513	(TP for LTE_feMob BL CR for TS 36.423): Early forwarding transfer for DAPS handover.

ANNEX - TP for NR_Mob_enh BL CR for TS 38.300

Start of change

##### 9.2.3.2.1	C-Plane Handling

The intra-NR RAN handover performs the preparation and execution phase of the handover procedure performed without involvement of the 5GC, i.e. preparation messages are directly exchanged between the gNBs. The release of the resources at the source gNB during the handover completion phase is triggered by the target gNB. The figure below depicts the basic handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.2.1-1: Intra-AMF/UPF Handover

0.	The UE context within the source gNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1.	The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source gNB decides to handover the UE, based on MeasurementReport and RRM information.

3.	The source gNB issues a Handover Request message to the target gNB passing a transparent RRC container with necessary information to prepare the handover at the target side. The information includes at least the target cell ID, KgNB*, the C-RNTI of the UE in the source gNB, RRM-configuration including UE inactive time, basic AS-configuration including antenna Info and DL Carrier Frequency, the current QoS flow to DRB mapping rules applied to the UE, the SIB1 from source gNB, the UE capabilities for different RATs, PDU session related information, and can include the UE reported measurement information including beam-related information if available. The PDU session related information includes the slice information and QoS flow level QoS profile(s).

NOTE:	After issuing a Handover Request, the source gNB should not reconfigure the UE, including performing Reflective QoS flow to DRB mapping.

4.	Admission Control may be performed by the target gNB. Slice-aware admission control shall be performed if the slice information is sent to the target gNB. If the PDU sessions are associated with non-supported slices the target gNB shall reject such PDU Sessions.

5.	The target gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB, which includes a transparent container to be sent to the UE as an RRC message to perform the handover.

NOTE:	As soon as the source gNB receives the HANDOVER REQUEST ACKNOWLEDGE, or as soon as the transmission of the handover command is initiated in the downlink, data forwarding may be initiated.

NOTE:	For DAPS Handover, downlink PDCP SDUs are forwarded with SN assigned by the source gNB for DRBs subject to be simultaneously served by both the source and target gNBs, regardless of RLC-AM or RLC-UM, until SN assignment is handed over to the target gNB in step 8b, for which the normal data forwarding follows as defined in 9.2.3.2.3.

6.	The source gNB triggers the Uu handover by sending an RRCReconfiguration message to the UE, containing the information required to access the target cell: at least the target cell ID, the new C-RNTI, the target gNB security algorithm identifiers for the selected security algorithms. It can also include a set of dedicated RACH resources, the association between RACH resources and SSB(s), the association between RACH resources and UE-specific CSI-RS configuration(s), common RACH resources, and system information of the target cell, etc.

NOTE:	In case of DAPS Handover, for DRBs subject to be simultaneously served by the source and the target gNBs, the source gNB does not stop transmitting downlink packets until it receives the HO SUCCESS message from the target gNB in step 8a.

7.	The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of DRBs for which PDCP status preservation applies (i.e. for RLC AM). The uplink PDCP SN receiver status includes at least the PDCP SN of the first missing UL PDCP SDU and may include a bit map of the receive status of the out of sequence UL PDCP SDUs that the UE needs to retransmit in the target cell, if any. The downlink PDCP SN transmitter status indicates the next PDCP SN that the target gNB shall assign to new PDCP SDUs, not having a PDCP SN yet. The source gNB may omit sending this message if none of DRBs shall be treated with PDCP status preservation.

NOTE:	In case of DAPS Handover, the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status are also conveyed for DRBs with RLC-UM, if simultaneously served by the source and the target gNBs. For DRBs which are not subject to be simultaneously served by the source and the target gNBs, the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status are not coveyed in the SN STATUS TRANSFER message, if with RLC-UM. If with RLC-AM, then the description in step 7 applies, for which the normal data forwarding follows as defined in 9.2.3.2.3.

NOTE:	In case of DAPS Handover, the EARLY FORWARDING TRANSFER message is triggered by the source gNB in step 7, instead of the SN STATUS TRANSFER message. For DRBs subject to be simultaneously served by the source and the target gNBs, the DL COUNT value conveyed in the EARLY FORWARDING TRANSFER message indicates PDCP SN and HFN of the first PDCP SDU that the source gNB forwards to the target gNB.

NOTE:	In case of DAPS Handover, for DRBs subject to be simultaneously served by the source and the target gNBs, the source gNB may additionally send the EARLY FORWARDING TRANSFER message(s) between step 7 and step 8b, to inform discarding of already forwarded PDCP SDUs, The target gNB does not transmit forwarded downlink PDCP SDUs to the UE, whose COUNT is less than the conveyed DL COUNT value and discards them if transmission has not been attempted already.

8.	The UE synchronises to the target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB.

8a/b	In case of DAPS Handover, the target gNB sends the HO SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message to the target gNB as described in step 7 and the normal data forwarding follows as defined in 9.2.3.2.3.

NOTE:	In case of DAPS Handover, for DRBs subject to be simultaneously served by the source and the target gNBs for which duplication avoidance is required, the source gNB does not stop deliverying uplink QoS flows to the UPF until it sends the SN STATUS TRANSFER message in step 8b. The target gNB does not forward QoS flows of the uplink PDCP SDUs successfully received in-sequence to the UPF until it receives the SN STATUS TRANSFER message, in which UL HFN and the first missing SN in the uplink PDCP SN receiver status indicates the start of uplink PDCP SDUs to be delivered to the UPF. The target gNB does not deliver any uplink PDCP SDUs which has an UL COUNT lower than the provided.

9.	The target gNB sends a PATH SWITCH REQUEST message to AMF to trigger 5GC to switch the DL data path towards the target gNB and to establish an NG-C interface instance towards the target gNB.

10.	5GC switches the DL data path towards the target gNB. The UPF sends one or more "end marker" packets on the old path to the source gNB per PDU session/tunnel and then can release any U-plane/TNL resources towards the source gNB.

11.	The AMF confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

12.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF, the target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the handover. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

The RRM configuration can include both beam measurement information (for layer 3 mobility) associated to SSB(s) and CSI-RS(s) for the reported cell(s) if both types of measurements are available. Also, if CA is configured, the RRM configuration can include the list of best cells on each frequency for which measurement information is available. And the RRM measurement information can also include the beam measurement for the listed cells that belong to the target gNB.

The common RACH configuration for beams in the target cell is only associated to the SSB(s). The network can have dedicated RACH configurations associated to the SSB(s) and/or have dedicated RACH configurations associated to CSI-RS(s) within a cell. The target gNB can only include one of the following RACH configurations in the Handover Command to enable the UE to access the target cell:

i)	Common RACH configuration;

ii)	Common RACH configuration + Dedicated RACH configuration associated with SSB;

iii)	Common RACH configuration + Dedicated RACH configuration associated with CSI-RS.

The dedicated RACH configuration allocates RACH resource(s) together with a quality threshold to use them. When dedicated RACH resources are provided, they are prioritized by the UE and the UE shall not switch to contention-based RACH resources as long as the quality threshold of those dedicated resources is met. The order to access the dedicated RACH resources is up to UE implementation.

End of change