3GPP TSG-RAN3 Meeting #107-e	R3-200713

E-Meeting, 24 February – 6 March, 2020

Title: 	(TP for NPN BL CR for TS 38.300): Access control for NPN

Source: 	Huawei

Agenda item:	16.2.2.1

Document for:	Discussion

Annex – TP for TS 38.300 (on the top of R3-200050)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

## 16.y	Public Network Integrated NPN

<Unchanged Text Omitted>

### 16.y.y2	Access Control

The 5GC supports the access control function as defined in TS 23.501 [3].

During the establishment of the UE-associated logical NG-connection towards the 5GC originating from a PLMN cell, the AMF checks that the UE is allowed to access non-CAG cells.

During the establishment of the UE-associated logical NG-connection towards the 5GC originating from a CAG cell, the AMF checks that the CAG-only access UE is allowed to access at least one of the CAGs which is signalled as supported by the CAG cell related to the selected PLMN.

During the establishment of the UE-associated logical NG-connection towards the 5GC originating from a CAG cell, the AMF checks that the UE enabled to access both CAG and PLMN is allowed to access at least one of the CAGs which is signalled as supported by the CAG cell, or to access the supported public network of the CAG cell, related to the selected PLMN.

If the check is successful, the AMF sets up the UE-associated logical NG-connection and provides the NG-RAN node with the list of CAGs allowed for the UE and, whether the UE is allowed to access non-CAG cells. This information is used by the NG-RAN for access control of subsequent mobility.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>