3GPP TSG-RAN WG3 Meeting #107-e	R3-200818

# Online, February 24th – March 6th 2020

Agenda Item:	13.2.4

Source:	Ericsson

Title:	On Multi-connectivity for IAB-nodes

Document for:	Agreement

# 1 Introduction

At RAN2#107 meeting, it was agreed that multi-connectivity can be supported via dual connectivity (DC), by assigning the roles of MN and SN to the IAB-nodes serving the outer leaf access IAB-node. This means that the IAB-node will be configured with 2 different cell groups using RRC signalling. For sending traffic over these cell groups, it has been agreed that BH RLC channels can only be mapped to a single cell group (MCG or SCG, i.e. no split bearer is supported). Essentially, this means that the two cell groups will be seen as two independent Network Interface Cards (NICs) towards higher layers.

At the RAN3#105bis meeting, the following was agreed:

BAP – Intra-donor configuration

- Configuration of downlink bearer mapping and routing should be performed by F1-AP

- Path id is derived from IP header and mapping provided by CU

- In the DL, for BAP path id derivation on the donor DU: IP address, IPv6 flow level and/or DS/DSCP can be used; all of these fields are optional in F1AP message to configure routing

IAB topology discovery

The IAB-donor CU discovers collocation of IAB MT and IAB DU from the IP address used by the IAB DU for F1-C, or from the IAB-node’s BAP address included in F1AP (e.g., F1 SETUP REQUEST).

This paper proposes IAB-node multi-connectivity via multiple IAB-MTs, considering the latest RAN3 agreements above.

# 2 Discussion

Currently, NR supports DC by setting up multiple UE bearer contexts in the DUs that serve the UE. These different UE contexts are identified as part of F1-U (GTP tunnels) towards the DU serving the UE. This is shown in Figure 1.

The DC aspects are transparent to UE application layers, i.e. the UEs just send/receive data from a DRB which could be configured as an MCG, SCG or a split DRB. In case of split DRB, the splitting point is below PDCP and relying on various NR PDCP functions to handle re-ordering, re-transmission and duplication removal.

Figure 1: Support for NR-DC to UEs

At the RAN2#105bis it was agreed to reuse the NR-DC framework for setting up multi-connectivity to IAB-nodes. It was also agreed that only MCG or SCG BH RLC channels should be used, meaning that the traffic will not be split, i.e. the split bearer will not be supported, as there is no NR PDCP for F1-U traffic to the IAB-node.

Observation 1: RAN2 agreed that NR-DC framework should be used to set up multi-connectivity for IAB-nodes, but there will be no support for split bearers to the IAB-node.

In case an IAB-node has been set up with multiple connections, it is possible to associate the different connections with different BAP Path IDs as part of the agreed BAP routing ID.

Observation 2: The BAP Path ID field of the BAP routing ID can be used to identify the different paths in the BAP layer.

The DC framework currently has some limitations:

- DC is only supported between different frequency bands;

- Only 2 cell groups are currently supported;

- For IAB-nodes using EN-DC, only a single NR SCG is supported.

One alternative to overcome these limitations is to extend the DC framework in the 3GPP specifications by allowing intra-frequency DC, multiple connectivity with more than two cell groups, etc. However, a simpler approach to solve the same problem is to allow the IAB to support multiple logical MTs, which set up independent connections to the network. This approach can support any number of connections on any frequency and, from a user plane point of view, is identical to the current DC-based solution.

It is expected that some IAB-nodes will support multiple radio units, capable of operating independently or only loosely coupled with each other, on the same frequency. These radio units could be associated with different antennas pointed in different directions, e.g. covering different sectors. Although these radio units would still be limited by half-duplex constraints (i.e. not being capable of simultaneous transmission and reception on the same frequency), they would support simultaneous transmission to, and simultaneous reception from multiple parent nodes on the same frequency. In this respect, some IAB-nodes will likely be very different from current UE chipset modems, which support tightly integrated DC solutions.

Observation 3: Some IAB-nodes are expected to support multiple radio units capable of simultaneous TX (no RX) and simultaneous RX (no TX) in different directions, on the same frequency band.

Observation 4: The current DC-based solution does not support simultaneous TX or RX scenarios on the same band.

One simple way to allow these nodes to set up multiple connections to the network is to allow them to operate the MT parts as independent MTs with separate control RRC/NAS connections to the network, and separate BAP addresses, etc. Similar to the DC-based solution, each connection can be treated as a separate NIC from the higher-layer point of view. With such a solution, the normal connectivity management procedure can be used for setting up the MT connections. The only additional functionality required is to ensure that the different MT connections are set up via different radio paths. Most likely, this can be supported based on IAB-node implementation, i.e. the IAB-node will only connect multiple paths if multiple parent cells are available. An added benefit of using multiple MTs is that more than 2 NR paths (or more than 1 NR path for EN-DC) can be supported without standard impacts.

Observation 5: Using multiple MTs is a way to support multiple connections to IAB-node(s) without the limitations associated with the current DC-based solution.

Observation 6: If an IAB-node has multiple MTs, no interface or communication between the different MTs is required.

Observation 7: Multiple MTs can be supported by reusing the existing connection setup mechanism. It could be left to IAB-node implementation/configuration to set up multiple MT connections when it can connect via different parent cells.

# 3 IAB Topology Discovery

The multiple MTs of a given IAB-node need to be associated/linked to the DU of the same IAB-node. For the baseline case of a single-MT IAB-node, RAN3 has agreed (from IAB BL CR for TS 38.401):

The IAB-donor CU discovers collocation of IAB MT and IAB DU from the IP address used by the IAB DU for F1-C, or from the IAB-node’s BAP address included in F1AP (e.g., F1 SETUP REQUEST).

Thus, the collocation of multiple IAB-MTs with an IAB-DU can be indicated by including multiple BAP addresses in the F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE.

Observation 8: The IAB topology discovery mechanism that associates the MT and DU of an IAB-node can easily be adapted to associate multiple MTs to one DU of an IAB-node.

Proposal 1: An IAB-node may establish multi-connectivity multiple MTs, based on implementation or configuration.

Proposal 2: The F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE may carry multiple BAP addresses for IAB topology discovery.

# 4 Conclusion

Based on the discussion in the previous sections we observe the following:

Observation 1: RAN2 agreed that NR-DC framework should be used to set up multi-connectivity for IAB-nodes, but there will be no support for split bearers to the IAB-node.

Observation 2: The BAP Path ID field of the BAP routing ID can be used to identify the different paths in the BAP layer.

Observation 3: Some IAB-nodes are expected to support multiple radio units capable of simultaneous TX (no RX) and simultaneous RX (no TX) in different directions, on the same frequency band.

Observation 4: The current DC-based solution does not support simultaneous TX or RX scenarios on the same band.

Observation 5: Using multiple MTs is a way to support multiple connections to IAB-node(s) without the limitations associated with the current DC-based solution.

Observation 6: If an IAB-node has multiple MTs, no interface or communication between the different MTs is required.

Observation 7: Multiple MTs can be supported by reusing the existing connection setup mechanism. It could be left to IAB-node implementation/configuration to set up multiple MT connections when it can connect via different parent cells.

Observation 8: The IAB topology discovery mechanism that associates the MT and DU of an IAB-node can easily be adapted to associate multiple MTs to one DU of an IAB-node.

Based on the discussion in the previous sections we propose the following:

Proposal 1: An IAB-node may establish multi-connectivity multiple MTs, based on implementation or configuration.

Proposal 2: The F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE may carry multiple BAP addresses for IAB topology discovery.