3GPP TSG-RAN WG3 Meeting #107e	R3-200844

# Online, 24th – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1457 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNB node identification in case it is associated with more than one PLMN ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There have been discussions in RAN3 revealing that there is no common understanding on how eNBs may be identified in case it serves cells with more than one PLMN ID associated to them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The statement defining the Global eNB ID in the IE section is amanded by Global identification of an eNB that serves cells associated with more than one PLMN Identity is specified in TS 36.300 [15]. An equivalent statement is given for the Global en-gNB ID. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification for implementations following the statement introduced by this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be still no common understanding how to identify eNBs or en-gNBs if more than one PLMN ID is associated with it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.22, 9.2.1.112 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.300 CR ... Rel15 TS 36.300 CR ... Rel16 TS 36.413 CR1756 Rel15 TS 36.413 CR1757 Rel16 TS 36.423 CR1458 Rel16 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 9.2.22	Global eNB ID

This IE is used to globally identify an eNB (see TS 36.401 [2]).  Global identification of an eNB that serves cells associated with more than one PLMN Identity is specified in TS 36.300 [15].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.4 |  | – |  |
| CHOICE eNB ID | M |  |  |  | – |  |
| >Macro eNB ID | M |  | BIT STRING (20) | Equal to the 20 leftmost bits of the value of the E-UTRAN Cell Identifier IE contained in the ECGI IE (see section 9.2.14) identifying each cell controlled by the eNB | – |  |
| >Home eNB ID | M |  | BIT STRING (28) | Equal to the value of the E-UTRAN Cell Identifier IE contained in the ECGI IE (see section 9.2.14) identifying the cell controlled by the eNB | – |  |
| >Short Macro eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the value of the E-UTRAN Cell Identifier IE contained in the ECGI IE (see section 9.2.14) identifying each cell controlled by the eNB. | – |  |
| >Long Macro eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the value of the E-UTRAN Cell Identifier IE contained in the ECGI IE (see section 9.2.14) identifying each cell controlled by the eNB. | – |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.112	Global en-gNB ID

This IE is used to globally identify an en-gNB (see TS 37.340 [32]).  Global identification of an en-gNB that serves cells associated with more than one PLMN Identity follows the same principles as specified for the global identification of an eNB as specified in TS 36.300 [15].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.4 |  |
| CHOICE en-gNB ID | M |  |  |  |
| >en-gNB ID |  |  |  |  |
| >>en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the en-gNB. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>