3GPP TSG-RAN WG3 Meeting #107e	R3-200853

# Online, 24th – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0349 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NG-RAN node identification in case it is associated with more than one PLMN ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There have been discussions in RAN3 revealing that there is no common understanding on how NG-RAN nodes may be identified in case it serves cells with more than one PLMN ID associated to them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the abnormal conditions of the NG Setup procedure, it is specified, that in case the PLMN Identity in the Global RAN node ID does not match with any of the PLMNs supported by the AMF, but at least one PLMN is supported by both, the AMF and the NG-RAN node, a subsequent RAN Configuration Update procedure is triggered - The definition of the Global RAN Node ID IE is amended by the following statement: Global identification of an NG-RAN node that serves cells associated with more than one PLMN Identity is specified in TS 38.300 [8] Impact Analysis: Impact assessment towards the previous version of the specification (same release): The definition of the Global RAN Node ID has no impact with the previous version of the specification for implementations following the statement introduced by this CR. The modification of the abnormal condition of the NG Setup procedure has isolated impact with the previous version of the specification (same release) as it concerns the NG Setup procedure only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be still no common understanding how to identify an NG-RAN node if more than one PLMN ID is associated with it. It would be unclear how to deal with a Global RAN Node ID during NG Setup indicating a PLMN not supported by the AMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.1.4, 9.3.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR ... Rel-15 TS 38.300 CR ... Rel-16 TS 38.413 CR0348 Rel-15 TS 38.423 CR0324 Rel-15 TS 38.423 CR0325 Rel-16 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

#### 8.7.1.3	Unsuccessful Operation

Figure 8.7.1.3-1: NG setup: unsuccessful operation

If the AMF cannot accept the setup, it should respond with an NG SETUP FAILURE message and appropriate cause value.

If the NG SETUP FAILURE message includes the Time to Wait IE, the NG-RAN node shall wait at least for the indicated time before reinitiating the NG Setup procedure towards the same AMF.

#### 8.7.1.4	Abnormal Conditions

If the NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the PLMN Identity IEs and none of the PLMNs provided by the NG-RAN node is identified by the AMF, then the AMF shall reject the NG Setup procedure with an appropriate cause value.

Interaction with the RAN Configuration Update procedure

If the NG SETUP REQUEST message includes a Global RAN Node ID containing a PLMN Identity not supported by the AMF, but the NG-RAN node and the AMF at least support one common PLMN Identity, the NG Setup procedure shall be terminated successfully and the NG-RAN node shall  trigger the RAN Configuration Update procedure including the Global RAN Node ID IE containing a PLMN Identity supported by the AMF.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.5	Global RAN Node ID

This IE is used to globally identify an NG-RAN node (see TS 38.300 [8]). Global identification of an NG-RAN node that serves cells associated with more than one PLMN Identity is specified in TS 38.300 [8].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NG-RAN node | M |  |  |  |
| >gNB |  |  |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >ng-eNB |  |  |  |  |
| >>Global ng-eNB ID | M |  | 9.3.1.8 |  |
| >N3IWF |  |  |  |  |
| >>Global N3IWF ID | M |  | 9.3.1.57 |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>