3GPP RAN WG3 Meeting #107-e	R3-201034

24-28 February, 2020

Agenda item:	15.4.1.2

Source:	Apple Inc.

Title:	On Early Data Forwarding for CHO

Document for:	Decision

# Introduction

In RAN3#106, it was agreed to specify early data forwarding for CHO [1-2].

In this contribution, we discuss the necessity of specifying a confirmation mechanism from a target cell to source cell agreeing on receiving early data forwarding for CHO and propose RAN3 to specify it.

# Discussion

## Background

In the RAN3 #106 meeting, it was finally agreed to specify both “early forwarding” and “late forwarding” for CHO in NR and LTE [1-2].

In early data forwarding for CHO, the serving cell can start forwarding DL data packet of the UE to potential target cells after sending CHO configuration to the UE. In this scenario, a target cell may receive data packets of a UE before the UE performing CHO to that target cell.

## Early Data Forwarding for CHO

In the RAN3#106, we showed the benefits of deploying early data forwarding for CHO particularly in FR2 [3]. We know that FR2 networks are essentially deployed to provide high traffic rate and it is noted that due to rapid change of RSRP in FR2 networks, multiple target cells may need to be prepared for a UE. Furthermore, any FR2 cell may simultaneously receive CHO requests for different UEs from neighboring cells. It is therefore understood that due to HW limitations (buffer size, available inter-node interface capacity, etc.) the target node may not be able to receive or record all early forwarded data.

Observation 1: The amount of data forwarded to a prepared CHO cell may be significant.

Observation 2: Due to HW limitations (buffer size, available inter-node interface capacity, etc.) the target node may not be able to receive or record early forwarded data.

We know that it is a network decision  to configure CHO or deploy early data forwarding. Per observation 2, the target cell may not be able to exploit early data forwarded from serving cell and there is currently no mechanism to inform source NB about usability of early forwarded data at target cell.

Observation 3: There is currently no mechanism no inform source NB about usability of early forwarded data at target cell.

We note that a target cell receives information about traffic rate of UE and its QoS requirements when being prepared for CHO. It is therefore expected for the target cell to anticipate if it will be able to receive early data forwarded for a UE.

Observation 4: Target cell has information needed to anticipate if it will be able to receive early data forwarded for a UE.

We think it is useful if target cells can confirm whether they are willing to receive early packet forwarding for a UE when they are prepared for CHO. It can done by the target cell when sending HO Req Ack message back to the source cell. We think it is easier and more efficient if we can include an optional bit in the HO Req Ack message to determine if target cell is willing to receive early data forwarding.

Proposal 1: RAN3 to specify a mechanism by which target cells can confirm to the serving cell whether they are willing to receive early data forwarding for CHO.

# Proposals (for approval)

We note that in order to improve mobility robustness and reliability by using CHO, the network may need to configure multiple neighboring cells. It is particularly important to prepare multiple cells in FR2 where RSRP may change quite rapidly due to UE rotation. In this paper, we noted that it is up to the network to use early data forwarding and we discussed that it is beneficial to specify a mechanism for target cells to confirm whether they are willing to receive early packet forwarding for a UE when sending HO Req Ack message back to the serving cell. Given above we make following observations and proposals:

Observation 1: The amount of data forwarded to a prepared CHO cell may be significant.

Observation 2: Due to HW limitations (buffer size, available inter-node interface capacity, etc.) the target node may not be able to receive or record early forwarded data.

Observation 3: There is currently no mechanism no inform source NB about usability of early forwarded data at target cell.

Observation 4: Target cell has information needed to anticipate if it will be able to receive early data forwarded for a UE.

Proposal 1: RAN3 to specify a mechanism by which target cells can confirm to the serving cell whether they are willing to receive early data forwarding for CHO.

# References

- R3-197617, “Conditional Handover - Data Forwarding”, Ericsson, 3GPP RAN3#106, Nov. 2019

- R3-197618, “Conditional Handover - Data Forwarding”, Ericsson, 3GPP RAN3#106, Nov. 2019

- R3-196597, “On Early Data Forwarding for CHO”, Apple Inc., 3GPP RAN3#106, Nov. 2019