3GPP TSG-RAN WG3 Meeting #107-e	R3-201234

E-meeting, 24 Feb – 6 Mar 2020

Agenda item:	9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary of the email discussion #101bis (ReviseProcTxtX2AP)

Document for:	Endorsement

# 1	Introduction

At RAN3 #107-e meeting, an email discussion was initiated to clarify correction to the description of the usage of the RRC container in the SN-initiated release procedure.

The paper summarises the discussion and proposes conclusions.

# 2	Discussion

During the email exchange, the scenario was clarified to concern SN change situation with delta configuration, where SCG configuration is not released in the UE.

Following text is proposed to be used in the CRs for X2AP:

If the SgNB to MeNB Container IE is included in the SGNB RELEASE REQUIRED message, the MeNB may use the contained information to apply delta configuration.

Following text is proposed to be used in the CRs for XnAP:

If the S-NG-RAN node to M-NG-RAN node Container IE is included in the SGNB / S-NODE RELEASE REQUIRED message, the MeNB/ M-NG-RAN node may use the contained information to apply delta configuration.

# 3	Conclusions

It is proposed to agree the CRs for X2AP (in R3-201235 [1] and R3-201236 [2]) and for XnAP (in R3-201262 [3] and R3-201263 [4]).

# References

- R3-201235, RAN3 #107-e

- R3-201236, RAN3 #107-e

- R3-201262, RAN3 #107-e

- R3-201263, RAN3 #107-e